|  | State of Alaska Department of Public Safety Division of Administrative Services |  |
| --- | --- | --- |
|  |  | Sean Parnell, Governor Keith L. Mallard, Commissioner |

TO: Bidders		 					              DATE: 11/26/2013

FROM: Frank D. Whittington

Facility Manager

STATEMENT OF WORK

Alaska Department of Public Safety Training Academy

Energy Lighting Upgrades

Sitka, Alaska

Project # 12-124368F

DPS/SUPPLY

SW-1 SCOPE:	The work to be completed by this project consists: developing an energy saving lighting system replacing all identified interior ceiling lights (T-12) with new high energy lighting to meet the criteria the State of Alaska Energy Program by 2020 in the Alaska DPS Training Facility in Sitka, Alaska 99835.  This work is being completed for the Facility Management Office, Department of Public Safety 524 E. 48th Avenue, Anchorage, Alaska 99503.

SW-2 LOCATION: The site for this work is located at The Department of Public Safety (DPS), Training Academy located at 877 Sawmill Creek Road, Sitka, AK 99835

SW-3 PRINCIPAL ITEMS OF WORK: The primary work requires the contractor to provide all materials, labor, tools, equipment, supervision, and overhead to complete all work as stated in SW-1 through SW-6.

The energy lighting upgrades will also include new controls, occupancy sensors in certain areas (to be identified in pre-bid walk through), and installation of new T-8 type light fixtures.  Most fixtures are drop ceiling type lights. We have classrooms, offices, dorm rooms, library, hallways, storage rooms, and stairwell areas.  The work will be on the first and second floor of the Training Academy.

Base bid:		Will be using the T-8 type light fixtures.

Alternate # 1 Bid:	This option depending on available funding will use LED lights in certain areas, classrooms & hallway.

Please break out bid price for these identified sections for comparison.

Primary Work: Removal of existing old light fixtures according to a developed schedule approved by the Department of Public Safety Facilities Manager. Proper disposal of all debris including any broken ceiling tile, light fixture, and other debris from this project will be the responsibility of the awarded contractor. The intent of this project is to renovate these areas during the downtime between training cycles from early December to the start of the new recruit class in late February. The completion date for this project will be February 17, 2013.  This is a firm date so if you cannot perform in these time periods please do not bid on this project.

- The contractor will submit fixture and control submittals with the bid documents of all proposed replacement lighting and accessories.  Time is of the essence in completion of this project so submittals will be required with the bid documents for fast approval.

- Inspection of work progress will be with the building maintenance manager, Randy Keaveny.

- The light fixtures will be UL Rated T-8 light fixtures.

- The contractor will secure all removed T-12 light bulbs and ship them to the DPS Supply Warehouse located at: 524 E 48th Avenue, Anchorage, Alaska 99503 upon completion of this project being careful to protect them from breakage during shipping.

- The contractor will provide guarantee that the replacement T-8 lights that are being installed meets the proper illumination on all work surfaces throughout the facility equal or better than existing conditions.

- The contractor will provide energy saving light controls throughout the facility. (Resistor type dimmers, dimming ballast, and motion sensors).

- Dimmers are to be installed in classrooms.

- Motion occupancy sensors will be installed in hallways.

- All light fixtures must be UL rated and meet commercial Federal and State codes.

- Coordination of work is to be with D. Whittington, DPS State Facilities Manager and DPS Training Academy Commander, Lt. Goeden. Contact information will be given once award of this project is completed.

- The contractor will be responsible for cleaning any work areas and leaving in a proper condition at completion of each day and during the progress of the project as these are high profile areas in the academy building.

- Proper disposal of all debris is the responsibility of the contractor, including renting a waste container (coordination on where to put this dumpster will be coordinated with the building facility manager, Randy Keaveny.

- The contractor will use approved materials and workmanship for this project.

- Any broken or soiled ceiling tile will be replaced by the contractor and approved by the building facilities manager.

- Base Bid will include price for T-8 type fixtures in the two classrooms with dimmers. (Break out pricing for this area in your bid)

- Alternate # 1 will include the option of using LED type fixtures in the two classrooms with dimmers. Also LED type fixtures in the 1st Floor Hallway (This option will be based on available funding)

- Contractor will field verify all measurements during the pre-bid walk-through.

- Contract award may be based on best value considerations based on the tight schedule and completion date requirements.

Fixture Count and Type

Basis for design of the lighting

Location			Fixture Count	Type		Base Bid

1st Floor Classroom # 1				32			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

1st Floor Classroom # 2				24			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

1st Floor Small Classroom			04			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

1st Floor Storage Room				06			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

1st Floor Offices (6)				36			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

1st Floor Library					12			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

1st Floor Hallway					40			T-8

Fixture Catalog #

Cooper Lighting 2GR8 2U6T8 A125 UNV FLEX LAMPS (835) EB81 U

Type: FL 2x2   Note: Add three (3) Occupancy Sensors in Classroom Hallway

2nd Floor Dorm rooms (Left Side)		10			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

2nd Floor Dorm rooms (Right Side)		16			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

2nd Floor Hallway					10			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

2nd Floor Break room # 224 (Right Side)	02			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

2nd Floor Stairwell # 201 (Right Side)		03			T-8

Fixture Catalog #

Cooper Lighting GR8-232A125-UNV-L8841-A3/8-2/18G-ER81-U 	Type: FL 1x4

Alternate # 1		LED Lights in Classrooms & 1st Floor Hallway

1st Floor Classroom # 1				32			LED

Fixture Catalog #

Cooper Lighting GR-LD1-32-A125-UNV-L840-CD1-U 	Type: LED 1x4

1st Floor Classroom # 2				24			LED

Fixture Catalog #

Cooper Lighting GR-LD1-32-A125-UNV-L840-CD1-U 	Type: LED 1x4

1st Floor Small Classroom			04			LED

Fixture Catalog #

Cooper Lighting GR-LD1-32-A125-UNV-L840-CD1-U 	Type: LED 1x4

Note: Add two (2) sets of double dimmers in each classroom

1st Floor Hallway					40			LED

Fixture Catalog #

Cooper Lighting 2GR LD1 32 A 125 UNV FLEX L835 CD 1 U

Type: LED 2x2 Note: Add three (3) Occupancy Sensors in Classroom Hallway

Confirm this is correct fixture before ordering (it looks like for suspended ceiling not surface mount)

All questions pertaining to this project and the administration of this contract will be sent via email to (frank.whittington@alaska.gov )

SW-4 TOUR OF JOB SITE: Site tour will be conducted on Tuesday, December 03, 2013 at 2:30 p.m. in Sitka, AK at: 877 Sawmill Creek Road, Sitka Alaska 99835.  Confirmation of participation must be arranged prior to this time by December 02, 2013, at 11:00 a.m.   No other contractors will be accepted after this time.  You must contact Mr. Frank D. Whittington, Facilities Manager at (907) 561-1092 Ext. 223, or with an email (frank.whittington@alaska.gov ) with the following information:

Business Name:

Business Point of Contact:

Business address:

Business Phone Number:

Business Fax Number:

Business email address:

SW-5 SPECIFICATIONS: Contractors must comply with OSHA Safety Standards.  All work is to be performed in accordance with industry Standards and all applicable codes. Contractor will provide all labor, materials, tools, equipment and supervision to complete this work. All materials removed during project will be the responsibility of the contractor to remove from job site for proper disposal.  Contractor will be required to provide signed waiver for background check of any personnel who will be working inside of this facility. Waiver will be provided to awarded contractor.

SW-6 BID DEADLINE: You must contact Mr. Frank D. Whittington, Facilities Manager at (907) 561-1092 Ext. 223, or Fax: 907-561-9178 or email (frank.whittington@alaska.gov ) with your bid.  The Bid Deadline for this project will be Wednesday, December 11, 2013 at 1:00 p.m.