- PURPOSE

To define the maintenance functions to support a hygienic, infection-free environment, at Alaska Psychiatric Institute (API).

- POLICY

The Maintenance Department at API provides a hygienically clean environment by systematic inspection and preventive maintenance of all equipment, by established routines for emergency repairs, and by proper care of the entire physical structure.

- DEFINITIONS

None

- PROCEDURE

  - THE MAINTENANCE DEPARTMENT WILL:

    - Clean all tools with appropriate disinfectant when isolation is in force.

    - Check window units for safety and cleanliness on a quarterly basis per manufacturer's preventive maintenance schedule.

    - Check and maintain heating systems daily.

    - Clean plumbing fixtures and drains with enzymes on a scheduled basis.

    - Check washers and dryers quarterly, to include filters, drains, and traps maintained by Maintenance.

    - Disinfect water lines by chlorination following repairs or service disruption. Inspect overhead waste lines and distilled water equipment for leakage.

    - Not use Remove aerators from faucets.

    - Wear mask when cleaning A/C ceiling filters and vents.

    - Keep the Maintenance Department clean by:

      - Maintaining proper equipment daily.

      - Using disinfectant soap.

      - Removing garbage daily.

      - Cleaning floor daily.

      - Cleaning tools routinely.

    - Notify the Infection Control Coordinator (ICC) when any area of the hospital floods and when any major construction is scheduled.

    - Change all air conditioning filters according to manufacturer’s specifications.

    - Coils and catch hoses need routing maintenance and cleaning three times a year and as needed. Hospital-approved disinfectant will be used.

    - Check refrigerators and hot service elements regularly for proper temperature control.

    - Ensure dishwasher temperatures stated in the Alaska Food Code are followed.

    - Check all surfaces, floors, walls and ceilings and repair any break in the walls, floors, ceilings, or windows screens.

    - Install vacuum breakers in water supply lines in the kitchens and check frequently to ensure proper function.

    - Equip all refrigerators with thermometers and ensure that temperature logs are maintained, whether automatically or manually.

    - Check spray head and faucets regularly for function and leakage.

    - Provide an active Preventive Maintenance Program.

    - Involve all employees in Infection Control annual In-Service and ensure orientation to Infection Control.

    - Ensure hand washing after using toilet, after completing work, and before and after eating.

    - Adhere to Standard Precautions. See P&P IC 201 Standard Precautions Policy and Infectious Waste handling.

    - Use protected footwear and clothing for procedure that involve sewer line maintenance.

    - Place plungers in red bag after use until cleaned with approved germicide.

- HISTORY OF REVISIONS

New: 09/18/00

Revised: 07/22/03; 10/16/03; 06/02/06; 07/13/06; 08/30/07; 07/18/19; 07/03/23

Reviewed: 07/03/23

- ATTACHMENT

None