SURVEY REQUEST

| Project name: | HNH Airport Rehabilitation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| From: (Section, Design Group) | Design Group 2 |  |  | Date Submitted: |  |  |  |
| Request initiated by: | William Redman |  |  | Phone: | (907) 229-3938 |  |  |
| State/Federal/AIP Project #: |  | SFAPT00523 |  |  |  |  |  |
| Desired Completion Date: |  | Spring 2024 | Program Code: | SFAPT00523 |  | Activity Code: | 015P |
| Project Scope & Survey Limits: (include exhibits as attachments) |  |  | Project Phase: | T02015 |  |  |  |

(Please contact Right of Way Engineering prior to filling this out, as their requirements may directly affect the survey effort required. Include their response)

For each area the following information is needed: Alignment data or physical location of line (exist CL, top bluff, etc.); Desired contour interval; Distance or physical limits left and/or right from line desired. Please provide an attachment for each alignment. High accuracy and point density

Scope of overall project:

This project proposes to rehabilitate the runway, taxiway, apron, and service road access, replace the NAVAIDS (wind socks, segmented circle), replace lighting, and reconstruct the service road access adjacent to the apron.

Scope of survey work requested:

Design-level airfield survey of the areas shown in the attached exhibit including existing terrain, grade breaks, ditch/creek channel alignments, fill limits of embankments, runway thresholds, runway and taxiway profiles/centerlines, airfield lighting, NAVAIDs, buildings, fences, culverts, utilities, and all other features contained within the area shown in the attached survey extents exhibit. The survey includes an ortho image and point cloud for all areas within the airport property limits. Collect survey data for the following: - Existing ground TIN surface within the survey limits (hatched area) on 50’ grid, to include 50 feet beyond the toe of existing embankments. LIDAR Survey is required on all hard/paved surfaces to support variable depth milling design. - Airfield Lighting systems, including all edge lights, threshold lights, REIL’s, underground power, meter bases, junction boxes, radio communication antenna’s, electrical equipment buildings, etc. - Survey monuments that are used to define the runway centerline - Location and height of primary wind cone - Location of the supplemental wind cone - Segmented Circle panels and associated embankment - Culverts and drainage features. Document pipe type (i.e. corrugated aluminum, corrugated steel, corrugated HDPE with smooth interior, etc) and size - Top and base (or finished floor) of all buildings, shipping containers, or other structures - Bathymetry of all water bodies within 50’ of the embankment fill limits - MLLW elevations at the edge of all water bodies and at all culvert crossings - Power, telephone, or other utilities (overhead and underground). FAA utility locates will require advance coordination. - All above ground objects greater than 3 in above ground within the airport property - Tree/vegetation limits, with highest tree within the group (if applicable) - Aircraft tie downs - Any objects with obstruction lights (location and base/top elevations) - Tie the centerline stationing and monumentation from the most recent as-built project - All primary and secondary survey control monuments - Airport Property Boundary. - Photogrammetry: Collect photogrammetric data controlled acceptably for design and mapping purposes. Conduct the work in leaf-off, snow free conditions in the middle of daylight hours. - AWOS system, ground at foundation, top elevation of wind sensor, and associated utility services - PAPI equipment - The horizontal survey point, and vertical survey point as identified in AC 150/5300-18B. - Ground at all foundation corners - Top of foundation at all corners - Highest point of total assembly - Light Housing Assemblies (LHA’s) Bottom of the lamp housing at corners, center elevation of light lens - PAPI Controls -- Top corners of panel box, pad and ground at corners of pad for a large control panel or ground and top at center of small control pedestal. (PAPI Controls are sometimes remotely located from PAPI light cabinets outside the RSA.)

Locate: (Fill empty slots if desired & Check all that apply)

| Improvements |  | Drainage |  | Utilities |  | Right of way/Monuments |  | Other |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Edge Pavement | X | Culverts | X | Above Ground | X |  |  | Trails | X |
| Structures | X | Ditches | X | Overhead Crossings | X | Private Property Corners | X | AWOS | X |
| NAVAIDs | X | Storm Drain | X | Inverts | X | Monuments in Roadway | X | Historical Flood Markers | X |
|  |  | Water Valves | X |  |  | Encroachments | X | Ordinary High Water | X |
|  |  | Water Bodies | X |  |  | Monuments on Airport | X | Drone Survey | X |
|  |  | Streams | X |  |  | Lease Lot Corners | X | LiDAR | X |
|  |  |  |  |  |  | Airport Property Limits | X |  |  |

Vertical Control

| Are there any elevation-critical features needing to be located? | ☒Yes ☐No |
| --- | --- |
| If Yes, which datum are these features to be referenced to? (MSL, MLLW, MHW, Project, Geoid__, etc…) |  |

Use State Plane AK83-1F, NAD83 (2011) NAVD 88

Monuments in the road

| Are there survey monuments in the roadway (from as-builts)? | ☐Yes ☐No |
| --- | --- |

Construction Schedule

| When is construction anticipated to occur? 2029 Expected, 2028 Contingent |  |
| --- | --- |

(This section for Survey Section use)

Survey Assigned to:	Estimated Completion Date:

Project History: