State Of Alaska

Department of Transportation & Public Facilities

State Equipment Fleet

2200 E 42nd Avenue

Anchorage, AK 99508

Invitation to Bid SEF-1868

Title of ITB: Airport Shuttle Buses for TSAIA

Date of Issue: July 19, 2013

Bid Opening Date: August 9, 2013 @ 2:30 PM

Contracting Officer: Kristi Futrel

Phone: (907) 269-0793

E-Mail: Kristi.futrel@alaska.gov

CLICK HERE TO REGISTER WITH PROCUREMENT OFFICER

Offerors Are Not Required To Return This Form.

Important Notice: If you received this solicitation from the State of Alaska’s “Online Public Notice” web site, you must register with the procurement officer listed in this document to receive subsequent amendments. Failure to contact the procurement officer may result in the rejection of your offer.

- NOTE:  This section will be used for Terms and Conditions that are "special" and/or unique to a particular ITB.  The paragraphs in this section may change from bid to bid.  Accordingly, bidders must review this section to insure compliance.

- INTENT:  This invitation to bid (ITB) is intended to result in the purchase of Two (2) Airport Shuttle Buses.

  - Contract Period: One Time Purchase

  - Quantities:  2

  - Location of Use: Ted Stevens Anchorage International Airport

  - Warranty locations: Anchorage

  - In addition to the State of Alaska requirements, the Municipality of Anchorage and other Alaska political subdivisions may cooperatively purchase from the resulting contract.

    - At no time may the contractor change the terms and conditions, alter the price to another entity, which differs from the contractual price, nor charge undisclosed administrative fees to allow cooperative purchasing.

- DELIVERY:

  - Pre-delivery service:  Prior to delivery, each vehicle, piece of equipment or attachment shall be serviced and inspected by the dealer or his agent.  Inspection must include the following (as applicable to the type of equipment):

    - Dealer and vehicle identification.

    - Check-off of service and inspection performed including a list of all fluids including type weight and specification that are in the equipment as delivered for all fluid compartments.

    - The vehicle's crankcase, differential and transmission, and other fluid compartments shall be filled to the manufacturer's recommended capacity.

    - Fuel tank shall be filled to at least register a minimum ¼ full on the fuel gauge, unless restricted by the commercial carrier, when the vehicle arrives at the delivery location.

    - The vehicle shall be clean and free from defects when delivered and should be ready for immediate and continued use upon delivery.

    - Units delivered in an incomplete state, or which have deficiencies per the specification, are subject to the damage charges as noted in paragraph 4.0 below.

  - Inspections:

    - The State's inspection of all materials and equipment upon delivery is for the sole purpose of identification.  Such inspection shall not be construed as final or as acceptance of the materials or equipment if materials or equipment do not conform to Contract requirements.  If there are any apparent defects in the materials or equipment at the time of delivery, the State will promptly notify the Contractor thereof.  Without limiting any other rights of the State, The State at its option, may require the Contractor to:

      - repair or replace at contractor's expense, any or all of the damaged goods,

      - refund the price of any or all of the damaged goods, or

      - accept the return of any or all of the damaged goods.

    - Costs of remedying all defects, indirect and consequential costs of correcting same, and/or removing or replacing any or all of the defective materials or equipment will be charged against the bidder.

  - Acceptance:

    - Units will not be considered “Accepted” until all deficiencies have been corrected.  This includes item 2.5 Line Sheets/Bill of Materials and 8.0 Publications.

  - Delivery Receipt:

    - A delivery receipt will be required.  The receipt must be filled out by the vendor, and acknowledged by state receiving personnel by signature and date of actual receipt of equipment.  One copy of this delivery receipt is to be given to the state-receiving agency.

    - Vendors are cautioned and advised that such delivery forms or other receiving type documents will not in any way be construed to mean the state has formally and fully accepted unit(s) referenced thereon as complete and meeting every specification set forth.  Only the Contracting Officer or designee may sign warranty documentation.

  - Line Sheets/Bill of Materials::

    - It is required at the time of delivery that the successful bidder provides a comprehensive listing of all components used to assemble the unit.

    - This includes any components installed by the manufacturer or any subcontractor or the successful bidder.

    - Information will include at a minimum, when available, make, model serial number on items such as engines, transmissions, axles, tires, bodies, etc.  The listings will be specific to each piece of equipment and will be provided on an individual CD for each unit delivered.

    - A minimum of one (1) CD per unit are to be provided and marked with the make, model, and last main numbers of the units serial number or State PO number.

- F.O.B. POINT:

  - The F.O.B. point is as listed in Section IV, Bid Schedule.  Ownership of and title will remain with the contractor until delivery is complete to final destination and accepted by the State.  Equipment is not to be driven on the Alcan Highway without prior written approval from the contracting officer.

  - The cost of shipping and delivery for orders beyond the limits of Seattle/Tacoma dock will be handled as follows.  The contractor will prepay the shipping and delivery charges to any destination named by the State in its order.  The contractor will charge-back those shipping and delivery charges to the State as a separate line item on the State’s invoice.

  - All shipping charges over $100 must be documented by a copy of the actual shipping invoice and received with the invoice charge to the State.

  - Shipping must be consolidated for the best possible price.  Shipping items separately must be pre-approved by the Contracting Officer PRIOR to shipment.  For example, GP Bucket or Spare Tire not being shipped with host unit must be pre-approved.

- DAMAGES FOR LATE DELIVERY AND NON-CONFORMING GOODS:

  - Time is of the essence in this contract.  The Bidder is expected to deliver goods that conform in all material respects to the contract specifications on or before the date provided therein, as may be amended by written agreement of the parties.

  - In the event that the equipment is delivered late or does not conform to the contract specifications, the State shall be entitled to offset against the Contract Price, as liquidated damages and not as a penalty, an amount equal to the cost of renting like equipment, multiplied by the number of calendar days elapsing between the delivery date provided in the bid schedule and the delivery date to the State.  In the case of equipment in this this class, that daily rental fee is determined to be $100.00.  The number of days for which liquidated damages shall apply shall include, in the case of non-conforming goods, the time reasonably necessary for the State to perform inspection.

  - These liquidated damages represent a reasonable estimate of amounts necessary to compensate the State for loss of use of the goods during the period in which the goods would have been available to the State if conforming goods had been timely delivered.

- EQUIPMENT RELIABILITY:

  - Reliability of equipment is of paramount importance to the State.  It is the policy of SEF to require minimum levels of reliability from owned or leased equipment for it to be considered acceptable.  Equipment offered for this bid must be capable of meeting the acceptable reliability standard stated below.

  - Acceptable Reliability:  The State will monitor equipment reliability.  Acceptable reliability for this contract is achieved when a machine achieves or maintains a Reliability Ratio (RR) equal to or exceeding the following:

    - .90 (90 percent) PR during any consecutive 12-months (365 days) during the warranty period.

    - .75 (75 percent) PR per operational month (recognizing operational as subject to weather and being defined by calendar days) during the consecutive 12-month period.

    - PR below the state percentages do not meet minimum reliability requirements for state owned equipment.

  - Machine Failure and Downtime:

    - Machine Failure is any and all loss of capability to perform fully, as specified, which is not attributed to Conditioned Failure.  Machine Failure resulting in the unit being out of service is defined as Downtime.

    - Conditioned Failure is any Machine Failure attributable to accident, operator abuse or other external cause not attributable to a defect in the machine itself.

    - Downtime is the actual number of days or fractions of days that the equipment is in a state of Machine Failure.  Downtime does not count time used for scheduled maintenance (including preventative maintenance and scheduled major overhauls), time lost for repair maintenance and scheduled major overhauls, time lost for repair of damage as a result of operator abuse or machine misapplication; or time lost as a result of accident or an act of God.  Downtime includes:

      - Actual shop hours (and/or field repair hours) required to return unit to full operational status following machine failure, including trouble-shooting, repair, necessary replacement of parts, and necessary adjustments, plus

      - Time lost waiting for parts and/or vendor assistance.  “Waiting downtime” also applies if need for parts/assistance is discovered during routine maintenance and return to service is deemed counterproductive.  In this case, “waiting time” clock begins with notice of need to vendor.  Allowance may be considered in “waiting time” calculations if arrival of parts/assistance is delayed by transportation shutdown, to include verifiable transportation scheduling difficulties such as infrequent flights as long as all reasonable alternatives have been exhausted.  Parts and assistance are to be provided by the quickest means reasonably possible to avoid unnecessary delays and downtime.

    - Out of Service Report (OSR):  Downtime resulting from machine failure is the actual number of hours a machine is out of service as recorded on the OSR or in the Equipment Maintenance Management System (EMS).

      - The State will record all downtime on an OSR or EMS work order, which will be originated for each occurrence of downtime.  The document will show the date and time a unit went down, the  location where the machine was based, the reason the machine is down, date and time the vendor was notified (if applicable), the date and time the machine was returned to service, and the total hours of downtime.

        - The Contract Manager will finalize and approve the OSR or EMS work order.  Both are available for contractor review.

    - Reporting Downtime:  The Contracting Officer will maintain documentation of all Downtime, and shall send copies of such documentation to the contractor.

    - Calculation of Reliability Ratio: RR is the mathematical ration of operated time (uptime) to out of service time (downtime).  The RR will be calculated according to the following formula:

RR = Days in a Month – Days Out of Service*   =   DM - DO

Days in a Month **                             DM

Note * : Fractional Days apply, i.e., a unit is out of service 8 hours in a 24 hour period  equals 1/3 or .33% of a day.

Note **: A day is allocated as 24 consecutive hours from 12:00 AM to 12:00 PM.

Example:  30 days DM with 2 days and 8 hours DT would result in:

RR =   30  -  2.33 = .92

30

    - Unacceptable Reliability: If an item of equipment fails to perform at an acceptable level of reliability during the warranty period, the Contracting Officer will notify the contractor and request immediate remedy.  Failure to remedy the piece of equipment within 30 days for failure will result in a breach of contract and the immediate return of the equipment and reimbursement of the Guaranteed Value (V) of the unit:

Original Cost of the unit less (-) Freight = $__________ (V)

Guaranteed Value (V) less (-) the Cost of Operation as listed in the Equipment Rental Rate Blue Book _________or comparable equipment or the current Federal Fixed Usage Rate for the Class for the State of Alaska (a, b or c per hour) times (X) the number of hours used = ___________(DV).

- Example:  Cost of a single unit, less freight = $150,000.  The hourly cost is $150.00 per hour.  The unit was used 150 hours prior to failing the acceptable reliability.  The contractor guarantees the unit’s worth at $127,500.00.

    - Prior to return, the State will correct all reasonable cosmetic deficiencies (such as excessive rust) and those deficiencies that are directly related to damage due to accidents, misuse of equipment or failure to operate or maintain equipment as prescribed by the vendor/manufacturer, prior to public auction.

    - The tires will be serviceable with at least 50% remaining thread.

    - Oil samples, as per manufacturer’s service manual recommendations, will be taken by State of Alaska maintenance personnel on the engine, transmission, differentials and hydraulics.

    - In the case of dispute, at the expense of the State, a qualified agent from Northern Adjusters, Inc. or another professionally recognized appraiser may be commissioned for an independent claim appraisal.  Such appraisal shall be binding upon the State and the contractor.

- WARRANTY:

  - Standard Warranty Package:  Unless otherwise stipulated by this ITB, the successful bidder will provide a one-year (12-month) warranty.

    - Full (100%) Parts and Labor Warranty Coverage of all components for 12 months (year one), from the date the unit is placed in service at the assigned location.

    - Full (100%) Warranty Coverage includes all cost of labor, parts, freight, lubricants, miscellaneous cost, etc., to place the unit in like-new condition.

    - Should the manufacturer’s standard warranty exceed the minimum State warranty requirements, the manufacturer’s warranty will run in conjunction with and enhance the State’s warranty, then continue for the remainder of its term.

    - For clarification, warranty does not apply to normal wear and tear or maintenance items, accident damages, misuse of equipment or failure to operate or maintain equipment as prescribed by vendor/manufacturer.

    - Warranty on Attachments:  Same as Standard Warranty Package.

    - In-Service Date:  Warranty on vehicles not placed in service immediately upon receipt because of time lag to construct body components and/or installation of special equipment, or due to seasonal usage or other delay, shall be warranted from the date the vehicle is placed in service.  The receiving agency shall notify the vendor/manufacturer in writing of the actual "in service" date.  Notification of the requirement for delayed warranty will be provided on delivery orders whenever possible.

  - Warranty Claims:

    - Warranty will be provided at the unit’s assigned (in-service) location.  Because of the remote location of some equipment it is not always practical to deliver equipment to authorized warranty repair facilities.  In these cases, the vendor may perform warranty work at the state's location or, the State of Alaska, at its discretion, reserves the right to perform the warranty work and be reimbursed by the vendor. If travel is required by State personnel to perform the work, actual costs will be used for reimbursement.

    - The State of Alaska has established a warranty procedure whereby the vendor is to be notified via letter, email, or fax, that warranty work needs to be performed.  If time is of the essence, a telephone call confirmed by one of the above written procedures may be utilized.

    - The vendor must notify the state within 24 hours of verbal or written notification that it will begin to perform the warranty work at the equipment location.

    - The State may, at its discretion, proceed to make warranty repairs with its own work force in the case of emergency situation or to preclude excessive downtime (greater than 24 hours).  The State will require a PO to perform the warranty work.

    - Failure to notify the State that the vendor intends to begin to perform warranty is considered a contractual breach.

    - The vendor will be invoiced for required warranty work performed by the state.  Warranty work performed by the state will be charged at the current SEF shop labor rate at the time of the repair.  Actual repair time will be used.

  - Warranty Performed by Vendor:

    - The State will reimburse travel costs not reimbursed by the manufacturer for travel to and from the bidder’s closest warranty service center within the State of Alaska to the location of the equipment under warranty.  Travel costs will be billed as follows

      - Mileage Charge: Mileage will only be reimbursed for travel within Alaska at the rate allowable by the IRS.

      - Meals are paid at actual and charges must be accompanied by receipts and are not to exceed the State authorized $60.00 per day.

      - Transportation, such as airfare, shall be reimbursed at actual and all charges are to be accompanied by a receipt/copy of the coach ticket.

      - Lodging shall be reimbursed at actual and shall not exceed $150.00 per night unless no other lodging is available.  Requests for reimbursement must be accompanied by a receipt.

    - Travel will only be reimbursed for time in Alaska.

    - After hours, weekend and holiday travel must be approved by the contracting officer to be considered for reimbursement.  The State will not pay for weather delays.

  - Authorized Warranty (Contractor/Bidder):

    - Contractor (bidder) must have Authorized Warranty Dealer that has all required licenses, facilities and factory certified and trained personnel necessary to perform the warranty servicing and repair work.

- Provide name and address for each Authorized Warranty Dealer for each location.

- (*)

- Provide contact name and contact information for Warranty Administrator:

- (*)

- Provide documentation of factory certified and trained personnel:

- (*)

    - The ultimate responsibility for warranty lies with the contractor (bidder).

    - The State reserves the right to inspect the warranty facility and diagnostic equipment prior to issuing the Notice of Intent to Award a contract.

  - Factory Recall:

    - Nationwide factory recall or product update programs are the responsibility of the vendor and/or manufacturer.  The State will attempt to bring affected equipment to an authorized repair facility.  However, because of the remoteness of some equipment this is not always practicable or economical.  In such cases, factory recall and modification work will be handled the same as warranty work.  Factory recall notices sent to the state should, in addition to serial number, include model, year, and dealer.

- REPAIR ORDERS AND DOCUMENTATION:

  - Any work performed by the contractor or approved subcontractor, whether warranty or any other work on a piece of equipment purchased under this ITB, will require a copy of the repair order, any invoices showing parts and commodities including oils and types used.

- PUBLICATIONS:

  - Paper publications are to be received by the State at the time of delivery.  Delivery will not be considered complete until the publications for each unit have been received by the State of Alaska.  Note: Publications, when required, will be ordered on the same Purchase Order as the unit itself.

    - All paper manuals are to be pre-assembled in factory binders prior to delivery.

    - Electronic publications may be requested.

  - Service Manuals:

    - Complete set(s) to include applicable information covering prime unit and attachments:

    - Body, chassis, and electrical

    - Engine, transmission, and differential(s) (service and rebuild)

    - Electrical and vacuum troubleshooting

    - Wiring diagrams

    - Service specifications

    - Engine/emission diagnosis

  - Parts Manuals:

    - Complete set(s) including all updates.  If updates are not provided during the warranty period, the State may order them from the manufacturer and bill the contractor for the full cost, including shipping.

    - Parts manuals are to be customized by serial number.

  - Operator’s Manuals:  Complete set(s) to include prime unit and attachments.

  - Quantities:  As per Section IV – Bid Price Schedule.

  - Service Bulletins, Etc.:  The successful bidder must provide appropriate service bulletins, technical support bulletins, service letters, product support bulletins, and/or any other information type notifications that are sent out to the vendor or used by the manufacturer in the maintenance and report of the vehicle, equipment or attachments being provided.  The intent of this clause is that the State of Alaska be provided notification of any and all changes or improvement’s that may affect the maintenance, reliability, longevity, and safety of our equipment.

- STATEMENT OF ORIGIN:  The bidder will be required to furnish a Manufacturer's Statement of Origin for Automotive or Non-Automotive rolling stock for each unit.  All such documents shall be delivered with the invoice to:

- DOT&PF, HQ State Equipment Fleet

- 2200 E. 42nd Avenue Room #318

- Anchorage, Alaska  99508

- WEIGHT VERIFICATION SLIPS:  If required in the Bid Price Schedule, a weight scale ticket of the completed unit will be included with the Statement of Origin.

- PRICE:

  - Price Guarantee: The Contractor is responsible to maintain prices under the contract firm for 180 days after bid opening.  All price increases or decreases must remain firm for the following 180 days.

  - NO RETROACTIVE PRICE INCREASES WILL BE ACCEPTED.

  - Price Decreases: During the period of the contract, the Contractor must pass on to the state all price decreases, such as fleet rebates.  A Contractor’s failure to adhere strictly and faithfully to this clause will be considered a material breach of contract.  The state reserves the right to cancel the contract if the contractor fails to properly perform the duties set out herein.

  - Manufacturer’s Rebate (Incentives):

    - In any circumstance during or prior to completion of the contract, whereupon the State of Alaska becomes eligible to receive a rebate for any vehicle purchased under this contract, it shall be the BIDDER'S responsibility to inform the Contracting officer in writing and to advise the procedures for obtaining such rebates.

- REPLACEMENT PARTS AND REPAIRS:

  - This contract encompasses a full parts and labor contract for manufacturer parts and repairs for the entire warranty period.

  - The State of Alaska shall expect the dealer or manufacturer to provide replacement wear parts at their authorized warranty facilities for the entire warranty period within seven (7) days of order.  All other parts must be available within ten (10) working days.

  - Back order procedures:  Back orders are acceptable; however, the ordering shop shall be appraised at time of original orders as to the expected delay in delivery.

  - Warranty:  All products supplied by the contractor shall be warranted against defects in materials and workmanship for a minimum of 90 days, commencing at the time of installation as long as the installation is within 12 months of purchase.  The cost of any defective product and the labor required to replace the defective product shall be the obligation of the contractor.

    - If the manufacturer’s warranty exceeds the stated warranty then manufacturer’s warranty supersedes.

    - Parts Return:  Within 12 months of the invoice date, the State is to be allowed to return new parts with full refund, less actual shipping charges. Cores returned within 12 months of original invoice date will receive full core credit. Returned parts will be in new, resellable condition. Refund will be in the form of a credit/invoice credited to the SOA account with the vendor.

    - Invoicing:  Full description of item is required on all invoices, packing lists and billings.

- ACCESSORIES:  When accessories are supplied, they must be certified to be compatible with the rest of the equipment.  Certification will be written evidence satisfactory to the State that the accessories are compatible.  The bidder’s failure to supply this evidence within the time required by the State will cause the State to consider the bid non-responsive and reject the bid.

- ADDITIONS OR DELETIONS:  The State reserves the right to add or delete items, agencies or locations as determined to be in the best interest of the State.  Added items, agencies or locations will be related to those on contract and will not represent a significant increase or decrease in size or scope of the contract.  Such additions or deletions will be documented via mutual agreement, will be at prices consistent with the original bid price margins, and will be evidenced by issuance of a written contract change notice from the Contracting officer.

- ALASKAN BIDDER'S PREFERENCE: Award will be made to the lowest responsive and responsible bidder after an Alaskan bidder's preference of five percent (5%) has been applied.  The preference will be given to a person who: (1) holds a current Alaska business license; (2) submits a bid for goods or services under the name on the Alaska business license; (3) has maintained a place of business within the state staffed by the bidder, or an employee of the bidder, for a period of six months immediately preceding the date of the bid; (4) is incorporated or qualified to do business under the laws of the state, is a sole proprietorship and the proprietor is a resident of the state, is a limited liability company organized under AS 10.50 and all members are residents of the state, or is a partnership under AS 32.05 or AS 32.11 and all partners are residents of the state; and, (5) if a joint venture, is composed entirely of venturers that qualify under (1) - (4) of this subsection.  AS 36.30.170(b).

- ALASKA BUSINESS LICENSE AND OTHER REQUIRED LICENSES:  At the time the bids are opened, all bidders must hold a valid Alaska business license and any necessary applicable professional licenses required by Alaska Statute.  Bids must be submitted under the name as appearing on the person’s current Alaska business license in order to be considered responsive.  Bidders should contact the Department of Commerce, Community and Economic Development, Division of Corporations, Business, and Professional Licensing, P. O. Box 110806, Juneau, Alaska 99811-0806, for information on these licenses. Bidders must submit evidence of a valid Alaska business license with the bid.

You are not required to hold a valid Alaska business license at the time bids are opened if you possess one of the

following licenses and are offering services or supplies under that specific line of business:

- fisheries business licenses issued by Alaska Department of Revenue or Alaska Department of Fish and Game,

- liquor licenses issued by Alaska Department of Revenue for alcohol sales only,

- insurance licenses issued by Alaska Department of Commerce, Community and Economic Development, Division of Insurance, or

- Mining licenses issued by Alaska Department of Revenue

- ALASKA PRODUCT PREFERENCE: A bidder that designates the use of an Alaska Product which meets the requirements of the ITB specification and is designated as a Class I, Class II or Class III Alaska Product by the Department of Community & Economic Development shall receive a preference in the bid evaluation in accordance with AS 36.30.332 and 3 AAC 92.010.

- ALASKA VETERANS’ PREFERENCE: Notwithstanding a provision in AS 36.30.170 to the contrary, if a bidder qualifies under AS 36.30.170(b) as an Alaska bidder and is a qualifying entity, a five percent bid preference shall be applied to the bid price. The preference may not exceed $5,000. In this subsection, “qualifying entity” means a:

- sole proprietorship owned by an Alaska veteran;

- partnership under AS 32.06 or AS 32.11 if a majority of the partners are Alaska veterans;

- limited liability company organized under AS 10.50 if a majority of the members are Alaska veterans; or

- corporation that is wholly owned by individuals and a majority of the individuals are Alaska veterans.

A preference under this section is in addition to any other preference for which the bidder qualifies.

To qualify for a preference under this section, a bidder must add value by the bidder itself actually

performing, controlling, managing and supervising a significant part of the services provided, or the bidder 	must 	have sold supplies of the general nature solicited to other state agencies, governments, or the general public.

In this section, “Alaska veteran” means an individual who is a:

- resident of this state; and

- veteran; in this paragraph, “veteran” means an individual who served in the

- armed forces of the United States, including a reserve unit of the United States armed forces; or

- Alaska Territorial Guard, the Alaska Army National Guard, the Alaska Air National Guard, or the Alaska Naval Militia; and

- was separated from service under a condition that was not dishonorable.

- ALASKANS WITH DISABILITIES PREFERENCE: If a bidder qualifies for the Alaskan bidder's preference, under AS 36.30.170(b), is a qualifying entity as defined in AS 36.30.170(e) and (j), and is the lowest responsive and responsible bidder with a bid that is no more than ten percent (10%) higher than the lowest bid, the procurement officer will make the award to that bidder, in accordance with AS 36.30.170(e).

- ALTERATIONS:  The contractor must obtain the written approval from the contracting officer prior to making any alterations to the specifications contained in this ITB.  The State will not pay for alterations that are not approved in advance and in writing by the contracting officer.

- AMENDMENTS:  Contract terms shall not be waived, altered, modified, supplemented or amended without prior written approval of the Contracting officer.

- ASSIGNMENT(S): Assignment of rights, duties, or payments under a contract resulting from this ITB is not permitted unless authorized in writing by the State of Alaska, Department of Administration, Division of General Services.  Bids that are conditioned upon the State’s approval of an assignment will be rejected as nonresponsive.

- AUTHORITY:  This solicitation is written in accordance with Alaska statutes AS 36.30 and 2 AAC 12.

- BILLING INSTRUCTIONS: Invoices must be billed to the ordering agency's address shown on the individual Purchase Order, Contract Award or Delivery Order, not to the Division of General Services.  The ordering agency will make payment after it receives the merchandise or service and the invoice.  Questions concerning payment must be addressed to the ordering agency.

- BID FORMS: Bidders shall use this bid and attached forms when submitting bids.

  - Bids, once submitted, may be modified in writing before the time and date set for bid opening.  Any modifications shall be prepared on company letterhead, signed by an authorized representative, and state that the new document supersedes or modifies the prior bid.  Modification must be submitted in a sealed envelope clearly marked "Bid Modification" and identifying the bid number and closing date and time.  Bidders may not modify bids after bid opening time.

  - Amendments may be faxed if they are not price related.

- BID FORMAT:

  - Bid submitted must contain, as a minimum:

    - Cover Page

    - Bid Price Schedule

    - Asterisked (*) item pages only (and/or) Letter of Clarification addressing asterisked (*) items

    - All pertinent amendments

  - Failure to return all of the pages listed above will cause your bid to be rejected.

  - Bids must conform to the requirements of the Invitation to Bid or Request for Proposal.  All necessary attachments (bid bonds, references, descriptive literature, etc.) must be submitted with the bid and in the required format.  Failure to comply with all requirements shall result in a non-responsive bid.

  - Faxed, scanned or e-mail transmitted bids ARE NOT ACCEPTED. (See 13.2 above)

- BID MODIFICATION:

  - Modifications or erasures made before bid submission should be initialed in ink by the person signing the bid.  A bid may be rejected if it contains a material alteration or erasure, which is not initialed by the signer of the bid and the intent of the bidder, is not reasonably ascertainable from the bid as submitted.

  - Bids, once submitted, may be modified in writing before the time and date set for bid opening.  Any modifications shall be prepared on company letterhead, signed by an authorized representative, and state that the new document supersedes or modifies the prior bid.  Modification must be submitted in a sealed envelope clearly marked "Bid Modification" and identifying the bid number and closing date and time.  Bidders may not modify bids after bid opening time.

- BID PREPARATION COSTS:  The State is not responsible for any costs incurred by the bidder in preparation of his bid.

- BID WITHDRAWALS:  Bids may be withdrawn in writing on company letterhead signed by an authorized representative and received by the Procurement Office prior to bid closing time.  Bids may also be withdrawn in person before bid opening time upon presentation of appropriate identification.

- BRAND AND MODEL OFFERED:  Unless otherwise specified, when brand names and model numbers are used to specify the type and quality of the goods desired, bidders must clearly indicate the brand names and model numbers they intend to provide.  The bidder’s failure to identify the brand and model offered will cause the State to consider the offer non-responsive and reject the bid.

- CERTIFICATION OF COMPLIANCE WITH AMERICAN'S WITH DISABILITIES ACT OF 1990:

  - By signature of their bid/proposal the bidder/proposer certifies that they comply with the American's with Disabilities Act of 1990 and the regulations issued there under by the federal government.

  - Services or activities furnished to the general public on behalf of the State must be fully accessible.  This is intended to ensure that agencies are in accordance with 28 CFR Part 35 Section 35.130 and that services, programs or activities furnished to the public through a contract do not subject qualified individuals with a disability to discrimination based on the disability.

- CLARIFICATION OF OFFER:

  - A letter of clarification is required to explain any deviations from terms, conditions, or specifications of this ITB.  Offers exceeding ITB specifications are regarded as deviations and should be clarified.

  - In order to prevent technical errors, following each asterisked is space that shall be used to address all of the asterisked items.  It is mandatory that a letter of clarification or the space behind the asterisked items be used to supply the required information.

  - When using a letter of clarification, or the space provided within the ITB, to address an asterisked (*) item, the statement must indicate specifically what is being provided.  Do not include statements such as "Complies to Spec", or "Meets or Exceeds Specifications".  This will be considered a non-responsive response.

  - Letters of Clarification are to be on company (Bidder’s) letterhead and be signed.  The only information in the Letters of Clarification should be the explanation(s) of deviation(s) and/or responses to asterisked (*) items.  Asterisked (*) item explanations should refer to that particular specification item number.

  - Letters of disclosure and clarification must be made part of the sealed bid.

- COMPLIANCE WITH ALL GOVERNMENT REGULATIONS:  The bidder must comply with all applicable federal, state, and borough regulations, codes, and laws, and pay all applicable federal, state, and borough taxes, and is liable for all required insurance, licenses, permits, and bonds.  Failure to comply with such requirements shall constitute a breach of contract and shall be grounds for contract cancellation.  Damages or costs resulting from noncompliance shall be the sole responsibility of the bidder.

- CONFLICT OF INTEREST: An officer or employee of the State of Alaska may not seek to acquire, be a party to, or possess a financial interest in, this contract if  (1) the officer or employee is an employee of the administrative unit that supervises the award of this contract; or  (2) the officer or employee has the power to take or withhold official action so as to affect the award or execution of the contract.

- CONSOLIDATION OF AWARDS: Due to high administrative costs associated with processing of purchase orders, a single low bid of $50 or less may, at the discretion of the State, be awarded to the next low bidder receiving other awards for consolidation purposes.  This paragraph is not subject to the protest terms enumerated in “INSTRUCTION TO BIDDERS”, “FILING A PROTEST” above.

- CONSUMER ELECTRICAL PRODUCT: AS 45.45.910 requires that "...a person may not sell, offer to sell, or otherwise transfer in the course of the person's business a consumer electrical product that is manufactured after August 14, 1990, unless the product is clearly marked as being listed by an approved third party certification program."  Electrical consumer products manufactured before August 14, 1990, must either be clearly marked as being third party certified or be marked with a warning label that complies with AS 45.45.910(e).  Even exempted electrical products must be marked with the warning label.  By signature on this bid the bidder certifies that the product offered is in compliance with the law.  A list of approved third party certifiers, warning labels and additional information is available from: Department of Labor and Workforce Development, Labor Standards & Safety Division, Mechanical Inspection Section, P.O. Box 107020, Anchorage, Alaska  99510-7020, (907)269-4925.

- CONTINUING OBLIGATION OF CONTRACTOR: Notwithstanding the expiration date of a contract resulting from this ITB, the contractor is obligated to fulfill its responsibilities until warranty, guarantee, maintenance and parts availability requirements have completely expired.

- CONTRACT ADMINISTRATION:  The administration of this contract, including any/all changes, is the responsibility of the Contracting Officer, HQ State Equipment Fleet.

- CONTRACT EXTENSION: Unless otherwise provided in this ITB, the State and the successful bidder/contractor agree:  (1) that any holding over of the contract excluding any exercised renewal options, will be considered as a month-to-month extension, and all other terms and conditions shall remain in full force and effect and (2) to provide written notice to the other party of the intent to cancel such month-to-month extension at least thirty (30) days before the desired date of cancellation.

- CONTRACT FUNDING: Bidders are advised that funds are available for the initial purchase and/or the first term of the contract.  Payment and performance obligations for succeeding purchases and/or additional terms of the contract are subject to the availability and appropriation of funds.

- DEFAULT:  In case of bidder default, the State may procure the goods or services from another source and hold the bidder responsible for any resulting excess costs and may seek other remedies under law or equity.  Alaska Statutes and Regulations provide for suspension and disbarment of non-responsible bidders.

- DELIVERY:  All deliveries shall be F.O.B. final destination point with all transportation and handling charges paid by bidder.  Responsibility and liability for loss or damage shall remain with bidder until final inspection and acceptance when responsibility shall pass to the State except as to latent defects, fraud and bidder's warranty obligations.

- DISCONTINUED ITEMS:  In the event an item is discontinued by the manufacturer during the life of the contract, another item may be substituted, provided that the contracting officer makes a written determination that it is equal or better than the discontinued item and provided that it is sold at the same price or less than the discontinued item.

- DISPUTES: Any dispute arising out of this agreement shall be resolved under the laws of Alaska.  Any appeal of an administrative order or any original action to enforce any provision of this agreement or to obtain any relief from or remedy in connection with this agreement may be brought only in the superior court for the State of Alaska.

- EMPLOYMENT PROGRAM PREFERENCE: If a bidder qualifies for the Alaskan bidder's preference, under AS 36.30.170(b), and is offering goods or services through an employment program, as defined under 36.30.990(10), and is the lowest responsive and responsible bidder with a bid that is no more than fifteen percent (15%) higher than the lowest bid, the procurement officer will make the award to that bidder, in accordance with AS 36.30.170(c) and 2 AAC 12.050.

- EMPLOYERS OF PEOPLE WITH DISABILITIES PREFERENCE: If a bidder qualifies for the Alaskan bidder's preference, under AS 36.30.170(b), and, at the time the bid is submitted, employs a staff that is made up of fifty percent (50%) or more people with disabilities, as defined in AS 36.30.170(j), and submits a responsive and responsible bid that is no more than ten percent (10%) higher than the lowest responsive and responsible bid, the procurement officer will make the award to that bidder, in accordance with AS 36.30.170(f).

- EXTENSION OF PRICES: In case of error in the extension of prices in the bid, the unit prices will govern;

in a lot bid, the lot prices will govern.

- FILING A PROTEST: A bidder may protest the award of a contract or the proposed award of a contract for supplies, services, or professional services.  The protest must be filed in writing and include the following information: (1) the name, address, and telephone number of the protester; (2) the signature of the protester or the protester's representative; (3) identification of the contracting agency and the solicitation or contract at issue; (4) a detailed statement of the legal and factual grounds of the protest, including copies of relevant documents; and (5) the form of relief requested.  Protests will be treated in accordance with Alaska Statutes (AS)36.30.560-36.30.610.

- FIRM OFFER: For the purpose of award, offers made in accordance with this ITB must be good and firm for a period of ninety (90) days from the date of bid opening.

- FIRM, UNQUALIFIED AND UNCONDITIONAL OFFER:  Bidders must provide enough information with their bid to constitute a definite, firm, unqualified and unconditional offer.  To be responsive a bid must constitute a definite, firm, unqualified and unconditional offer to meet all of the material terms of the ITB.  Material terms are those, which could affect price, quantity, quality, or delivery.  Also included, as material terms are those, which are clearly identified in the ITB and which, for reasons of policy, must be complied with at risk of bid rejection for non-responsiveness.

- FORCE MAJEURE (Impossibility to perform):  Neither party to this contract shall be held responsible for delay or default caused by acts of God and/or war, which is beyond that party's reasonable control.  The State may terminate this contract upon written notice after determining such delay or default will reasonably prevent successful performance of the contract.

- FORMAL WORKSHOP:

- The Formal Workshop (Pre-Bid) is to discuss with prospective bidders the "draft" Invitation to Bid (ITB).  The formal workshop is the only opportunity for prospective bidders, manufacturers and other interested parties, in a conference setting, to discuss Standard Terms and Conditions, Special Terms and Conditions, Specifications, and to provide clarification as needed for this ITB.

- Formal workshops are for information only.  Any statements made at formal workshops will not be official until verified in the "final" ITB.  Participation by teleconference may be possible by making arrangements with the SEF Contracting Office in advance.

- HUMAN TRAFFICKING:

  - By signature on this contract, the offeror certifies that:

    - the offeror is not established and headquartered or incorporated and headquartered, in a country recognized as Tier 3 in the most recent United States Department of State’s Trafficking in Persons Report; or

  - The most recent United States Department of State’s Trafficking in Persons Report can be found at the following website:  www.state.gov/g/tip/

  - Failure to comply with this requirement will cause the state to reject the bid or proposal as non-responsive, or cancel the contract.

  - This pertains to goods and services above $50,000.00.

- INDEMNIFICATION:  The Contractor shall indemnify, hold harmless, and defend the contracting agency from and against any claim of, or liability for error, omission or negligent act of the Contractor under this agreement.  The Contractor shall not be required to indemnify the contracting agency for a claim of, or liability for, the independent negligence of the contracting agency. If there is a claim of, or liability for, the joint negligent error or omission of the Contractor and the independent negligence of the Contracting agency, the indemnification and hold harmless obligation shall be apportioned on a comparative fault basis. “Contractor” and “Contracting agency”, as used within this and the following article, include the employees, agents and other contractors who are directly responsible, respectively, to each.  The term “independent negligence” is negligence other than in the Contracting agency’s selection, administration, monitoring, or controlling of the Contractor and in approving or accepting the Contractor’s work.

- INSPECTIONS:  Goods furnished under this contract are subject to inspection and test by the State at times and places determined by the State.  If the State finds goods furnished to be incomplete or not in compliance with bid specifications, the State may reject the goods and require bidder to either correct them without charge or deliver them at a reduced price, which is equitable under the circumstances.  If bidder is unable or refuses to correct such goods within a time deemed reasonable by the State, the State may cancel the order in whole or in part.  Nothing in this paragraph shall adversely affect the State's rights as buyer, including all remedies and rights granted by Alaska statutes.

- INSURANCE:

  - Without limiting Contractor's indemnification, it is agreed that Contractor shall purchase at its own expense and maintain in force at all times during the performance of services under this agreement the following policies of insurance.  Where specific limits are shown, it is understood that they shall be the minimum acceptable limits. If the Contractor's policy contains higher limits, the state shall be entitled to coverage to the extent of such higher limits.  Certificates of Insurance must be furnished to the contracting officer prior to beginning work and must provide for a notice of cancellation, non-renewal, or material change of conditions in accordance with policy provisions.  Failure to furnish satisfactory evidence of insurance or lapse of the policy is a material breach of this contract and shall be grounds for termination of the Contractor's services. All insurance policies shall comply with, and be issued by insurers licensed to transact the business of insurance under AS 21.

  - Proof of insurance is required for the following:

    - Workers' Compensation Insurance:  The Contractor shall provide and maintain, for all employees engaged in work under this contract, coverage as required by AS 23.30.045, and; where applicable, any other statutory obligations including but not limited to Federal U.S.L. & H. and Jones Act requirements.  The policy must waive subrogation against the State.

    - Commercial General Liability Insurance:  covering all business premises and operations used by the Contractor in the performance of services under this agreement with minimum coverage limits of $300,000 combined single limit per occurrence.

    - Commercial Automobile Liability Insurance:  covering all vehicles used by the Contractor in the performance of services under this agreement with minimum coverage limits of $300,000 combined single limit per occurrence.

  - Failure to supply satisfactory proof of insurance within the time required will cause the State to declare the bidder nonresponsive and to reject the bid.

- INVITATION TO BID (ITB) REVIEW: Bidders shall carefully review this ITB for defects and questionable or objectionable material.  Bidders' comments concerning defects and questionable or objectionable material in the ITB must be made in writing and received by the purchasing authority at least ten (10) days before the bid opening date.  This will allow time for an amendment to be issued if one is required.  It will also help prevent the opening of a defective bid, upon which award cannot be made, and the resultant exposure of bidders' prices.  Bidders' original comments should be sent to the purchasing authority listed on the front of this ITB.

- ITEM UPGRADES:  The State reserves the right to accept upgrades to models on the basic contract when the upgrades improve the way the equipment operates or improve the accuracy of the equipment.  Such upgraded items must be at the same price as the items in the basic contract.

- LOCAL AGRICULTURAL AND FISHERIES PRODUCTS PREFERENCE: When agricultural, dairy, timber, lumber, or fisheries products are purchased using state money, only those products harvested in Alaska, or in the case of fisheries products harvested or processed within the jurisdiction of Alaska, will be purchased, provided they are available, of comparable quality, and priced no more than seven percent (7%) higher than products harvested outside the state, or in the case of fisheries products harvested or processed outside the jurisdiction of the state, in accordance with AS 36.15.050.

- METHOD OF AWARD:

  - DOT/PF is required by law to award “to the lowest responsible and responsive bidder whose bid conforms in all MATERIAL respects to the requirements and criteria set out in the invitation to bid.” AS 36.30.170.

  - All specifications in this ITB are important and bidders should carefully address each item.  Deviation from a specification MAY result in rejection of a bid at the discretion of the Contracting Officer if the deviation is material.  A material deviation is one that is significantly different from an essential aspect of a specification.

  - The Contracting Officer may be justified in accepting a non-material deviation - it will be determined upon the facts, circumstances, and the proposal in the bid.  If a bid deviation has a negligible effect on price, quality, performance, delivery or contract conditions, it is not material and may be waived or corrected by the Contracting Officer.  The State reserves the right to waive minor informalities or technical defects, which are not material, when such waiver is in the best interest of the State.

  - Nothing in this section shall be construed as depriving the State of its discretion in the matter of determining the lowest responsible bidder.

  - A Notice of Intent to Award (ITA) will be issued for all solicitations in accordance with regulations.  The notice of intent to award does not constitute a formal award of a contract.  Formal award is achieved through execution of the "Offer and Contract Award" section of the Invitation to Bid.

- NEW EQUIPMENT: Equipment offered in response to this ITB must be new equipment.  New equipment means equipment that is currently in production by the manufacturer and is still the latest model, edition or version generally offered.  The equipment must be warranted as new by the manufacturer and may not have been used for any purpose, other than display (not demonstration), prior to its sale to the State.  The State will not accept remanufactured, used or reconditioned equipment, including used or reconditioned components or parts of.  It is the contractor’s responsibility to ensure that each piece of equipment delivered to the State complies with this requirement.  A contract’s failure to comply with this requirement will cause the State to seek remedies under breach of contract.

- NON-RESPONSIVE BIDS:

  - Bids will be considered non-responsive and shall be rejected for the following:

    - Failure to sign the bid.

    - Failure to submit the bid at the place and time required in the solicitation.

    - Failure to have a valid Alaska Business License at the time of bid opening.

    - Failure to acknowledge amendments requiring signature.

    - Failure to provide product literature, brochures, or a letter of clarification, when specifically required by the Invitation to Bid.

    - Bids which specify "Offered Delivery Time" in excess of "Required Delivery" number of calendar days after the receipt of an order.

- ORDER DOCUMENTS: Except as specifically allowed under this ITB, an ordering agency will not sign any vendor contract.  The State is not bound by a vendor contract signed by a person who is not specifically authorized to sign for the State under this ITB.  The State of Alaska Purchase Order, Contract Award and Delivery Order are the only order documents that may be used to place orders against the contract(s) resulting from this ITB.

- PAYMENT:  Payment for agreements under $500,000 for the undisputed purchase of goods or services provided to a State agency will be made within 30 days of the receipt of a proper billing or the delivery of the goods or services to the location(s) specified in the agreement, whichever is later.  A late payment is subject to 1.5% interest per month on the unpaid balance.  Interest will not be paid if there is a dispute or if there is an agreement, which establishes a lower interest rate or precludes the charging of interest.

- PREFERENCE QUALIFICATION LETTER: Regarding preferences 5, 6, and 7 above, the Division of Vocational Rehabilitation in the Department of Labor and Workforce Development maintains lists of Alaskan; [1] employment programs that qualify for preference, [2] individuals who qualify for preference as Alaskan's with disabilities, and, [3] employers who qualify for preference as employers of people with disabilities.  In accordance with AS 36.30.170(j), in order to qualify for one of these preferences, a bidder must add value by actually performing, controlling, managing, and supervising the services provided, or for supplies, a bidder must have sold supplies of the general nature solicited to other state agencies, governments, or the general public.

As evidence of an individual's or a business' right to a certain preference, the Division of Vocational Rehabilitation will issue a certification letter.  To take advantage of the preferences 5, 6, or 7 above, an individual or business must be on the appropriate Division of Vocational Rehabilitation list, at the time the bid is opened, and must provide the procurement officer a copy of their certification letter.  Bidders must attach a copy of their certification letter to their bid.  The bidder's failure to provide the certification letter mentioned above, with their bid, will cause the State to disallow the preference.

- PRICE EXTENSION:  In case of error in the extension of prices in the bid, the unit prices will govern.

- PRICES:  The bidder shall state prices according to the requirements of this ITB.  Prices quoted for commodities or services must be in U.S. funds and include applicable federal duty, brokerage fees, packaging, and transportation cost to the FOB point so that upon transfer of title the commodity or service can be utilized without further cost.

- PRODUCT EQUIVALENT SUBSTITUTION:  Bidders may submit bids for substantially equivalent products unless the Invitation to Bid provides that a specific brand is necessary because of compatibility requirements. In those instances where the ITB specifies "a name brand or State approved equivalent", bidders must request approval of a product equivalent substitution NO LATER THAN 10 days prior to the bid opening date.  A written response will be provided by the Contracting Officer.

- PRODUCT IDENTIFICATION:  Bidders must clearly identify all products bid.  Brand name and model or number must be shown for the prime unit as well as attachments where applicable.  The State reserves the right to reject any bid when the product information submitted with the bid is incomplete and/or is not easily ascertainable from the bid documents or through independent verification.

- PRODUCT LITERATURE:

  - ANNOTATED LITERATURE:  Bidders must annotate their product literature to identify for the State the location of the supporting information regarding each product specification set out in this ITB.  A bidder's failure to comply with this clause, within the time set by the State, will cause the State to consider the offer non-responsive and reject the bid.

  - Product brochures, descriptive literature and other supporting documentation may be required in some solicitations.  Descriptive literature is for the purpose of providing a reference to the product offered.  While it is not practicable to require documentation to support every single item of a specification, it is sometimes necessary to require the bidder to provide product literature that will help the Contracting officer to determine the bidder's compliance with important specifications.

  - Requirements for product literature will vary from bid to bid. Certain specification items may be marked with an asterisk (*) indicating that descriptive literature or supporting documentation (such as a letter of clarification) will be required.

  - When product literature is required by the ITB, please provide a minimum of one (1) complete set to aid in bid review and evaluation.

  - Failure to provide product literature or brochures when required WILL result in your bid being deemed non-responsive.

  - A bidder's signature on the contract offer and award is taken as a promise to comply with the ITB specifications.  In case of a difference between the literature and the specifications, the specifications will govern.

  - The State does not honor disclaimer statements often evident in manufacturer's published specification sheets or brochures.  Such disclaimers will not release the successful bidder from providing the specified item, at the time of acceptance by the State.

- PROPRIETARY INFORMATION AND STATEMENTS OF CONFIDENTIALITY:

  - Except as set forth in this provision, all information in all bids will be made public under AS 36.30.530 not later than the time of issuance of a notice of intent to award.

  - If the offeror submits information considered by it to constitute a trade secret or proprietary data, such information may be expressly designated as such, and must be accompanied by the offeror's certification that (1) the information has consistently been maintained by its owner as a trade secret or as proprietary information, (2) the owner of the information has taken due care to protect it from release to non-privileged persons, and  (3) to the best knowledge of the offeror, the information has not lost its status as trade secret or proprietary information, whether by lack of diligent protection, release to any non-privileged person or otherwise.

  - Absence of such certification, any claim of confidentiality will be ignored, and the bidder may not hold any reasonable expectation of confidentiality.

  - Any information so certified will be held confidential so long as the contracting officer concurs that it constitutes a trade secret or proprietary data, and deems it not critical to determination of the price, quantity, or delivery terms bid, or the responsiveness of the bid.

  - By submission of a bid, the offeror consents to the contracting officer’s exercise of reasonable judgment as to concurrence with any assertion of confidentiality, and waives any and all claims for release of information that the contracting officer reasonably deems not confidential notwithstanding a certified assertion of confidentiality.

  - A certified assertion of confidentiality in which the contracting officer concurs, with respect to information the contracting officer deems critical to determination of the price, quantity, or delivery terms bid, or the responsiveness of the bid, will cause the bid to be rejected as a non-responsive bid.

- PROTESTS:  A bidder may protest the Intent to Award a contract pursuant to this Invitation to Bid in accordance with provisions in Alaska Statute AS 36.30.560.  The protest must be filed in writing within ten (10) days of the notice of Intent to Award, and must include the following information:  (1) the name, address, and telephone number of the protester; (2) the signature of the protester or the protester's representative; (3) identification of the contracting agency and the solicitation or contract at issue; (4) a detailed statement of the legal and factual grounds of the protest, including copies of relevant documents; and (5) the form of relief requested.  Protests must be filed with:  Contracting officer, State Equipment Fleet Purchasing, 2200 East 42nd Ave, Room #311, Anchorage, AK  99508.  Protests and any subsequent appeal will be processed per AS 36.30.

- QUANTITIES:  The State reserves the right to reduce or increase the quantity of items ordered under any contract resulting from this Invitation to Bid.

- REJECTION OF BIDS:  All bids or proposals may be rejected prior to award for the following reasons:

  - the supplies or services being procured are no longer required;

  - ambiguous or otherwise inadequate specifications were part of the solicitation;

  - the solicitation did not provide for consideration of all factors of significance to the State;

  - prices exceed available money and it would not be appropriate to adjust quantities to accommodate available money;

  - all otherwise acceptable bids or proposals received are at unacceptable prices;

  - there is reason to believe that the bids or proposals may not have been independently arrived at in open competition, may have been collusive, or may have been submitted in bad faith; or

  - the award is not in the best interests of the State.

- SEVERABILITY: If any provision of the contract is declared by a court to be illegal or in conflict with any law, the validity of the remaining terms and provisions will not be affected; and, the rights and obligations of the parties will be construed and enforced as if the contract did not contain the particular provision held to be invalid.

- SHIPPING DAMAGE:  The State will not accept or pay for damaged goods.  The contractor must file all claims against the carrier(s) for damages incurred to items in transit from the point of origin to the ultimate destination.  The State will provide the contractor with written notice when damaged goods are received.

- SIGNATURE ON BID:  BIDS MUST BE SIGNED IN INK BY AN OFFICIAL OF THE BIDDING ORGANIZATION authorized to bind the bidder to its provisions.  Signature certifies, under penalty of perjury under the laws of the United States, that the price(s) submitted was/were independently arrived at without collusion, and that neither bidder nor bidder’s firm, association or corporation of which bidder is a member, has either directly or indirectly, entered into any agreement, participated in any collusion, or otherwise taken any action in restraint of free competitive bidding in connection with this bid per 2AAC12.800.

- STANDARD AND SPECIAL TERMS AND CONDITIONS:  The terms and conditions of this section are standard to State of Alaska, Department of Transportation and Public Facilities, Statewide Equipment Fleet contracts for the purchase of goods.  There may also be other special terms and conditions in an Invitation to Bid or Request for Proposal which apply only to this contract.  In the event of a conflict between the standard and special terms and conditions, the Special Terms and Conditions take precedence.

- SUBCONTRACTOR(S): Within five (5) working days of notice, the apparent low bidder must submit a list of the subcontractors that will be used in the performance of the contract.  The list must include the name of each subcontractor and the location of the place of business for each subcontractor and evidence of each subcontractor's valid Alaska business license.  Subcontractors can only be changed per AS 36.30.115 (b).

- SUBMITTING BIDS: Envelopes containing bids must be sealed, marked, and addressed as shown in the

example below.  Do not put the ITB number and opening date on the envelope of a request for bid information.

Envelopes with ITB numbers annotated on the outside will not be opened until the scheduled bid opening.

Bidder’s Return Address	STAMP

Department of Transportation & Public Facilities

HQ State Equipment Fleet

2200 E. 42nd Avenue, Room #311

### Anchorage, AK  99508

### Bid No.: SEF- ______

Opening Date:_____________

- FORMAL WORKSHOP: (Attendance is Mandatory IF a workshop is scheduled)

  - The Formal Workshop (Pre-Bid) is to discuss with prospective bidders the "draft" Invitation to Bid (ITB).  The formal workshop is the only opportunity for prospective bidders, manufacturers and other interested parties, in a conference setting, to discuss Standard Terms and Conditions, Special Terms and Conditions, Specifications, and to provide clarification as needed for this ITB.  All bidders (a representative from the bidding company) must attend in person or by teleconference in order to bid.

  - Formal workshops are for information only.  Any statements made at formal workshops will not be official until verified in the "final" ITB.  Participation by teleconference may be possible by making arrangements with the SEF Contracting Office in advance.

- TIMELINESS OF BID:  Sealed bids must be received and time-stamped by the Procurement Office prior to the time set for receipt of bids.  Bids received after bid-closing time will not be considered.  Bidders must ensure that their bid and any amendments requiring signature are delivered prior to the scheduled bid opening time.  LATE BIDS WILL NOT BE ACCEPTED.

- FORMAL WORKSHOP: (Attendance is Mandatory IF a workshop is scheduled)

  - The Formal Workshop (Pre-Bid) is to discuss with prospective bidders the "draft" Invitation to Bid (ITB).  The formal workshop is the only opportunity for prospective bidders, manufacturers and other interested parties, in a conference setting, to discuss Standard Terms and Conditions, Special Terms and Conditions, Specifications, and to provide clarification as needed for this ITB.  All bidders (a representative from the bidding company) must attend in person or by teleconference in order to bid.

  - Formal workshops are for information only.  Any statements made at formal workshops will not be official until verified in the "final" ITB.  Participation by teleconference may be possible by making arrangements with the SEF Contracting Office in advance.

- TIMELINESS OF BID:  Sealed bids must be received and time-stamped by the Procurement Office prior to the time set for receipt of bids.  Bids received after bid-closing time will not be considered.  Bidders must ensure that their bid and any amendments requiring signature are delivered prior to the scheduled bid opening time.  LATE BIDS WILL NOT BE ACCEPTED.

- SUCCESSORS IN INTEREST:  This contract shall be binding upon successors and assigns.

- SUITABLE MATERIALS:  All materials, supplies or equipment offered by a bidder shall be new, unused, of recent manufacture, and suitable for the manufacturer's intended purpose unless the specifications allow for used, rebuilt or remanufactured equipment.

- SUPPORTING INFORMATION:

  - The State strongly desires that bidders submit all required technical, specification, and other supporting information with their bid, so that a detailed analysis and determination can be made, by the contracting officer that the product offered meets the ITB specifications and that other requirements of the ITB have been met.  However, provided a bid meets the requirements for a definite, firm, unqualified, and unconditional offer, the State reserves the right to request supplemental information from the bidder, after the bids have been opened, to ensure that the products offered completely meet the ITB requirements.  The requirement for such supplemental information will be at the reasonable discretion of the State and may include the requirement that a bidder will provide a sample product(s) so that the State can make a first-hand examination and determination.

  - A bidder's failure to provide this supplemental information or the product sample(s), within the time set by the State, will cause the State to consider the offer non-responsive and reject the bid.

- TAXES:  Prices quoted in bids must be exclusive of federal, state, and local taxes.  If the bidder believes that certain taxes are payable by the State, the bidder may list such taxes separately, directly below the bid price for the affected item.  The State is exempt from Federal Excise Tax because articles purchased are for the exclusive use of the State of Alaska.

- TIMELINESS OF BID:  Sealed bids must be received and time-stamped by the Procurement Office prior to the time set for receipt of bids.  Bids received after bid-closing time will not be considered.  Bidders must ensure that their bid and any amendments requiring signature are delivered prior to the scheduled bid opening time.  LATE BIDS WILL NOT BE ACCEPTED.

- USE OF LOCAL FOREST PRODUCTS: In a project financed by state money in which the use of timber, lumber and manufactured lumber is required, only timber, lumber and manufactured lumber products originating in this state shall be used unless the use of those products has been determined to be impractical, in accordance with AS 36.15.010.

- WARRANTY:   Unless otherwise stated, all equipment shall be new and current model and shall carry full factory warranties.  Bidder warrants all goods delivered to be free from defects in labor, material and manufacture and to be in compliance with bid specifications.  All implied or expressed warranty provisions of the Uniform Commercial Code apply.  All warranties shall be for and benefit the State.

SECTION III – SPECIFICATIONS

SPECIFICATION #204-15WC4x2

4x2 Airport Shuttle Bus

April 29, 2011

GENERAL SPECIFICATION:

It is the purpose of this specification to describe a new, and of the latest manufacturer's current model and design 4x2 Paratransit vehicles with wheelchair lift.  Seating to include driver with passenger seating for 14 ambulatory persons and have the capability of a configuration change to allow driver with passenger seating for minimum of ten (10) ambulatory persons and a minimum of two (2) wheelchair positions and 4 folding seats to be utilized when wheelchair positions are not in use.  Refer to seating specifications and to the layout drawing at end of specifications.  To be equipped with one (1) passenger door, one (1) driver’s door, and (1) curbside wheelchair accessible door.

The vehicle is to provide maximum passenger appeal in appearance, comfort and safety combined with excellence in reliability, operating characteristics and economy of operation.  The vehicle shall be designed so that continuous operation can be performed without any additional alterations being necessary for extreme weather conditions as experienced in Anchorage, Alaska.

The vehicle is for service in routes, in town use with relatively frequent stops.  The vehicle should be capable of legal highway speeds, yet be able to negotiate grades of up to 15% (percent) with a standing load.  The vehicle shall afford features essential for safe, fast, efficient and comfortable operation by the conditions and adequate means for safe passenger movement.  The vehicle must be maneuvered easily in normal and heavy traffic.  The design of the body, chassis and equipment that the manufacturer proposes to furnish shall be such as to produce a vehicle of substantial and durable construction in all respects.

No advantage shall be taken by the manufacturer in the omission of any part or detail, which goes to make the bus complete and ready for service, even though such parts or detail are not mentioned in these specifications.  Unit to include all standard equipment and accessories as advertised in manufacturer's specification sheet of model offered, unless otherwise specified herein.

The vehicle must meet all Federal and Alaska State requirements for this type of vehicle in effect at the time of delivery.  The vehicle must meet the specifications in the Department of Transportation 49 CFR, Part 38 – Americans with Disabilities Act (ADA), Accessible Specifications for Transportation Vehicles – Subparts A and B.  The vehicle offered is not to require the driver to obtain a Commercial Driver’s License (CDL).

APPLICATION:

General-purpose use as a passenger and wheelchair passenger conveyance vehicle.  Varying terrain and weather conditions to minus 20 degrees Fahrenheit.

DOCUMENTATION REQUIRED WITH THE BID PACKAGE:

One (1) copy of manufacturer's product brochure is to be provided with bid package.

Users List:  The bidder will provide a user’s list of at least five (5) northern tier properties, of which at least three (3) must be U.S. properties, currently using similarly configured minibuses.  The user’s list shall include current contact names with phone numbers.

In addition, specifications marked with an asterisk (*) require supporting documentation in the form of a product brochure, manufacturer’s technical data sheet, or a letter of clarification, which must indicate specifically what the bidder intends to supply in regard to said items, and/or how the specifications will be met.  In order to help prevent technical errors, following each asterisked is space that may be used to address all of the asterisked items.  It is required that a letter of clarification or the space behind the asterisked items be used to supply the required information.

TYPICAL UNITS:

FORD E-450 Super Duty Cutaway.  Provided all of the following specifications are met.

- POWER TRAIN:

  - Engine:

    - (*)  Gasoline, V-10, 6.8 liter minimum.

    - Starting Aids:  Engine Block Heater:  One (1) each, immersion type, 110 volt AC, of highest wattage as provided by OEM engine manufacturer.

    - Alternator:  120-amp minimum.  Polygroove belt driven Motorcraft 140 AMP High Output.

    - Batteries:

      - Two (2) each, 12V-DC, minimum 1200 total CCA, maintenance free, sealed type.

      - Batteries to be accessible by a pull-out battery tray with stainless steel bearing slides.

      - Battery enclosure to be designed to prevent snow and dirt accumulation on batteries.

    - Anti-freeze:  Permanent type, affording protection to minus 50 degrees Fahrenheit.

  - Exhaust System:  Shall terminate near the rear of the vehicle on the driver’s side.

- DRIVE TRAIN:

  - 4x2 (2-wheel drive) configuration

  - Transmission:  Automatic, Torqshift 5-speed auto override with integral cooler.

  - Differential, Rear:

    - (*)  Capacity:  7,800 pounds minimum.

    - (*)  Ratio:  To provide a cruise speed of 65 MPH, minimum.

    - (*)  Limited slip or locking type. TRACTECH No-SPIN aftermarket is acceptable.

- CHASSIS:

  - (*)  GVWR:  Minimum of 14,500 pounds.

  - (*)  Wheelbase:  Minimum of 176 inches.

  - (*)  Front Suspension:  Minimum 4,600 pounds.

  - (*)  Rear Suspension:  Minimum 9,450 pounds.

    - Rear suspension is required to include a rear curbside spring spacer to eliminate sag caused by the additional wheelchair lift.

  - Steering:  Power assisted.

  - Brakes:  ABS braking system (front and rear).  Hydra-boost 4-wheel disc with FORD OEM 13.58 inch diameter rotors (front and rear).

  - Front stabilizer bar, to be FORD OEM 0.906 inch diameter minimum.

  - Rear stabilizer bar, to be FORD OEM 1.13 inch diameter minimum.

  - Shock Absorbers:  Heavy duty front and rear.

  - Tires and Wheels:

    - Seven (7) each, LT225/75Rx16E BSW "All Season", steel belted radials, mounted on OEM steel rims, minimum.

    - Spare tire and wheel (same as above) to be shipped loose.

  - Fuel Tank Capacity:  55 gallons usable minimum.

  - Tow Hooks: Two (2) each rear, bolted to frame.  Each tow hook of sufficient strength to tow fully loaded unit.

- GAUGES, INDICATORS, AND INSTRUMENTS:

  - Chassis to include the following, as a minimum:

    - Tilt steering.

    - Speedometer, in miles per hour.

    - Odometer, in miles.

    - Voltmeter.

    - Oil pressure gauge.

    - Engine coolant temperature gauge.

    - Fuel level gauge.

    - Parking brake warning light.

    - All instruments shall be easily accessible for maintenance and repair and shall be mounted so that each instrument is clearly visible to the driver.

    - Each instrument shall be sufficiently illuminated.

    - Windshield wipers, multiple speeds, with washers and OEM installed intermittent swipe feature.

    - Temperature/fan controls for front and rear heater.  Front and rear to be separately controlled.

    - Air conditioning, chassis OEM minimum.

- BODY:

  - Cab over engine, cut-away configuration:

  - Construction:

    - Body construction shall be manufacturer’s standard design for maximum structural integrity, passenger safety and body lift.

      - The front area of the bus, where the bus body joins the chassis, design and construction shall be done in a manner that smoothly blends the two differing assemblies into one.

      - All metal body parts and frame members shall be treated to prevent corrosion.

      - Body sides, rear and roof shall be thermally and acoustically insulated.

      - Body construction shall have sufficient rigidity to prevent vibration or flexing in service.

    - Understructure, including body, chassis, and wheel wells, shall be undercoated.

      - All conversion metal to be etched with rust inhibitors prior to application of undercoat, with a minimum three (3) year rustproof warranty.

        - (*)  To include 2-stage process consisting of Zinc Chromate anti-corrosion primer or similar and also WAXOYL or similar cavity protection using a fogging process for inside the tubular steel areas.

    - The cleaning, sealing, undercoating, priming, and/or painting process shall be those methods and products which are used as standard production and/or manufacturing techniques by the manufacturer.

      - Those products used by the manufacture in their techniques shall be acceptable unless specified otherwise in these specifications.

      - (*)  These processes and related products shall be submitted with the bid.

    - Welding of body understructure to the chassis frame is not acceptable.

    - Wheel wells shall be steel reinforced.

    - (*)  The bus being bid is to have passed the Federal Transit Administration’s Altoona Bus Research and Testing Center’s standards.  The apparent low bidder will be required to submit in writing, within seven (7) days after the Intent to Award has been issued, that they meet this required testing.

    - All exterior panels shall be close end riveted, welded or bonded to the body framing.  Screws may be used in connection with rivets; however, no exposed metal screws shall be permitted.  Rivets, if used, shall be flush or truss head type and not protrude above the panel surface.  Rivet spacing shall be in accordance with the best practice of the bus industry.

    - All panels shall be installed so that they will shed water.  The leading panel shall be lapped over the following panel and, in no case, shall the sealing of the panels be dependent on caulking alone.

    - The body shall be made tight to prevent leakage and thoroughly water tested.  The chosen vendor shall provide written documentation, at not later than delivery of the unit, outlining water test procedures including the results of these tests.

    - Rub Rail:  A full-length, heavy-duty, transit style consisting of two (2) inch rubber installed into an aluminum trim framework on each side of the vehicle to help protect the bus from damage from other vehicles or obstructions.

    - Insulation:

      - Walls and ceiling to be insulated to completely fill the void between exterior and interior panels.

      - Roof, sidewalls, front and rear crown’s insulation, minimum R6, shall be either VERTICEL composite or foam seal urethane insulation, and provide the most efficient insulation in an operating environment which ranges from minus 20 to plus 100 degrees in Fahrenheit.

      - The body shall also be well insulated against operating noises and vibrations and limit interior and exterior noise levels to 85 dba or less under full load conditions throughout the vehicle.

      - Insulation shall be provided in both walls and roof with a minimum of R6 rating.

  - Floor:

    - The entire floor shall be supported by a fully welded steel sub floor understructure.

    - The sub floor shall be equivalent to a minimum of three (3) inch “C” channel, 11 gauge steel with 16 gauge cross members.  Sub floor shall be securely welded to the cab section and bolted to the chassis frame.

    - Over the sub floor structure shall be bolted a sheet or sheets of marine grade plywood, minimum ¾ (0.75) inch thick, seven (7) ply.  Undercoated and sealed prior to and after installation.

    - Floor covering to be smooth RUBBER SOLUTIONS N.A. or  R.C.A. TRANSIT, minimum 1/8 (0.125) inch thick for areas under seats.  Entrance and isle to be ribbed rubber type, 3/16 (0.1875) inch thick minimum.  All seams to be completely sealed to help prevent moisture from access to sub floor.  Color to be black.

  - Interior:

    - Passenger area to provide 80 inches of headroom from floor to ceiling.

    - Interior panels shall be of aluminum or fiberglass reinforced plastic, and shall provide a durable finish.

      - Color to be coordinated to interior color scheme.

      - Aluminum shall have baked enamel or vinyl clad finish.

    - Passenger area headliner to be gel coated.

      - Color to be coordinated to interior color scheme.

    - Stanchions, Grab Handles and Modesty Panels:

      - Step well:

        - A vertical stanchion and modesty panel shall be installed in the entryway at the rear of the step well.

        - Stainless steel handrail shall be provided on both sides of the entry, accessible from the ground.

        - The handrails shall be capable of withstanding a force of 100 pounds concentrated at any point on the handrail with permanent deformation of the rail or its supporting structure.

      - A vertical stanchion and modesty panel shall be installed directly rear of the driver’s seat.

      - Contoured vertical grab handles shall be installed both right and left of the entryway, and shall be securely fastened to the interior of the doorframe.

      - Two (2) overhead hand rails shall be installed in the passenger seating area.

      - All stanchions and handrails shall be between 1-1/4 (1.25) and 1-1/2 (1.5) inch minimum diameter, textured stainless steel or powder coated aluminum and shall be securely fastened into structural members at all mounting points.  Stanchions shall not be mounted to sheet metal, fiberglass, or other non-reinforced areas.

      - All entry and exit stanchions and railing shall be yellow powder coated.  Fittings shall be stainless steel.  Painted, other than powder coating, stanchions and railings are not acceptable.

        - Exception is given to the wheelchair lift, which may be manufacturer’s standard.

      - (*)  Bidder shall provide a diagram with their bid package of all stanchions, handrails, and modesty panels.

  - Seating (Also refer to layout drawings at end of specification):

    - Seating will not exceed 15 persons, including driver.  The vehicle offered is not to require the driver to obtain a Commercial Driver’s License (CDL).

      - (*)  Seating to include one (1) driver with seating for 14 passengers and have the capability of a configuration change to allow driver with passenger seating for minimum of nine (9) persons and a minimum of two (2) wheelchair positions.  Bidder is to submit with their bid package a diagram showing seating arrangement.

      - Two (2) wheelchair positions with a four (4) point tie down systems. Must meet all ADA regulations. (Refer to "SPECIAL REQUIREMENTS" later in this specification for more details).

    - Driver’s Seat:

      - High back bucket with folding inside armrest.

      - Seat position shall be adjustable forward and rearward with tilt reclining and shall include mechanical adjustable lumbar back support.

      - Cloth covered, not vinyl, coordinated to interior color scheme.

    - Passenger Seats:

      - There is to be nine (9) each single passenger, fixed low back perimeter style seats (permanently placed) FREEDMAN SEATING SEATS, INC., low-back single contour seats.

      - As per the drawing, there is to be one (1) each 17 inch FREEDMAN Featherweight flip seat and two (2) each 34 inch FREEDMAN Featherweight flip seats.

        - These seats will fold-up to provide room for the two (2) wheelchair positions.

      - Cushion Depth:  Minimum 17 inches.

      - Aisle width to comply with minimum ADA requirements.

      - Upholstered with Level 5 fabric, treated for fading and stain resistant, meeting FMVSS 302 - Flammability of Interior Materials, coordinated to interior color scheme. Vinyl is not acceptable.

      - Two (2) each, 24-inch in length, seat belt extensions that are compatible with passenger seat belts and with the restraint system used in the wheelchair securement location, are to be supplied.

    - Wheelchair Positions:

      - There will be Two (2) permanent forward facing wheelchair positions.

      - Securement system shall conform in all respects to the requirements of the ADA and will secure the wheelchair to the vehicle and provide a seatbelt for the wheelchair passenger.

      - The system will include fully recessed “L” track front to rear, both sides of the position and will include wheelchair passenger lap and shoulder restraints.

        - Each track is to be minimum 48 inches in length.

      - All securement devices shall b adjustable to accommodate several types and sizes of wheelchairs.

      - Wheelchair station shall have four (4) point securement devices with anchor points of sufficient strength to secure a wheelchair.

      - No anchoring points or securement shall be attached to any door or walls of the vehicle.

      - Heavy-duty cam straps are preferred.

      - All restraint and securement devices shall be SURE-Lok or Q-Straint retractable self-tensioning belts

      - The fully recessed tracks shall be mounted flush to the floor with SAE grade-5 tapered head bolts and self-locking nuts of sufficient size to meet minimum force standards.

        - Bolts shall be mounted at five (5) inch intervals (maximum distance) with the bolt heads flush in the track to permit easy anchor movement along the track.

      - All wheelchair strap restraints shall meet recommended force requirements as specified in Guideline Specification for Wheelchair Securement Devices, US Department of Transportation and 49 CFR 38.23(d) which are hereby incorporated as part of these Technical Specifications.

      - There shall be a minimum of four (4) strap assemblies for each wheelchair position in the vehicle to secure the wheelchairs to the tie down track.  Each strap assembly shall consist of a fixed end strap, buckle, closed hood, “D” ring and end fitting.

        - One (1) each, additional strap assembly set shall be provided.

      - Each wheelchair securement location shall be equipped with passenger restraint lap and shoulder belts intended to hold the passenger in a secure seated position in normal operation.

        - Self-retractable belts are not acceptable.

        - Seatbelt assemblies shall lock into the wheelchair tie down track and have a rated strength of 2,500 pounds minimum.

      - Two (2) each, storage pouches shall be made and installed near the wheelchair securement areas for the storage of all seatbelt and strap assemblies.

        - These pouches shall not interfere with the flexible placement of wheelchairs in the various tie-down stations inside the vehicle.

      - Detailed written instructions with diagram(s) shall be supplied with detailed instructions on the proper operation, placement, and removal of wheelchair securement straps and seatbelts.

  - Wheelchair Lift:

    - Wheelchair lift shall be located on the right side (curbside) of the vehicle, rear of the rear drive axle.

    - (*)  BRAUN Model NCL919I-2B or RICON Model S5510.

    - To be fully automatic to independently lift all sizes of manual or electric wheelchairs into the vehicle.

    - Load capacity of 800 pounds, minimum.

    - All attachments will be done through the vehicle frame.  Securement of lift to vehicle shall be rigid with no flexing movement between lift frame and vehicle body.

    - Electrical connection of lift shall pass through an automatic circuit breaker at the battery end of the connection.

    - The wheelchair lift shall be completely operational while the vehicle engine is off and transmission in park.  The battery shall be sufficient to provide for a minimum of 12 complete cycles of the lift while the engine is off and there is 300 pounds on the lift.

    - Lift controls to operate shall be interlocked with the vehicle emergency brakes, wheelchair lift door and transmission to ensure that the vehicle cannot be moved when the lift is not stowed and the lift cannot be deployed unless the interlock is engaged.

    - For the wheelchair lift installed, the contractor shall furnish, at time of delivery:

      - A complete set of written operating instructions.

      - A complete trouble-shooting guide.

      - A factory warranty schedule of inspection and service.

      - The name, address and phone number of at least one (1) factory authorized mechanic who can service the lift within 50 miles of point of delivery.

      - A complete schematic color-coded wiring diagram for the lift and installation.

  - Glass/Windows:

    - Windshield, chassis manufacturer equipped with nontransparent sun visor.

    - Driver’s side window to be power or manual roll up/down.

    - Window(s) forward of passenger entrance door shall be body manufacturer’s standard for this model vehicle.  A one (1) piece full view curb window is preferable; however a two (2) piece window is acceptable.  If two (2) windows, the lower window shall be the largest rectangular available from the body manufacturer.

    - Passenger’s entry door windows to be manufacturer’s standard full glass or split glass (two-piece) located both halves of panels; minimum 31% (percent) tinted tempered glass.

    - Side passenger windows shall be 41” h x 29” w, T-bar sliding located at top of windows tinted approximately 31% (percent) luminous transmittance as per ASTM D-1003.  Window sizes may vary so long as the total square inches of glass do not decrease from stated sizes.

      - One (1) window on each side shall be emergency egress, kick-out type, and clearly marked as such.

    - Wheelchair lift door window(s) to be manufacturer’s standard.  If double doors, minimum of one window in each door.

    - Water deflection drip molding shall be installed over all windows.

  - Doors:

    - Driver’s door per OEM specification.

    - Passenger Entrance Door:

      - To be manufacturer’s standard double-leaf outward opening.

      - Electric articulated.

      - Dimensions:

        - 80 inches high x 40 inches wide with minimum 36 inch clear opening between assist rails of doors when open.

        - Full height, minimum.

      - The meeting edges of the door shall be equipped with manufacturer’s standard design material on each section that overlaps when doors are closed, forming a tight seal.

      - The entry door, when closed, shall match the contour of the bus body as closely as possible.

      - Door pivot points shall be fitted with easy access lubrication fittings or have sealed roller bearings.

      - Door shall be equipped with a manual emergency opening system.

      - There shall be an auxiliary weatherproof momentary push button or weatherproof metal toggle switch located in the chassis grill area or adjacent to entry door, easily accessed by maintenance personnel.

    - Wheelchair Lift Door:

      - Single or double, manufacturer’s standard with clear opening to be sufficient to accommodate specified wheelchair lift assembly, with “L” handle, two (2) point latch with key lock.

      - Door(s) will be held in the open position by individual gas cylinders.

      - The door(s) shall be located rear of the rear axle.

    - All doorframes shall be of stainless steel, 14-guage minimum, frame integrally welded to the floor frame and sidewall frame.

    - Door frames must we warranted against corrosion perforation for a minimum of 5 years or 75,000 miles whichever comes first.

    - Maintenance access doors shall be provided where necessary to service the engine, radiator, batteries, electrical fuses and relays, door controls, etc.

    - All doors shall be trimmed with channel trim on edge to help protect against damage or injury.

    - Water deflection drip molding shall be installed over all doors.  If the body design is such that dripping is not a problem at the passenger and wheel chair doors, then this requirement may be deleted for those two (2) areas only.

  - Step well:

    - Enclosed type.

    - The ground to first step height is to be 14 inches maximum (or as per ADA requirement).  If a motor home type step is utilized, it shall be heavy-duty and include safety grating to help eliminate snow and ice buildup.  A motor home type step, if utilized, would need to be automatic positioning when the passenger door is opened.

    - Step Width:  36 inches between assist rails.

    - Step Tread Depth:  Eight (8) inches minimum.  Step tread edges shall be flush with the step.

    - Step treads material to be heavy-duty, non-skid rubber.

      - Step treads to be 3/16 (0.1875) inch thick ribbed and include two (2) inch contrasting white nosing, properly sealed, on each step.

      - In addition, a 1/8 (0.125) inch thick smooth rubber cove molding shall be installed from the floor to the sidewall seat track.  Color to be determined after bid opening.

    - Step Risers:  Seven (7) inches minimum, 9-1/2 (9.5) inches maximum.

    - To be equipped with step well light as described later in this specification under “Special Lighting”.

    - Step well shall be of stainless steel or galvaneal steel (14-guage minimum), frame integrally welded to the floor frame and sidewall frame.  A composite design is acceptable.

    - Step well must we warranted against cracking or corrosion perforation for a minimum of five (5) years or 75,000 miles whichever comes first.

- SPECIAL REQUIREMENTS:

  - The van and equipment shall meet or exceed all ADA requirements in all aspects.

  - Instruction Plates and Markings:

    - Where important procedures, safety warnings, or instructions are required, instruction plates must be provided at locations applicable and readily visible.

    - NO SMOKING signs are to be placed at conspicuous locations inside the van.

    - The plates shall be of heavy plastic laminate and engraved in a manner that instructions cannot be removed or defaced.

      - DYMO type or stick on labels is not acceptable.

    - The instructions shall be of marked contrasting color from the background material.

    - The unit shall also have painted or decaled in a conspicuous location an international handicap emblem to indicate the van is ADA accessible.

  - Mirrors:

    - Exterior:

      - Motorized (both left-side and right-side).

      - To be a minimum size of 90 square inches (includes main and convex).

      - Mirror and brackets are to be stainless steel or anodized aluminum or powder coated.  Poly mirror housings are acceptable.

      - Main mirrors are to be motorized for minimum left/right and up/down adjustment.

      - A convex mirror (built-in or separate), approximately eight (8) inches in diameter (semi-rectangular is acceptable) is to be included directly below the main mirror (mounting in the main mirror’s housing or directly below the main mirror’s housing).

      - Right side (curbside) mirror may be fender mounted for easier viewing.  Bracing is to be adequate to prevent vibration of the mirrors.

      - Mirror heat (both main and convex) to be thermostatically controlled or include a separate, easily accessed, switch with an “ON” indicator light.

      - Controls for mirror movement to be easily accessed.

    - Interior:  Approximately 6 x 16 inches with up/down adjustment, located forward and above driver’s seat for viewing passengers.

  - Heating, Rear Passenger Area:

    - In addition to the chassis manufacturer’s heavy-duty front heater and defroster for the windshield and driver’s area, a minimum of one above-floor interior heater unit is to be provided and located so as to maintain a comfortable atmosphere for the passengers and the driver as described below.

      - Rear heater to be a minimum of 65,000 BTU.

      - The capacity of the heating system shall maintain a minimum of 60 degrees Fahrenheit inside the vehicle when operating in service at minus 20 degrees Fahrenheit.

      - In addition, the heating system must be capable of keeping condensation and frost off inside of the windows at ambient temperatures to minus 30 degrees Fahrenheit.

      - Heat is to be directed to the interior steps to help prevent ice buildup (heated air or electric pad type heaters may be utilized).  To be driver controlled.

      - An auxiliary circulation pump for the rear heaters is to be installed.

      - Heater hoses to be insulated wrapped and routed to prevent damage.  Heater hose insulation to be THERMOCEL or equivalent.

      - Roof and sidewall insulation to include maximum “R” value rating.

      - All heaters and defrost fans to be minimum two-speed, independently switched, and driver controlled.

  - Air Conditioning:

    - To be AMERICAN COOLING TECHNOLGY (ACT) 60,000 BTU or CARRIER AC-622MAX 60,000 BTU with 65,000 BTU heat strip in evaporator

  - Electrical:

    - Special Lighting:

      - To be provided at the interior roof and side wall locations to the following areas:

        - Driver's station courtesy light located in the driver area and shall be activated when driver’s door is opened or with head lamp switch.

        - Main passenger compartment lights mounted under seats equidistant throughout the vehicle.  Master switch for lights will be located on driver’s control panel.  To be LED.

        - Front passenger door and step well areas.  To be activated when door is opened.  In addition to the step well areas, an exterior light shall be included as per ADA.  To be LED.

        - Wheelchair lift area (inside and outside, above lift minimum), activated when door(s) open. To be LED.

        - Emergency egress window designation lights (LED), installed above each emergency egress window, red in color and activated with ignition switch in “ON” or “Accessory” position.

        - Interior Overhead Lighting:

          - To be LED lighting for the entire passenger compartment.

          - To activate when exit/entrance or wheel-chair door is open.  To also include a driver controlled activation switch.

      - Roof exterior marker lights, red rear and amber front and sides shall be LED, DIALIGHT or OPTRONICS or equivalent, located in uppermost areas of the front and rear caps of the vehicle to provide better visibility.

      - Stop, turn, tail and back-up lights are to be four (4) inch round LED, DIALIGHT or OPTRONICS or equivalent.

      - Headlights "ON", audible warning device for when key is turned off and headlight switch are left on (if required).

      - Exterior Strobe Lights:  Two (2), to be clear and mounted on the roof.  Double flash.

    - Wiring:

      - Accessory wiring to include rated breaker switches located for ease of maintenance.

      - To be color coded or continually numbered.

      - To be located for maximum protection from road splash, stone abrasion, grease, oil, fuel, and heat from engine and components.

      - Routing through structural members to be protected by grommets.

      - To be secured by clips at intervals to prevent rubbing or chafing due to movement.

      - The use of SCOTCHLOCK or similar electrical connectors is not acceptable.

      - Ground Plane and prewired for two way radio.

  - Bumpers:

    - Rear:  ROMEO RIM help bumper.

    - Tow Hooks:  Rear:  Two (2) each.

  - Warning Triangles:  To include one (1) set of three (3) each, collapsible warning triangles, complete with plastic storage box.  KING p/n 1005 or equivalent.

  - Backup Alarm:  Electronic, self-adjusting sound level, ACORN PRODUCTS Model 1D-112AA, PRECO Model Preco-Matic 1040, STAR Model Starmatic 63-000, or WARN Model Reactor 2100504, located on rear of unit per manufacturer’s recommendations.

  - Fire Extinguisher:  One (1) each, five (5) pound dry chemical with gauge, mounted with quick release mechanism in driver's area.

  - Luggage Rack:  90 inches in length with shelf to be installed behind the driver in the forward section of the passenger compartment, to be constructed of aluminum.

  - First Aid Kit:  24 units.

  - Illuminated Destination Signs:  Two (2). To be LED, LUMINATOR programmable (www.luminatormasstransit.com), Vista model.

    - One (1) sign to be located on the front and one (1) located on the curbside.

    - Sign to be pre-programmed with the following:

      - NORTH TERMINAL

      - SOUTH TERMINAL

      - REPUBLIC PARKING

      - AIRPORT SHUTTLE

      - OUT OF SERVICE

    - Driver controlled.

  - Running Board:  Drivers door.  Polished aluminum diamond plate, heavy-duty (no flex), 12 inches wide, full length.

- MATERIALS AND WORKMANSHIP:

  - Workmanship shall be of the best grade and shall conform in all respects to the best practices in the industry.

  - Welding procedures, welding materials, and qualifications of welders shall be in accordance with standards of the ASTM and the American Welding Society (AWS) or Canadian Welding Bureau (CWB).

    - All exposed welds shall be ground smooth after welding, if necessary, to present a smooth, workmanlike appearance.

    - Prior to welding, the contact surfaces shall be free of scale, grease, paint, and rust.

  - All materials used in construction of vehicle shall conform to ASTM, SAE, or similar association standards.  All materials shall be first quality.

  - All exposed bright metal shall be stainless steel, anodized aluminum, or chrome plated steel.

  - All exterior joints shall be protected and sealed by a non-shrinking sealant prior to assembly.

  - Where dissimilar metals are riveted or bolted together, contact surfaces shall be thoroughly cleaned and coated with heavy duty zinc chromate primer or butyl tape.

    - All rivets shall completely fill the holes.

    - Rivet heads externally shall be concentric with the body of the rivets and free from rings, pits and fins.

  - (*)  All bolts or rods passing through wood shall be cadmium, zinc plated, or stainless steel.  Where wood and wood are placed together, both surfaces shall be coated with powdered aluminum and spar varnish, linseed oil and titanium oxide, urethane sealer, or ISOCLAD prior to assembly.  Sealer/coating type must be submitted with bid.

  - All metal shall be thoroughly cleaned, acid etched, and painted, unless otherwise noted in this specification.

  - All burrs and sharp edges shall be dressed to prevent injury to passengers, operator and maintenance personnel.

  - All materials installed shall be new and free of rust.

  - Weather stripping shall have no coating of paint, adhesive or other alien material.

  - The body shall be free of all cracks, dents, and defects.

    - Special care shall be taken with the outside sheathing, roof, roof bonnets, and the interior finish so that all kinks and buckles are removed before assembly to present a true and smooth finish without excessive grinding off of the material so as to weaken the structure.

  - Understructure, including body, chassis, and wheel wells, shall be undercoated, using manufacturer’s standard process.

    - All conversion metal to be etched with rust inhibitors prior to application of undercoat, with a minimum three (3) year rustproof warranty.

  - The cleaning, sealing, undercoating, priming, and/or painting process shall be those methods and products which are used as standard production and/or manufacturing techniques by the manufacturer.

    - Those products used by the manufacture in their techniques shall be acceptable unless specified otherwise in these specifications.

    - These processes and related products shall be submitted with the bid.

  - All windows shall be free of any defects due to welding slag or assembly damage.

  - All hoses supplied in assembling the unit described in these specifications shall have PSI and temperature ratings in excess of maximum operating temperatures and pressures of fluids or materials being transferred.

  - All switch labeling and interior signs shall be permanent.  “DYMO” type labels are not acceptable.

  - All brackets, nuts, bolts and miscellaneous features for attaching front and rear license plates shall be provided.

  - Water Testing:  After vehicle is assembled and painted, Body Manufacturer shall thoroughly water test the vehicle for leakage.  All leaks shall be sealed or otherwise corrected before vehicle ships.

- MISCELLANEOUS:

  - Paint:  Exterior to be White and Navy Blue.  Refer to Attached Drawing (Vendor is not responsible for graphics or lettering, only white and navy blue paint of bus body).  To be DUPONT Spectra Mastek BS904 or equivalent.

  - Drive Cam recording system:  To include Wireless Event Recorder (DCII) and Hindsight S/W 5.4.  System to be shipped loose and will be installed by the State of Alaska.

  - Inspection:  For compliance to specifications will be held at FOB point.

DRAWINGS (Drawings are not to scale)

BID SCHEDULE INSTRUCTIONS:

All bids must include both unit pricing and extensions and be otherwise complete in the format requested.

DELIVERY:  Indicate, in the space(s) provided, the time required to make delivery after the receipt of an order (ARO).  Failure to make an entry in the space(s) provided will be construed as an offer to deliver within “Required Delivery” NUMBER of calendar days ARO.  Bids which specify deliveries in excess of “Required Delivery” NUMBER of calendar days ARO will be considered non-responsive and the bid(s) will be rejected.

The materials, equipment or services bid will conform to and meet the requirements of the Contract Bid Documents and are hereby incorporated into this document:

Complete the attached bid price sheets and return with other bid documents as required in Section II – Standard Terms and Conditions, paragraph 13.

Item 		Total

#        Unit	Description		Amount

1a	  1 ea	Airport Shuttle Bus	$

Year, Make & Model Offered:

1b	  1 ea	Publications

Per Section I, paragraph 8.0.

Total Price (1a and 1b) for Award Purposes   $

The actual F.O.B. point for all items purchased under this contract is the final destination anywhere within the State of Alaska.  Ownership of and title to the ordered items remains with the contractor until the items have been delivered at their final destination and are accepted by the State.

For pricing purposes, the F.O.B. point is dockside Seattle/Tacoma.

The cost of shipping and delivery for orders beyond the limits of Seattle/Tacoma dock will be handled as follows.  The contractor will prepay the shipping and delivery charges to any destination named by the State in its order.  The contractor will charge-back those shipping and delivery charges to the State as a separate line item on the State’s invoice.  All shipping charges over $100 must be documented by a copy of the actual shipping invoice and received with the invoice charge to the State.

Required Delivery:  Maximum 180 days after receipt of order (ARO).

Offered Delivered Time:        Days ARO.

THIS IS NOT AN ORDER

SEALED BIDS MUST BE SUBMITTED TO THE STATE EQUIPMENT FLEET PRIOR TO BID OPENING AT WHICH THEY WILL BE PUBLICLY OPENED.

IMPORTANT NOTICE:  If you received this solicitation from the State’s “Online Public Notice” web site, you must register with the Procurement Officer listed on this document to receive subsequent amendments.  Failure to contact the Procurement Officer may result in the rejection of your offer.

BIDDER'S NOTICE:  By signature on this form, the bidder certifies that:

- the bidder has a valid Alaska business license, submits a bid under the name as appearing on the person’s current Alaska business license and has written the license number below or has submitted one of the following forms of evidence of an Alaska business license with the bid:

- a canceled check for the business license fee;

- a copy of the business license application with a receipt date stamp from the State's business license office;

- a receipt from the State’s business license office for the license fee;

- a copy of the bidder’s valid business license;

- a sworn notarized affidavit that the bidder has applied and paid for a business license;

- the price(s) submitted was arrived at independently and without collusion and that the bidder is complying with:

- the laws of the State of Alaska;

- the applicable portion of the Federal Civil Rights Act of 1964;

- the Equal Employment Opportunity Act and the regulations issued thereunder by the State and Federal Government;

- all terms and conditions set out in this Invitation to Bid (ITB).

If a bidder fails to comply with (1) at the time designated in the ITB for opening the State will reject the bid.  If a bidder fails to comply with (2) of this paragraph, the State may reject the bid, terminate the contract, or consider the contractor in default.  Bids must be submitted under the name as appearing on the person’s current Alaska business license in order to be considered responsive.

If a contract award is made by the State of Alaska, the undersigned offers and agrees to furnish materials, equipment and services in compliance with all terms and conditions specified in ITB/Contract.  Submittal of this document with authorized signature constitutes full understanding and a promise to comply with all specifications, terms and conditions of ITB/Contract.

| TO BE COMPLETED BY BIDDER: COMPANY SUBMITTING BID ____________________________ AUTHORIZED SIGNATURE PRINTED NAME DATE | ADDRESS CITY, STATE, ZIP PHONE/FAX E-MAIL ADDRESS | ALASKA BUSINESS LICENSE NO: DOES YOUR BUSINESS QUALIFY FOR PREFERENCES? YES NO DOES YOUR BUSINESS QUALIFY FOR THE VETERANS PREFERENCES? YES NO |
| --- | --- | --- |

Revised 03/14/2012