State Of Alaska

Department of Transportation & Public Facilities

State Equipment Fleet

2200 E 42nd Avenue

Anchorage, AK 99508

Invitation to Bid SEF-1884

Title of ITB: Contract for Paratransit Vehicles

Date of Issue: November 25, 2013

Bid Opening Date: December 17, 2013 @ 2:30PM

Contracting Officer: Anne Ritala

Phone: (907) 269-0791

E-Mail: anne.ritala@alaska.gov

CLICK HERE TO REGISTER WITH PROCUREMENT OFFICER

Offerors Are Not Required To Return This Form.

Important Notice: If you received this solicitation from the State of Alaska’s “Online Public Notice” web site, you must register with the procurement officer listed in this document to receive subsequent amendments. Failure to contact the procurement officer may result in the rejection of your offer.

- NOTE:  This section will be used for Terms and Conditions that are "special" and/or unique to a particular ITB.  The paragraphs in this section may change from bid to bid.  Accordingly, bidders must review this section to insure compliance.

- INTENT:  This invitation to bid (ITB) is intended to result in one contract award of 4x2 paratransit vehicles with multiple floor plan options and the option of 4x4:

  - Contract Period: 1 year, with optional (2) two 1 year extensions

  - Quantities: 2 paratransit vehicles at time of initial contract

  - Location of Use: Statewide

  - Warranty locations: At a minimum to be in Anchorage and Fairbanks

  - In addition to the State of Alaska requirements, the Municipality of Anchorage and other Alaska political subdivisions may cooperatively purchase from the resulting contract.

    - At no time may the contractor change the terms and conditions, alter the price to another entity, which differs from the contractual price, nor charge undisclosed administrative fees to allow cooperative purchasing.

- DELIVERY:

  - Pre-delivery service:  Prior to delivery, each vehicle, piece of equipment or attachment shall be serviced and inspected by the dealer or his agent.  Inspection must include the following (as applicable to the type of equipment):

    - Dealer and vehicle identification.

    - Check-off of service and inspection performed including a list of all fluids including type weight and specification that are in the equipment as delivered for all fluid compartments.

    - The vehicle's crankcase, differential and transmission, and other fluid compartments shall be filled to the manufacturer's recommended capacity.

    - Fuel tank shall be filled to at least register a minimum ¼ full on the fuel gauge, unless restricted by the commercial carrier, when the vehicle arrives at the delivery location.

    - The vehicle shall be clean and free from defects when delivered and should be ready for immediate and continued use upon delivery.

    - Units delivered in an incomplete state, or which have deficiencies per the specification, are subject to the damage charges as noted in paragraph 4.0 below.

  - Inspections:

    - The State's inspection of all materials and equipment upon delivery is for the sole purpose of identification.  Such inspection shall not be construed as final or as acceptance of the materials or equipment if materials or equipment do not conform to Contract requirements.  If there are any apparent defects in the materials or equipment at the time of delivery, the State will promptly notify the Contractor thereof.  Without limiting any other rights of the State, The State at its option, may require the Contractor to:

      - repair or replace at contractor's expense, any or all of the damaged goods,

      - refund the price of any or all of the damaged goods, or

      - accept the return of any or all of the damaged goods.

    - Costs of remedying all defects, indirect and consequential costs of correcting same, and/or removing or replacing any or all of the defective materials or equipment will be charged against the bidder.

  - Acceptance:

    - Units will not be considered “Accepted” until all deficiencies have been corrected.  This includes item 2.5 Line Sheets/Bill of Materials and 8.0 Publications.

  - Delivery Receipt:

    - A delivery receipt will be required.  The receipt must be filled out by the vendor, and acknowledged by state receiving personnel by signature and date of actual receipt of equipment.  One copy of this delivery receipt is to be given to the state-receiving agency.

    - Vendors are cautioned and advised that such delivery forms or other receiving type documents will not in any way be construed to mean the state has formally and fully accepted unit(s) referenced thereon as complete and meeting every specification set forth.  Only the Contracting Officer or designee may sign warranty documentation.

  - Line Sheets/Bill of Materials::

    - It is required at the time of delivery that the successful bidder provides a comprehensive listing of all components used to assemble the unit.

    - This includes any components installed by the manufacturer or any subcontractor or the successful bidder.

    - Information will include at a minimum, when available, make, model serial number on items such as engines, transmissions, axles, tires, bodies, etc.  The listings will be specific to each piece of equipment and will be provided on an individual CD for each unit delivered.

    - A minimum of one (1) CD per unit are to be provided and marked with the make, model, and last main numbers of the units serial number or State PO number.

- F.O.B. POINT:

  - The F.O.B. point is as listed in Section IV, Bid Schedule.  Ownership of and title will remain with the contractor until delivery is complete to final destination and accepted by the State.  Equipment is not to be driven on the Alcan Highway without prior written approval from the contracting officer.

  - The cost of shipping and delivery for orders beyond the limits of Seattle/Tacoma dock will be handled as follows.  The contractor will prepay the shipping and delivery charges to any destination named by the State in its order.  The contractor will charge-back those shipping and delivery charges to the State as a separate line item on the State’s invoice.

  - All shipping charges over $100 must be documented by a copy of the actual shipping invoice and received with the invoice charge to the State.

  - Shipping must be consolidated for the best possible price.  Shipping items separately must be pre-approved by the Contracting Officer PRIOR to shipment.  For example, GP Bucket or Spare Tire not being shipped with host unit must be pre-approved.

- DAMAGES FOR LATE DELIVERY AND NON-CONFORMING GOODS:

  - Time is of the essence in this contract.  The Bidder is expected to deliver goods that conform in all material respects to the contract specifications on or before the date provided therein, as may be amended by written agreement of the parties.

  - In the event that the equipment is delivered late or does not conform to the contract specifications, the State shall be entitled to offset against the Contract Price, as liquidated damages and not as a penalty, an amount equal to the cost of renting like equipment, multiplied by the number of calendar days elapsing between the delivery date provided in the bid schedule and the delivery date to the State.  In the case of equipment in this this class, that daily rental fee is determined to be $237.00.  The number of days for which liquidated damages shall apply shall include, in the case of non-conforming goods, the time reasonably necessary for the State to perform inspection.

  - These liquidated damages represent a reasonable estimate of amounts necessary to compensate the State for loss of use of the goods during the period in which the goods would have been available to the State if conforming goods had been timely delivered.

- EQUIPMENT RELIABILITY:

  - Reliability of equipment is of paramount importance to the State.  It is the policy of SEF to require minimum levels of reliability from owned or leased equipment for it to be considered acceptable.  Equipment offered for this bid must be capable of meeting the acceptable reliability standard stated below.

  - Acceptable Reliability:  The State will monitor equipment reliability.  Acceptable reliability for this contract is achieved when a machine achieves or maintains a Reliability Ratio (RR) equal to or exceeding the following:

    - .90 (90 percent) PR during any consecutive 12-months (365 days) during the warranty period.

    - .75 (75 percent) PR per operational month (recognizing operational as subject to weather and being defined by calendar days) during the consecutive 12-month period.

    - PR below the state percentages do not meet minimum reliability requirements for state owned equipment.

  - Machine Failure and Downtime:

    - Machine Failure is any and all loss of capability to perform fully, as specified, which is not attributed to Conditioned Failure.  Machine Failure resulting in the unit being out of service is defined as Downtime.

    - Conditioned Failure is any Machine Failure attributable to accident, operator abuse or other external cause not attributable to a defect in the machine itself.

    - Downtime is the actual number of days or fractions of days that the equipment is in a state of Machine Failure.  Downtime does not count time used for scheduled maintenance (including preventative maintenance and scheduled major overhauls), time lost for repair maintenance and scheduled major overhauls, time lost for repair of damage as a result of operator abuse or machine misapplication; or time lost as a result of accident or an act of God.  Downtime includes:

      - Actual shop hours (and/or field repair hours) required to return unit to full operational status following machine failure, including trouble-shooting, repair, necessary replacement of parts, and necessary adjustments, plus

      - Time lost waiting for parts and/or vendor assistance.  “Waiting downtime” also applies if need for parts/assistance is discovered during routine maintenance and return to service is deemed counterproductive.  In this case, “waiting time” clock begins with notice of need to vendor.  Allowance may be considered in “waiting time” calculations if arrival of parts/assistance is delayed by transportation shutdown, to include verifiable transportation scheduling difficulties such as infrequent flights as long as all reasonable alternatives have been exhausted.  Parts and assistance are to be provided by the quickest means reasonably possible to avoid unnecessary delays and downtime.

    - Out of Service Report (OSR):  Downtime resulting from machine failure is the actual number of hours a machine is out of service as recorded on the OSR or in the Equipment Maintenance Management System (EMS).

      - The State will record all downtime on an OSR or EMS work order, which will be originated for each occurrence of downtime.  The document will show the date and time a unit went down, the  location where the machine was based, the reason the machine is down, date and time the vendor was notified (if applicable), the date and time the machine was returned to service, and the total hours of downtime.

        - The Contract Manager will finalize and approve the OSR or EMS work order.  Both are available for contractor review.

    - Reporting Downtime:  The Contracting Officer will maintain documentation of all Downtime, and shall send copies of such documentation to the contractor.

    - Calculation of Reliability Ratio: RR is the mathematical ration of operated time (uptime) to out of service time (downtime).  The RR will be calculated according to the following formula:

RR = Days in a Month – Days Out of Service*   =   DM - DO

Days in a Month **                             DM

Note * : Fractional Days apply, i.e., a unit is out of service 8 hours in a 24 hour period  equals 1/3 or .33% of a day.

Note **: A day is allocated as 24 consecutive hours from 12:00 AM to 12:00 PM.

Example:  30 days DM with 2 days and 8 hours DT would result in:

RR =   30  -  2.33 = .92

30

    - Unacceptable Reliability: If an item of equipment fails to perform at an acceptable level of reliability during the warranty period, the Contracting Officer will notify the contractor and request immediate remedy.  Failure to remedy the piece of equipment within 30 days for failure will result in a breach of contract and the immediate return of the equipment and reimbursement of the Guaranteed Value (V) of the unit:

Original Cost of the unit less (-) Freight = $__________ (V)

Guaranteed Value (V) less (-) the Cost of Operation as listed in the Equipment Rental Rate Blue Book or comparable equipment or the current Federal Fixed Usage Rate for the Class for the State of Alaska (a, b or c per hour) times (X) the number of hours used = ___________(DV).

- Example:  Cost of a single unit, less freight = $150,000.  The hourly cost is $150.00 per hour.  The unit was used 150 hours prior to failing the acceptable reliability.  The contractor guarantees the unit’s worth at $127,500.00.

    - Prior to return, the State will correct all reasonable cosmetic deficiencies (such as excessive rust) and those deficiencies that are directly related to damage due to accidents, misuse of equipment or failure to operate or maintain equipment as prescribed by the vendor/manufacturer, prior to public auction.

    - The tires will be serviceable with at least 50% remaining thread.

    - Oil samples, as per manufacturer’s service manual recommendations, will be taken by State of Alaska maintenance personnel on the engine, transmission, differentials and hydraulics.

    - In the case of dispute, at the expense of the State, a qualified agent from Northern Adjusters, Inc. or another professionally recognized appraiser may be commissioned for an independent claim appraisal.  Such appraisal shall be binding upon the State and the contractor.

- WARRANTY:

  - Standard Warranty Package:  Unless otherwise stipulated by this ITB, the successful bidder will provide a one-year (12-month) warranty.

    - Full (100%) Parts and Labor Warranty Coverage of all components for 12 months (year one), from the date the unit is placed in service at the assigned location.

    - Full (100%) Warranty Coverage includes all cost of labor, parts, freight, lubricants, miscellaneous cost, etc., to place the unit in like-new condition.

    - Should the manufacturer’s standard warranty exceed the minimum State warranty requirements, the manufacturer’s warranty will run in conjunction with and enhance the State’s warranty, then continue for the remainder of its term.

    - For clarification, warranty does not apply to normal wear and tear or maintenance items, accident damages, misuse of equipment or failure to operate or maintain equipment as prescribed by vendor/manufacturer.

    - Warranty on Attachments:  Same as Standard Warranty Package.

    - In-Service Date:  Warranty on vehicles not placed in service immediately upon receipt because of time lag to construct body components and/or installation of special equipment, or due to seasonal usage or other delay, shall be warranted from the date the vehicle is placed in service.  The receiving agency shall notify the vendor/manufacturer in writing of the actual "in service" date.  Notification of the requirement for delayed warranty will be provided on delivery orders whenever possible.

  - Warranty Claims:

    - Warranty will be provided at the unit’s assigned (in-service) location.  Because of the remote location of some equipment it is not always practical to deliver equipment to authorized warranty repair facilities.  In these cases, the vendor may perform warranty work at the state's location or, the State of Alaska, at its discretion, reserves the right to perform the warranty work and be reimbursed by the vendor. If travel is required by State personnel to perform the work, actual costs will be used for reimbursement.

    - The State of Alaska has established a warranty procedure whereby the vendor is to be notified via letter, email, or fax, that warranty work needs to be performed.  If time is of the essence, a telephone call confirmed by one of the above written procedures may be utilized.

    - The vendor must notify the state within 24 hours of verbal or written notification that it will begin to perform the warranty work at the equipment location.

    - The State may, at its discretion, proceed to make warranty repairs with its own work force in the case of emergency situation or to preclude excessive downtime (greater than 24 hours).  The State will require a PO to perform the warranty work.

    - Failure to notify the State that the vendor intends to begin to perform warranty is considered a contractual breach.

    - The vendor will be invoiced for required warranty work performed by the state.  Warranty work performed by the state will be charged at the current SEF shop labor rate at the time of the repair.  Actual repair time will be used.

  - Warranty Performed by Vendor:

    - The State will reimburse travel costs not reimbursed by the manufacturer for travel to and from the bidder’s closest warranty service center within the State of Alaska to the location of the equipment under warranty.  Travel costs will be billed as follows

      - Mileage Charge: Mileage will only be reimbursed for travel within Alaska at the rate allowable by the IRS.

      - Meals are paid at actual and charges must be accompanied by receipts and are not to exceed the State authorized $60.00 per day.

      - Transportation, such as airfare, shall be reimbursed at actual and all charges are to be accompanied by a receipt/copy of the coach ticket.

      - Lodging shall be reimbursed at actual and shall not exceed $150.00 per night unless no other lodging is available.  Requests for reimbursement must be accompanied by a receipt.

    - Travel will only be reimbursed for time in Alaska.

    - After hours, weekend and holiday travel must be approved by the contracting officer to be considered for reimbursement.  The State will not pay for weather delays.

  - Authorized Warranty (Contractor/Bidder):

    - Contractor (bidder) must have Authorized Warranty Dealer that has all required licenses, facilities and factory certified and trained personnel necessary to perform the warranty servicing and repair work.

- Provide name and address for each Authorized Warranty Dealer for each location.

- (*)

- Provide contact name and contact information for Warranty Administrator:

- (*)

- Provide documentation of factory certified and trained personnel:

- (*)

    - The ultimate responsibility for warranty lies with the contractor (bidder).

    - The State reserves the right to inspect the warranty facility and diagnostic equipment prior to issuing the Notice of Intent to Award a contract.

  - Factory Recall:

    - Nationwide factory recall or product update programs are the responsibility of the vendor and/or manufacturer.  The State will attempt to bring affected equipment to an authorized repair facility.  However, because of the remoteness of some equipment this is not always practicable or economical.  In such cases, factory recall and modification work will be handled the same as warranty work.  Factory recall notices sent to the state should, in addition to serial number, include model, year, and dealer.

- REPAIR ORDERS AND DOCUMENTATION:

  - Any work performed by the contractor or approved subcontractor, whether warranty or any other work on a piece of equipment purchased under this ITB, will require a copy of the repair order, any invoices showing parts and commodities including oils and types used.

- PUBLICATIONS:

  - Paper publications are to be received by the State at the time of delivery.  Delivery will not be considered complete until the publications for each unit have been received by the State of Alaska.  Note: Publications, when required, will be ordered on the same Purchase Order as the unit itself.

    - All paper manuals are to be pre-assembled in factory binders prior to delivery.

    - Electronic publications may be requested.

  - Service Manuals:

    - Complete set(s) to include applicable information covering prime unit and attachments:

    - Body, chassis, and electrical

    - Engine, transmission, and differential(s) (service and rebuild)

    - Electrical and vacuum troubleshooting

    - Wiring diagrams

    - Service specifications

    - Engine/emission diagnosis

  - Parts Manuals:

    - Complete set(s) including all updates.  If updates are not provided during the warranty period, the State may order them from the manufacturer and bill the contractor for the full cost, including shipping.

    - Parts manuals are to be customized by serial number.

  - Operator’s Manuals:  Complete set(s) to include prime unit and attachments.

  - Quantities:  As per Section IV – Bid Price Schedule.

- Service Bulletins, Etc.:  The successful bidder must provide appropriate service bulletins, technical support bulletins, service letters, product support bulletins, and/or any other information type notifications that are sent out to the vendor or used by the manufacturer in the maintenance and report of the vehicle, equipment or attachments being provided.  The intent of this clause is that the State of Alaska be provided notification of any and all changes or improvement’s that may affect the maintenance, reliability, longevity, and safety of our equipment.

- STATEMENT OF ORIGIN:  The bidder will be required to furnish a Manufacturer's Statement of Origin for Automotive or Non-Automotive rolling stock for each unit.  All such documents shall be delivered with the invoice to:

- DOT&PF, HQ State Equipment Fleet

- 2200 E. 42nd Avenue Room #318

- Anchorage, Alaska  99508

- WEIGHT VERIFICATION SLIPS:  If required in the Bid Price Schedule, a weight scale ticket of the completed unit will be included with the Statement of Origin.

- PRICE:

  - Price Guarantee: The Contractor is responsible to maintain prices under the contract firm for 180 days after bid opening.  All price increases or decreases must remain firm for the following 180 days.

  - NO RETROACTIVE PRICE INCREASES WILL BE ACCEPTED.

  - Price adjustments, increases or decreases, for subsequent orders, may be made by providing the Contracting Officer satisfactory evidence that all of the following conditions exist:

    - The increase is a result of the increased cost at the manufacturer’s level and not costs under the contractor’s control, and that;

      - The increase will not produce a higher profit margin for the contractor than that on the original contract, and that;

      - The increase affects only the item(s) that are clearly identified by the contractor.

      - Satisfactory forms of the evidence of the above facts may include a certified invoice from the manufacturer, or an affidavit from an independent professional price-tracking firm that is recognized by the industry as reputable and knowledgeable.  The contractor must be able to show the difference between the prior year’s price and the current difference in the price being requested.

  - Price Decreases: During the period of the contract, the Contractor must pass on to the state all price decreases, such as fleet rebates.  A Contractor’s failure to adhere strictly and faithfully to this clause will be considered a material breach of contract.  The state reserves the right to cancel the contract if the contractor fails to properly perform the duties set out herein.

  - Manufacturer’s Rebate (Incentives):

    - In any circumstance during or prior to completion of the contract, whereupon the State of Alaska becomes eligible to receive a rebate for any vehicle purchased under this contract, it shall be the BIDDER'S responsibility to inform the Contracting officer in writing and to advise the procedures for obtaining such rebates.

- REPLACEMENT PARTS AND REPAIRS:

  - This contract encompasses a full parts and labor contract for manufacturer parts and repairs for the entire warranty period.

  - The State of Alaska shall expect the dealer or manufacturer to provide replacement wear parts at their authorized warranty facilities for the entire warranty period within seven (7) days of order.  All other parts must be available within ten (10) working days.

  - Back order procedures:  Back orders are acceptable; however, the ordering shop shall be appraised at time of original orders as to the expected delay in delivery.

  - Warranty:  All products supplied by the contractor shall be warranted against defects in materials and workmanship for a minimum of 90 days, commencing at the time of installation as long as the installation is within 12 months of purchase.  The cost of any defective product and the labor required to replace the defective product shall be the obligation of the contractor.

    - If the manufacturer’s warranty exceeds the stated warranty then manufacturer’s warranty supersedes.

    - Parts Return:  Within 12 months of the invoice date, the State is to be allowed to return new parts with full refund, less actual shipping charges. Cores returned within 12 months of original invoice date will receive full core credit. Returned parts will be in new, resellable condition. Refund will be in the form of a credit/invoice credited to the SOA account with the vendor.

    - Invoicing:  Full description of item is required on all invoices, packing lists and billings.

- ACCESSORIES:  When accessories are supplied, they must be certified to be compatible with the rest of the equipment.  Certification will be written evidence satisfactory to the State that the accessories are compatible.  The bidder’s failure to supply this evidence within the time required by the State will cause the State to consider the bid non-responsive and reject the bid.

- ADDITIONS OR DELETIONS:  The State reserves the right to add or delete items, agencies or locations as determined to be in the best interest of the State.  Added items, agencies or locations will be related to those on contract and will not represent a significant increase or decrease in size or scope of the contract.  Such additions or deletions will be documented via mutual agreement, will be at prices consistent with the original bid price margins, and will be evidenced by issuance of a written contract change notice from the Contracting officer.

- ALASKA BIDDER PREFERENCE: Award will be made to the lowest responsive and responsible bidder after an Alaska bidder preference of five percent (5%) has been applied. The preference will be given to a person who: (1) holds a current Alaska business license at the time designated in the invitation to bid for bid opening; (2) submits a bid for goods or services under the name on the Alaska business license; (3) has maintained a place of business within the state staffed by the bidder, or an employee of the bidder, for a period of six months immediately preceding the date of the bid; (4) is incorporated or qualified to do business under the laws of the state, is a sole proprietorship and the proprietor is a resident of the state, is a limited liability company organized under AS 10.50 and all members are residents of the state, or is a partnership under AS 32.06 or AS 32.11 and all partners are residents of the state; and, (5) if a joint venture, is composed entirely of ventures that qualify under (1) - (4) of this subsection. AS 36.30.170, AS 36.30.321(a) and AS 36.30.990[25]

- ALASKA BUSINESS LICENSE AND OTHER REQUIRED LICENSES:  At the time the bids are opened, all bidders must hold a valid Alaska business license and any necessary applicable professional licenses required by Alaska Statute.  Bids must be submitted under the name as appearing on the person’s current Alaska business license in order to be considered responsive.  Bidders should contact the Department of Commerce, Community and Economic Development, Division of Corporations, Business, and Professional Licensing, P. O. Box 110806, Juneau, Alaska 99811-0806, for information on these licenses. Bidders must submit evidence of a valid Alaska business license with the bid.

You are not required to hold a valid Alaska business license at the time bids are opened if you possess one of the

following licenses and are offering services or supplies under that specific line of business:

- fisheries business licenses issued by Alaska Department of Revenue or Alaska Department of Fish and Game,

- liquor licenses issued by Alaska Department of Revenue for alcohol sales only,

- insurance licenses issued by Alaska Department of Commerce, Community and Economic Development, Division of Insurance, or

- Mining licenses issued by Alaska Department of Revenue

- ALASKA PRODUCT PREFERENCE: A bidder that designates the use of an Alaska Product which meets the requirements of the ITB specification and is designated as a Class I, Class II or Class III Alaska Product by the Department of Community & Economic Development shall receive a preference in the bid evaluation in accordance with AS 36.30.332 and 3 AAC 92.010.

- ALASKA VETERAN PREFERENCE: If a bidder qualifies for the Alaska bidder preference under AS 36.30.321(a) and AS 36.30.990[25] and is a qualifying entity as defined in AS 36.30.321(f), they will be awarded an Alaska veteran preference of five percent (5%). The preference will be given to a (1) sole proprietorship owned by an Alaska veteran; (2) partnership under AS 32.06 or AS 32.11 if a majority of the partners are Alaska veterans; (3) limited liability company organized under AS 10.50 if a majority of the members are Alaska veterans; or (4) corporation that is wholly owned by individuals and a majority of the individuals are Alaska veterans, and may not exceed $5,000. The bidder must also add value by actually performing, controlling, managing, and supervising the services provided, or for supplies, the bidder must have sold supplies of the general nature solicited to other state agencies, other governments, or the general public - AS 36.30.321(i).

- ALASKANS WITH DISABILITIES PREFERENCE: If a bidder qualifies for the Alaska bidder preference under AS 36.30.321(a) and AS 36.30.990[25], and is a qualifying entity as defined in AS 36.30.321(d), they will be awarded an Alaskans with Disabilities Preference of ten percent (10%) in accordance with AS 36.30.321(d). A bidder may not receive both an Employment Program Preference and an Alaskans with Disabilities Preference.

- ALTERATIONS:  The contractor must obtain the written approval from the contracting officer prior to making any alterations to the specifications contained in this ITB.  The State will not pay for alterations that are not approved in advance and in writing by the contracting officer.

- AMENDMENTS:  Contract terms shall not be waived, altered, modified, supplemented or amended without prior written approval of the Contracting officer.

- ASSIGNMENT(S): Assignment of rights, duties, or payments under a contract resulting from this ITB is not permitted unless authorized in writing by the State of Alaska, Department of Administration, Division of General Services.  Bids that are conditioned upon the State’s approval of an assignment will be rejected as nonresponsive.

- AUTHORITY:  This solicitation is written in accordance with Alaska statutes AS 36.30 and 2 AAC 12.

- BILLING INSTRUCTIONS: Invoices must be billed to the ordering agency's address shown on the individual Purchase Order, Contract Award or Delivery Order, not to the Division of General Services.  The ordering agency will make payment after it receives the merchandise or service and the invoice.  Questions concerning payment must be addressed to the ordering agency.

- BID FORMS: Bidders shall use this bid and attached forms when submitting bids.

  - Bids, once submitted, may be modified in writing before the time and date set for bid opening.  Any modifications shall be prepared on company letterhead, signed by an authorized representative, and state that the new document supersedes or modifies the prior bid.  Modification must be submitted in a sealed envelope clearly marked "Bid Modification" and identifying the bid number and closing date and time.  Bidders may not modify bids after bid opening time.

  - Amendments may be faxed if they are not price related.

- BID FORMAT:

  - Bid submitted must contain, as a minimum:

    - Cover Page

    - Bid Price Schedule

    - Asterisked (*) item pages only (and/or) Letter of Clarification addressing asterisked (*) items

    - All pertinent amendments

  - Failure to return all of the pages listed above will cause your bid to be rejected.

  - Bids must conform to the requirements of the Invitation to Bid or Request for Proposal.  All necessary attachments (bid bonds, references, descriptive literature, etc.) must be submitted with the bid and in the required format.  Failure to comply with all requirements shall result in a non-responsive bid.

- BID MODIFICATION:

  - Modifications or erasures made before bid submission should be initialed in ink by the person signing the bid.  A bid may be rejected if it contains a material alteration or erasure, which is not initialed by the signer of the bid and the intent of the bidder, is not reasonably ascertainable from the bid as submitted.

  - Bids, once submitted, may be modified in writing before the time and date set for bid opening.  Any modifications shall be prepared on company letterhead, signed by an authorized representative, and state that the new document supersedes or modifies the prior bid.  Modification must be submitted in a sealed envelope clearly marked "Bid Modification" and identifying the bid number and closing date and time.  Bidders may not modify bids after bid opening time.

- BID PREPARATION COSTS:  The State is not responsible for any costs incurred by the bidder in preparation of his bid.

- BID WITHDRAWALS:  Bids may be withdrawn in writing on company letterhead signed by an authorized representative and received by the Procurement Office prior to bid closing time.  Bids may also be withdrawn in person before bid opening time upon presentation of appropriate identification.

- BRAND AND MODEL OFFERED:  Unless otherwise specified, when brand names and model numbers are used to specify the type and quality of the goods desired, bidders must clearly indicate the brand names and model numbers they intend to provide.  The bidder’s failure to identify the brand and model offered will cause the State to consider the offer non-responsive and reject the bid.

- CERTIFICATION OF COMPLIANCE WITH AMERICAN'S WITH DISABILITIES ACT OF 1990:

  - By signature of their bid/proposal the bidder/proposer certifies that they comply with the American's with Disabilities Act of 1990 and the regulations issued there under by the federal government.

  - Services or activities furnished to the general public on behalf of the State must be fully accessible.  This is intended to ensure that agencies are in accordance with 28 CFR Part 35 Section 35.130 and that services, programs or activities furnished to the public through a contract do not subject qualified individuals with a disability to discrimination based on the disability.

- CLARIFICATION OF OFFER:

  - A letter of clarification is required to explain any deviations from terms, conditions, or specifications of this ITB.  Offers exceeding ITB specifications are regarded as deviations and should be clarified.

  - In order to prevent technical errors, following each asterisked is space that shall be used to address all of the asterisked items.  It is mandatory that a letter of clarification or the space behind the asterisked items be used to supply the required information.

  - When using a letter of clarification, or the space provided within the ITB, to address an asterisked (*) item, the statement must indicate specifically what is being provided.  Do not include statements such as "Complies to Spec", or "Meets or Exceeds Specifications".  This will be considered a non-responsive response.

  - Letters of Clarification are to be on company (Bidder’s) letterhead and be signed.  The only information in the Letters of Clarification should be the explanation(s) of deviation(s) and/or responses to asterisked (*) items.  Asterisked (*) item explanations should refer to that particular specification item number.

  - Letters of disclosure and clarification must be made part of the sealed bid.

- COMPLIANCE WITH ALL GOVERNMENT REGULATIONS:  The bidder must comply with all applicable federal, state, and borough regulations, codes, and laws, and pay all applicable federal, state, and borough taxes, and is liable for all required insurance, licenses, permits, and bonds.  Failure to comply with such requirements shall constitute a breach of contract and shall be grounds for contract cancellation.  Damages or costs resulting from noncompliance shall be the sole responsibility of the bidder.

- CONFLICT OF INTEREST: An officer or employee of the State of Alaska may not seek to acquire, be a party to, or possess a financial interest in, this contract if  (1) the officer or employee is an employee of the administrative unit that supervises the award of this contract; or  (2) the officer or employee has the power to take or withhold official action so as to affect the award or execution of the contract.

- CONSOLIDATION OF AWARDS: Due to high administrative costs associated with processing of purchase orders, a single low bid of $50 or less may, at the discretion of the State, be awarded to the next low bidder receiving other awards for consolidation purposes.  This paragraph is not subject to the protest terms enumerated in “INSTRUCTION TO BIDDERS”, “FILING A PROTEST” above.

- CONSUMER ELECTRICAL PRODUCT: AS 45.45.910 requires that "...a person may not sell, offer to sell, or otherwise transfer in the course of the person's business a consumer electrical product that is manufactured after August 14, 1990, unless the product is clearly marked as being listed by an approved third party certification program."  Electrical consumer products manufactured before August 14, 1990, must either be clearly marked as being third party certified or be marked with a warning label that complies with AS 45.45.910(e).  Even exempted electrical products must be marked with the warning label.  By signature on this bid the bidder certifies that the product offered is in compliance with the law.  A list of approved third party certifiers, warning labels and additional information is available from: Department of Labor and Workforce Development, Labor Standards & Safety Division, Mechanical Inspection Section, P.O. Box 107020, Anchorage, Alaska  99510-7020, (907)269-4925.

- CONTINUING OBLIGATION OF CONTRACTOR: Notwithstanding the expiration date of a contract resulting from this ITB, the contractor is obligated to fulfill its responsibilities until warranty, guarantee, maintenance and parts availability requirements have completely expired.

- CONTRACT ADMINISTRATION:  The administration of this contract, including any/all changes, is the responsibility of the Contracting Officer, HQ State Equipment Fleet.

- CONTRACT EXTENSION: Unless otherwise provided in this ITB, the State and the successful bidder/contractor agree:  (1) that any holding over of the contract excluding any exercised renewal options, will be considered as a month-to-month extension, and all other terms and conditions shall remain in full force and effect and (2) to provide written notice to the other party of the intent to cancel such month-to-month extension at least thirty (30) days before the desired date of cancellation.

- CONTRACT FUNDING: Bidders are advised that funds are available for the initial purchase and/or the first term of the contract.  Payment and performance obligations for succeeding purchases and/or additional terms of the contract are subject to the availability and appropriation of funds.

- DEFAULT:  In case of bidder default, the State may procure the goods or services from another source and hold the bidder responsible for any resulting excess costs and may seek other remedies under law or equity.  Alaska Statutes and Regulations provide for suspension and disbarment of non-responsible bidders.

- DELIVERY:  All deliveries shall be F.O.B. final destination point with all transportation and handling charges paid by bidder.  Responsibility and liability for loss or damage shall remain with bidder until final inspection and acceptance when responsibility shall pass to the State except as to latent defects, fraud and bidder's warranty obligations.

- DISCONTINUED ITEMS:  In the event an item is discontinued by the manufacturer during the life of the contract, another item may be substituted, provided that the contracting officer makes a written determination that it is equal or better than the discontinued item and provided that it is sold at the same price or less than the discontinued item.

- DISPUTES: Any dispute arising out of this agreement shall be resolved under the laws of Alaska.  Any appeal of an administrative order or any original action to enforce any provision of this agreement or to obtain any relief from or remedy in connection with this agreement may be brought only in the superior court for the State of Alaska.

- EMPLOYMENT PROGRAM PREFERENCE: If a bidder qualifies for the Alaska bidder preference under AS 36.30.321(a) and AS 36.30.990[25], and is offering goods or services through an employment program as defined under 36.30.990(11), they will be awarded an Employment Program Preference of fifteen percent (15%) in accordance with AS 36.30.321(b).

- EMPLOYERS OF PEOPLE WITH DISABILITIES PREFERENCE: If a bidder qualifies for the Alaskan bidder's preference, under AS 36.30.170(b), and, at the time the bid is submitted, employs a staff that is made up of fifty percent (50%) or more people with disabilities, as defined in AS 36.30.170(j), and submits a responsive and responsible bid that is no more than ten percent (10%) higher than the lowest responsive and responsible bid, the procurement officer will make the award to that bidder, in accordance with AS 36.30.170(f).

- EXTENSION OF PRICES: In case of error in the extension of prices in the bid, the unit prices will govern;

in a lot bid, the lot prices will govern.

- FILING A PROTEST: A bidder may protest the award of a contract or the proposed award of a contract for supplies, services, or professional services.  The protest must be filed in writing and include the following information: (1) the name, address, and telephone number of the protester; (2) the signature of the protester or the protester's representative; (3) identification of the contracting agency and the solicitation or contract at issue; (4) a detailed statement of the legal and factual grounds of the protest, including copies of relevant documents; and (5) the form of relief requested.  Protests will be treated in accordance with Alaska Statutes (AS)36.30.560-36.30.610.

- FIRM OFFER: For the purpose of award, offers made in accordance with this ITB must be good and firm for a period of ninety (90) days from the date of bid opening.

- FIRM, UNQUALIFIED AND UNCONDITIONAL OFFER:  Bidders must provide enough information with their bid to constitute a definite, firm, unqualified and unconditional offer.  To be responsive a bid must constitute a definite, firm, unqualified and unconditional offer to meet all of the material terms of the ITB.  Material terms are those, which could affect price, quantity, quality, or delivery.  Also included, as material terms are those, which are clearly identified in the ITB and which, for reasons of policy, must be complied with at risk of bid rejection for non-responsiveness.

- FORCE MAJEURE (Impossibility to perform):  Neither party to this contract shall be held responsible for delay or default caused by acts of God and/or war, which is beyond that party's reasonable control.  The State may terminate this contract upon written notice after determining such delay or default will reasonably prevent successful performance of the contract.

- FORMAL WORKSHOP:

- The Formal Workshop (Pre-Bid) is to discuss with prospective bidders the "draft" Invitation to Bid (ITB).  The formal workshop is the only opportunity for prospective bidders, manufacturers and other interested parties, in a conference setting, to discuss Standard Terms and Conditions, Special Terms and Conditions, Specifications, and to provide clarification as needed for this ITB.

- Formal workshops are for information only.  Any statements made at formal workshops will not be official until verified in the "final" ITB.  Participation by teleconference may be possible by making arrangements with the SEF Contracting Office in advance.

- HUMAN TRAFFICKING:

  - By signature on this contract, the offeror certifies that:

    - the offeror is not established and headquartered or incorporated and headquartered, in a country recognized as Tier 3 in the most recent United States Department of State’s Trafficking in Persons Report; or

  - The most recent United States Department of State’s Trafficking in Persons Report can be found at the following website:  www.state.gov/g/tip/

  - Failure to comply with this requirement will cause the state to reject the bid or proposal as non-responsive, or cancel the contract.

  - This pertains to goods and services above $50,000.00.

- INDEMNIFICATION:  The Contractor shall indemnify, hold harmless, and defend the contracting agency from and against any claim of, or liability for error, omission or negligent act of the Contractor under this agreement.  The Contractor shall not be required to indemnify the contracting agency for a claim of, or liability for, the independent negligence of the contracting agency. If there is a claim of, or liability for, the joint negligent error or omission of the Contractor and the independent negligence of the Contracting agency, the indemnification and hold harmless obligation shall be apportioned on a comparative fault basis. “Contractor” and “Contracting agency”, as used within this and the following article, include the employees, agents and other contractors who are directly responsible, respectively, to each.  The term “independent negligence” is negligence other than in the Contracting agency’s selection, administration, monitoring, or controlling of the Contractor and in approving or accepting the Contractor’s work.

- INSPECTIONS:  Goods furnished under this contract are subject to inspection and test by the State at times and places determined by the State.  If the State finds goods furnished to be incomplete or not in compliance with bid specifications, the State may reject the goods and require bidder to either correct them without charge or deliver them at a reduced price, which is equitable under the circumstances.  If bidder is unable or refuses to correct such goods within a time deemed reasonable by the State, the State may cancel the order in whole or in part.  Nothing in this paragraph shall adversely affect the State's rights as buyer, including all remedies and rights granted by Alaska statutes.

- INSURANCE:

  - Without limiting Contractor's indemnification, it is agreed that Contractor shall purchase at its own expense and maintain in force at all times during the performance of services under this agreement the following policies of insurance.  Where specific limits are shown, it is understood that they shall be the minimum acceptable limits. If the Contractor's policy contains higher limits, the state shall be entitled to coverage to the extent of such higher limits.  Certificates of Insurance must be furnished to the contracting officer prior to beginning work and must provide for a notice of cancellation, non-renewal, or material change of conditions in accordance with policy provisions.  Failure to furnish satisfactory evidence of insurance or lapse of the policy is a material breach of this contract and shall be grounds for termination of the Contractor's services. All insurance policies shall comply with, and be issued by insurers licensed to transact the business of insurance under AS 21.

  - Proof of insurance is required for the following:

    - Workers' Compensation Insurance:  The Contractor shall provide and maintain, for all employees engaged in work under this contract, coverage as required by AS 23.30.045, and; where applicable, any other statutory obligations including but not limited to Federal U.S.L. & H. and Jones Act requirements.  The policy must waive subrogation against the State.

    - Commercial General Liability Insurance:  covering all business premises and operations used by the Contractor in the performance of services under this agreement with minimum coverage limits of $300,000 combined single limit per occurrence.

    - Commercial Automobile Liability Insurance:  covering all vehicles used by the Contractor in the performance of services under this agreement with minimum coverage limits of $300,000 combined single limit per occurrence.

  - Failure to supply satisfactory proof of insurance within the time required will cause the State to declare the bidder nonresponsive and to reject the bid.

- INVITATION TO BID (ITB) REVIEW: Bidders shall carefully review this ITB for defects and questionable or objectionable material.  Bidders' comments concerning defects and questionable or objectionable material in the ITB must be made in writing and received by the purchasing authority at least ten (10) days before the bid opening date.  This will allow time for an amendment to be issued if one is required.  It will also help prevent the opening of a defective bid, upon which award cannot be made, and the resultant exposure of bidders' prices.  Bidders' original comments should be sent to the purchasing authority listed on the front of this ITB.

- ITEM UPGRADES:  The State reserves the right to accept upgrades to models on the basic contract when the upgrades improve the way the equipment operates or improve the accuracy of the equipment.  Such upgraded items must be at the same price as the items in the basic contract.

- LOCAL AGRICULTURAL AND FISHERIES PRODUCTS PREFERENCE: When agricultural, dairy, timber, lumber, or fisheries products are purchased using state money, a seven percent (7%) preference shall be applied to the price of the products harvested in Alaska, or in the case of fisheries products, the products harvested or processed within the jurisdiction of Alaska, in accordance with AS 36.15.050.

- METHOD OF AWARD:

  - DOT/PF is required by law to award “to the lowest responsible and responsive bidder whose bid conforms in all MATERIAL respects to the requirements and criteria set out in the invitation to bid.” AS 36.30.170.

  - All specifications in this ITB are important and bidders should carefully address each item.  Deviation from a specification MAY result in rejection of a bid at the discretion of the Contracting Officer if the deviation is material.  A material deviation is one that is significantly different from an essential aspect of a specification.

  - The Contracting Officer may be justified in accepting a non-material deviation - it will be determined upon the facts, circumstances, and the proposal in the bid.  If a bid deviation has a negligible effect on price, quality, performance, delivery or contract conditions, it is not material and may be waived or corrected by the Contracting Officer.  The State reserves the right to waive minor informalities or technical defects, which are not material, when such waiver is in the best interest of the State.

  - Nothing in this section shall be construed as depriving the State of its discretion in the matter of determining the lowest responsible bidder.

  - A Notice of Intent to Award (ITA) will be issued for all solicitations in accordance with regulations.  The notice of intent to award does not constitute a formal award of a contract.  Formal award is achieved through execution of the "Offer and Contract Award" section of the Invitation to Bid.

- NEW EQUIPMENT: Equipment offered in response to this ITB must be new equipment.  New equipment means equipment that is currently in production by the manufacturer and is still the latest model, edition or version generally offered.  The equipment must be warranted as new by the manufacturer and may not have been used for any purpose, other than display (not demonstration), prior to its sale to the State.  The State will not accept remanufactured, used or reconditioned equipment, including used or reconditioned components or parts of.  It is the contractor’s responsibility to ensure that each piece of equipment delivered to the State complies with this requirement.  A contract’s failure to comply with this requirement will cause the State to seek remedies under breach of contract.

- NON-RESPONSIVE BIDS:

  - Bids will be considered non-responsive and shall be rejected for the following:

    - Failure to sign the bid.

    - Failure to submit the bid at the place and time required in the solicitation.

    - Failure to have a valid Alaska Business License at the time of bid opening.

    - Failure to acknowledge amendments requiring signature.

    - Failure to provide product literature, brochures, or a letter of clarification, when specifically required by the Invitation to Bid.

    - Bids which specify "Offered Delivery Time" in excess of "Required Delivery" number of calendar days after the receipt of an order.

- ORDER DOCUMENTS: Except as specifically allowed under this ITB, an ordering agency will not sign any vendor contract.  The State is not bound by a vendor contract signed by a person who is not specifically authorized to sign for the State under this ITB.  The State of Alaska Purchase Order, Contract Award and Delivery Order are the only order documents that may be used to place orders against the contract(s) resulting from this ITB.

- PAYMENT:  Payment for agreements under $500,000 for the undisputed purchase of goods or services provided to a State agency will be made within 30 days of the receipt of a proper billing or the delivery of the goods or services to the location(s) specified in the agreement, whichever is later.  A late payment is subject to 1.5% interest per month on the unpaid balance.  Interest will not be paid if there is a dispute or if there is an agreement, which establishes a lower interest rate or precludes the charging of interest.

- PREFERENCE QUALIFICATION LETTER: Regarding preferences 6 and 7 above, the Division of Vocational Rehabilitation in the Department of Labor and Workforce Development maintains lists of Alaskan; [1] employment programs that qualify for preference, and [2] individuals who qualify for preference as Alaskan's with disabilities. In accordance with AS 36.30.321(i), in order to qualify for one of these preferences, a bidder must add value by actually performing, controlling, managing, and supervising the services provided, or for supplies, a bidder must have sold supplies of the general nature solicited to other state agencies, governments, or the general public.

As evidence of an individual's or a business' right to a certain preference, the Division of Vocational Rehabilitation will issue a certification letter. To take advantage of the preferences 6 or 7 above, an individual or business must be on the appropriate Division of Vocational Rehabilitation list at the time the bid is opened, and must attach a copy of their certification letter to their bid. The bidder's failure to provide this certification letter with their bid will cause the State to disallow the preference.

- PRICE EXTENSION:  In case of error in the extension of prices in the bid, the unit prices will govern.

- PRICES:  The bidder shall state prices according to the requirements of this ITB.  Prices quoted for commodities or services must be in U.S. funds and include applicable federal duty, brokerage fees, packaging, and transportation cost to the FOB point so that upon transfer of title the commodity or service can be utilized without further cost.

- PRODUCT EQUIVALENT SUBSTITUTION:  Bidders may submit bids for substantially equivalent products unless the Invitation to Bid provides that a specific brand is necessary because of compatibility requirements. In those instances where the ITB specifies "a name brand or State approved equivalent", bidders must request approval of a product equivalent substitution NO LATER THAN 10 days prior to the bid opening date.  A written response will be provided by the Contracting Officer.

- PRODUCT IDENTIFICATION:  Bidders must clearly identify all products bid.  Brand name and model or number must be shown for the prime unit as well as attachments where applicable.  The State reserves the right to reject any bid when the product information submitted with the bid is incomplete and/or is not easily ascertainable from the bid documents or through independent verification.

- PRODUCT LITERATURE:

  - ANNOTATED LITERATURE:  Bidders must annotate their product literature to identify for the State the location of the supporting information regarding each product specification set out in this ITB.  A bidder's failure to comply with this clause, within the time set by the State, will cause the State to consider the offer non-responsive and reject the bid.

  - Product brochures, descriptive literature and other supporting documentation may be required in some solicitations.  Descriptive literature is for the purpose of providing a reference to the product offered.  While it is not practicable to require documentation to support every single item of a specification, it is sometimes necessary to require the bidder to provide product literature that will help the Contracting officer to determine the bidder's compliance with important specifications.

  - Requirements for product literature will vary from bid to bid. Certain specification items may be marked with an asterisk (*) indicating that descriptive literature or supporting documentation (such as a letter of clarification) will be required.

  - When product literature is required by the ITB, please provide a minimum of one (1) complete set to aid in bid review and evaluation.

  - Failure to provide product literature or brochures when required WILL result in your bid being deemed non-responsive.

  - A bidder's signature on the contract offer and award is taken as a promise to comply with the ITB specifications.  In case of a difference between the literature and the specifications, the specifications will govern.

  - The State does not honor disclaimer statements often evident in manufacturer's published specification sheets or brochures.  Such disclaimers will not release the successful bidder from providing the specified item, at the time of acceptance by the State.

- PROPRIETARY INFORMATION AND STATEMENTS OF CONFIDENTIALITY:

  - Except as set forth in this provision, all information in all bids will be made public under AS 36.30.530 not later than the time of issuance of a notice of intent to award.

  - If the offeror submits information considered by it to constitute a trade secret or proprietary data, such information may be expressly designated as such, and must be accompanied by the offeror's certification that (1) the information has consistently been maintained by its owner as a trade secret or as proprietary information, (2) the owner of the information has taken due care to protect it from release to non-privileged persons, and  (3) to the best knowledge of the offeror, the information has not lost its status as trade secret or proprietary information, whether by lack of diligent protection, release to any non-privileged person or otherwise.

  - Absence of such certification, any claim of confidentiality will be ignored, and the bidder may not hold any reasonable expectation of confidentiality.

  - Any information so certified will be held confidential so long as the contracting officer concurs that it constitutes a trade secret or proprietary data, and deems it not critical to determination of the price, quantity, or delivery terms bid, or the responsiveness of the bid.

  - By submission of a bid, the offeror consents to the contracting officer’s exercise of reasonable judgment as to concurrence with any assertion of confidentiality, and waives any and all claims for release of information that the contracting officer reasonably deems not confidential notwithstanding a certified assertion of confidentiality.

  - A certified assertion of confidentiality in which the contracting officer concurs, with respect to information the contracting officer deems critical to determination of the price, quantity, or delivery terms bid, or the responsiveness of the bid, will cause the bid to be rejected as a non-responsive bid.

- PROTESTS:  A bidder may protest the Intent to Award a contract pursuant to this Invitation to Bid in accordance with provisions in Alaska Statute AS 36.30.560.  The protest must be filed in writing within ten (10) days of the notice of Intent to Award, and must include the following information:  (1) the name, address, and telephone number of the protester; (2) the signature of the protester or the protester's representative; (3) identification of the contracting agency and the solicitation or contract at issue; (4) a detailed statement of the legal and factual grounds of the protest, including copies of relevant documents; and (5) the form of relief requested.  Protests must be filed with:  Contracting officer, State Equipment Fleet Purchasing, 2200 East 42nd Ave, Room #311, Anchorage, AK  99508.  Protests and any subsequent appeal will be processed per AS 36.30.

- QUANTITIES:  The State reserves the right to reduce or increase the quantity of items ordered under any contract resulting from this Invitation to Bid.

- REJECTION OF BIDS:  All bids or proposals may be rejected prior to award for the following reasons:

  - the supplies or services being procured are no longer required;

  - ambiguous or otherwise inadequate specifications were part of the solicitation;

  - the solicitation did not provide for consideration of all factors of significance to the State;

  - prices exceed available money and it would not be appropriate to adjust quantities to accommodate available money;

  - all otherwise acceptable bids or proposals received are at unacceptable prices;

  - there is reason to believe that the bids or proposals may not have been independently arrived at in open competition, may have been collusive, or may have been submitted in bad faith; or

  - the award is not in the best interests of the State.

- SEVERABILITY: If any provision of the contract is declared by a court to be illegal or in conflict with any law, the validity of the remaining terms and provisions will not be affected; and, the rights and obligations of the parties will be construed and enforced as if the contract did not contain the particular provision held to be invalid.

- SHIPPING DAMAGE:  The State will not accept or pay for damaged goods.  The contractor must file all claims against the carrier(s) for damages incurred to items in transit from the point of origin to the ultimate destination.  The State will provide the contractor with written notice when damaged goods are received.

- SIGNATURE ON BID:  BIDS MUST BE SIGNED IN INK BY AN OFFICIAL OF THE BIDDING ORGANIZATION authorized to bind the bidder to its provisions.  Signature certifies, under penalty of perjury under the laws of the United States, that the price(s) submitted was/were independently arrived at without collusion, and that neither bidder nor bidder’s firm, association or corporation of which bidder is a member, has either directly or indirectly, entered into any agreement, participated in any collusion, or otherwise taken any action in restraint of free competitive bidding in connection with this bid per 2AAC12.800.

- STANDARD AND SPECIAL TERMS AND CONDITIONS:  The terms and conditions of this section are standard to State of Alaska, Department of Transportation and Public Facilities, Statewide Equipment Fleet contracts for the purchase of goods.  There may also be other special terms and conditions in an Invitation to Bid or Request for Proposal which apply only to this contract.  In the event of a conflict between the standard and special terms and conditions, the Special Terms and Conditions take precedence.

- SUBCONTRACTOR(S): Within five (5) working days of notice, the apparent low bidder must submit a list of the subcontractors that will be used in the performance of the contract.  The list must include the name of each subcontractor and the location of the place of business for each subcontractor and evidence of each subcontractor's valid Alaska business license.  Subcontractors can only be changed per AS 36.30.115 (b).

- SUBMITTING BIDS: Envelopes containing bids must be sealed, marked, and addressed as shown in the

example below.  Do not put the ITB number and opening date on the envelope of a request for bid information.

Envelopes with ITB numbers annotated on the outside will not be opened until the scheduled bid opening.

Bidder’s Return Address	STAMP

Department of Transportation & Public Facilities

HQ State Equipment Fleet

2200 E. 42nd Avenue, Room #311

### Anchorage, AK  99508

### Bid No.: SEF- ______

Opening Date:_____________

- FORMAL WORKSHOP: (Attendance is Mandatory IF a workshop is scheduled)

  - The Formal Workshop (Pre-Bid) is to discuss with prospective bidders the "draft" Invitation to Bid (ITB).  The formal workshop is the only opportunity for prospective bidders, manufacturers and other interested parties, in a conference setting, to discuss Standard Terms and Conditions, Special Terms and Conditions, Specifications, and to provide clarification as needed for this ITB.  All bidders (a representative from the bidding company) must attend in person or by teleconference in order to bid.

  - Formal workshops are for information only.  Any statements made at formal workshops will not be official until verified in the "final" ITB.  Participation by teleconference may be possible by making arrangements with the SEF Contracting Office in advance.

- TIMELINESS OF BID:  Sealed bids must be received and time-stamped by the Procurement Office prior to the time set for receipt of bids.  Bids received after bid-closing time will not be considered.  Bidders must ensure that their bid and any amendments requiring signature are delivered prior to the scheduled bid opening time.  LATE BIDS WILL NOT BE ACCEPTED.

- FORMAL WORKSHOP: (Attendance is Mandatory IF a workshop is scheduled)

  - The Formal Workshop (Pre-Bid) is to discuss with prospective bidders the "draft" Invitation to Bid (ITB).  The formal workshop is the only opportunity for prospective bidders, manufacturers and other interested parties, in a conference setting, to discuss Standard Terms and Conditions, Special Terms and Conditions, Specifications, and to provide clarification as needed for this ITB.  All bidders (a representative from the bidding company) must attend in person or by teleconference in order to bid.

  - Formal workshops are for information only.  Any statements made at formal workshops will not be official until verified in the "final" ITB.  Participation by teleconference may be possible by making arrangements with the SEF Contracting Office in advance.

- TIMELINESS OF BID:  Sealed bids must be received and time-stamped by the Procurement Office prior to the time set for receipt of bids.  Bids received after bid-closing time will not be considered.  Bidders must ensure that their bid and any amendments requiring signature are delivered prior to the scheduled bid opening time.  LATE BIDS WILL NOT BE ACCEPTED.

ELECTRONIC BID SUBMISSION: Bids may be emailed to anne.moeller@alaska.gov, no later than the date and time listed on page one of this ITB as the deadline for receipt of bids, and must contain the ITB number in the subject line of the email. Emailed bids must be submitted as an attachment in PDF format. Please note that the maximum size of a single email (including all text and attachments) that can be received by the state is 20mb (megabytes). If the email containing the bid exceeds this size, the bid must be sent in multiple emails that are each less than 20 megabytes and each email must comply with the requirements described above. The state is not responsible for unreadable, corrupt, or missing attachments. It is the bidder’s responsibility to contact the issuing office at (907) ______ to confirm that the bid has been received. Failure to follow the above instructions may result in the bid being found non-responsive and rejected.

FAX BID SUBMISSION: Bids may be faxed to (907) 269-0801, no later than the date and time listed on page one of this ITB as the deadline for receipt of bids. It is the bidder’s responsibility to contact the issuing office at (907) 269-0791 to make arrangements prior to faxing the bid and to confirm that the bid has been received. Failure to follow the above instructions may result in the bid being found non-responsive and rejected.

- SUCCESSORS IN INTEREST:  This contract shall be binding upon successors and assigns.

- SUITABLE MATERIALS:  All materials, supplies or equipment offered by a bidder shall be new, unused, of recent manufacture, and suitable for the manufacturer's intended purpose unless the specifications allow for used, rebuilt or remanufactured equipment.

- SUPPORTING INFORMATION:

  - The State strongly desires that bidders submit all required technical, specification, and other supporting information with their bid, so that a detailed analysis and determination can be made, by the contracting officer that the product offered meets the ITB specifications and that other requirements of the ITB have been met.  However, provided a bid meets the requirements for a definite, firm, unqualified, and unconditional offer, the State reserves the right to request supplemental information from the bidder, after the bids have been opened, to ensure that the products offered completely meet the ITB requirements.  The requirement for such supplemental information will be at the reasonable discretion of the State and may include the requirement that a bidder will provide a sample product(s) so that the State can make a first-hand examination and determination.

  - A bidder's failure to provide this supplemental information or the product sample(s), within the time set by the State, will cause the State to consider the offer non-responsive and reject the bid.

- TAXES:  Prices quoted in bids must be exclusive of federal, state, and local taxes.  If the bidder believes that certain taxes are payable by the State, the bidder may list such taxes separately, directly below the bid price for the affected item.  The State is exempt from Federal Excise Tax because articles purchased are for the exclusive use of the State of Alaska.

- TIMELINESS OF BID:  Sealed bids must be received and time-stamped by the Procurement Office prior to the time set for receipt of bids.  Bids received after bid-closing time will not be considered.  Bidders must ensure that their bid and any amendments requiring signature are delivered prior to the scheduled bid opening time.  LATE BIDS WILL NOT BE ACCEPTED.

- USE OF LOCAL FOREST PRODUCTS: In a project financed by state money in which the use of timber, lumber and manufactured lumber is required, only timber, lumber and manufactured lumber products originating in this state from Alaska forests shall be used unless the use of those products has been determined to be impractical, in accordance with AS 36.15.010 and AS 36.30.322..

- WARRANTY:   Unless otherwise stated, all equipment shall be new and current model and shall carry full factory warranties.  Bidder warrants all goods delivered to be free from defects in labor, material and manufacture and to be in compliance with bid specifications.  All implied or expressed warranty provisions of the Uniform Commercial Code apply.  All warranties shall be for and benefit the State.

# GENERAL SPECIFICATION:

It is the purpose of this specification to describe a new, and of the latest manufacturer's current production model and design Standard Body Paratransit vehicle with wheelchair lift.

Seating (including optional) to allow for at least three (3) different seating arrangements:

[1] Include driver and passenger seating for 12 ambulatory persons and two (2) wheelchair positions.

[2] Priced as an option.  Include driver and passenger seating for minimum six (6) ambulatory persons and three (3) wheelchair positions.

[3] Priced as an option.  Include driver and passenger seating for minimum six (6) ambulatory persons and four (4) wheelchair positions.

Refer to seating specifications.  To be equipped with one (1) passenger door, one (1) driver’s door, and one (1) wheelchair access door.  Also, an optional rear emergency door is to be priced.

Note:  Other options, such as remote engine start, tire chains, 4x4, and diesel engine are to be priced separately as well.

The vehicle is to provide maximum passenger appeal in appearance, comfort and safety combined with excellence in reliability, operating characteristics and economy of operation.

The vehicle shall be designed so that continuous operation can be performed without any additional alterations being necessary for extreme weather conditions as experienced in Anchorage, Alaska.

The vehicle is for service in routes, which include both highway uses with relatively long distance between stops and in town use with relatively frequent stops.

The vehicle should be capable of legal highway speeds, yet be able to negotiate grades of up to 15% (percent) with a standing load.

The vehicle shall afford features essential for safe, fast, efficient and comfortable operation by the conditions and adequate means for safe passenger movement.

The vehicle must be maneuvered easily in normal and heavy traffic.

The design of the body, chassis and equipment, which the manufacturer proposes to furnish, shall be such as to produce a vehicle of substantial and durable construction in all respects.

No advantage shall be taken by the manufacturer in the omission of any part or detail, which goes to make the bus complete and ready for service, even though such parts or detail are not mentioned in these specifications.

Unit to include all standard equipment and accessories as advertised in manufacturer's specification sheet of model offered, unless otherwise specified herein.

The vehicle must meet all Federal and Alaska State requirements for this type of vehicle in effect at the time of delivery.  The vehicle must meet the specifications in the Department of Transportation 49 CFR, Part 38 – Americans with Disabilities Act (ADA), Accessible Specifications for Transportation Vehicles – Subparts A and B.  The vehicle offered is not to require the driver to obtain a Commercial Driver’s License (CDL).

# APPLICATION:

General-purpose use as ambulatory and wheelchair passenger conveyance.  Varying terrain and weather conditions to minus 50 degrees Fahrenheit.

# DOCUMENTATION REQUIRED WITH THE BID PACKAGE:

One (1) copy of manufacturer’s product brochure to be provided with bid package.

In addition, specifications marked with an asterisk (*) require supporting documentation, which indicates specifically what the bidder intends to supply in regard to said items and/or how specifications will be met.  In order to help prevent technical errors, following each asterisked is space that may be used to address all of the asterisked items.  It is required that a letter of clarification or the space behind the asterisked items be used to supply the required information.  You may use the area behind the asterisked item to refer to a product brochure, manufacturer’s technical data sheet, or letter of clarification, which indicates specifically what you, the bidder, intend to supply in regard to said items and/or how specifications are met.

# TYPICAL UNITS:

# FORD E-450 Super Duty Cutaway.  Provided all of the following specifications are met.

- POWER TRAIN:

  - Engine:

    - (*)  Gasoline, V10, 6.8 liter, 305 HP, 420-pound foot torque, minimum.

    - Anti-freeze:  Permanent type, affording protection to minus 50 degrees Fahrenheit.

  - Exhaust System:  Highest quality OEM. Shall terminate near the rear of the vehicle on the driver’s side.

  - Starting Aids:  Engine Block Heater

- DRIVE TRAIN:

  - 4 x 2 configurations with dual rear wheels.

  - Two (2) wheel Drive:

  - Four wheel drive (OEM approved) OPTIONAL ITEM:

    - In lieu of 4x2 configuration.

    - To include dual rear wheels.

    - Front and rear tires must be the same size.  Different size tires on front and rear will not be accepted.

  - Transmission:  Automatic.

  - Differential, Rear:

    - (*)  Ratio:  To provide a cruise speed of 65 MPH, minimum.

    - (*)  Limited slip or locking type, OEM required.

- CHASSIS:

  - GVWR:  Minimum of 14,500 pounds.

  - Wheelbase:  Minimum of 158 inches.

  - Front Suspension:  Minimum 5,000 pounds.

  - Rear Suspension:  Minimum 9,450 pounds.  Rear suspension, right side, may require a spring spacer to be installed to eliminate sag caused by the additional weight of the wheelchair lift.

    - MOR/Ryde OPTIONAL ITEM:

      - (*)  In lieu of standard rear spring suspension, a MOR/ryde system is to be provided on the rear, rated at 7,800 pounds minimum.  As above, the right side may require a spring spacer to be installed to eliminate sag caused by the additional weight of the wheelchair lift.

  - Frame:  To be completely prepped and painted, prior to installation of body.

  - Steering:  Power assisted.

  - Brakes:

    - ABS braking system, front and rear.

    - Disc brakes front and rear.

    - OEM brake shields.

  - Front stabilizer bar, OEM, one (1) inch diameter minimum.

  - Shock Absorbers:  Heavy-duty front and rear.

  - Tires and Wheels:

    - (*)  Six (6) each, LT225/75Rx16E BSW "All Season", steel belted radials, mounted on OEM steel rims, minimum.

    - If 4X4 OEM approved 4X4 is selected, manufacture recommended conversion and installation is acceptable.

    - Spare tire and wheel (same as above "All Season") to be shipped loose.

  - Tire Chains OPTIONAL ITEM:

    - To be low clearance, INSTA-CHAIN or ONSPOT, automatic chain system.  Installed.

    - Compressor for automatic chain system shall not be exposed to the elements. (per am 2)

  - Fuel Tank Capacity:  55 gallons minimum.

- ELECTRICAL:

  - 12-volt negative ground.

  - Alternator:  155 amp minimum.

  - Batteries:

    - Two (2) each, maintenance free rated, sealed type, 1100 CCA at zero degrees Fahrenheit, minimum.

      - OEM Battery package is acceptable providing it meets the specifications.

    - The battery storage compartment will be fitted with hooded vents that allow a continuous exchange of air through the battery compartment.

    - Batteries shall not exposed to the elements.

    - Battery compartment shall be capable of being flushed with cleaning solution.

    - A 12V DC jump lug shall be installed under hood and easily accessible.

    - Master Battery Disconnect Switch:

      - COLE HERSEE #284-02 or POLLAK #51-315 or KISSLING #35-213-151-R-900, to cut off power source from battery to the ground (ground side if possible, positive side if not) and remainder of electrical system.

      - To be located for easy access.  Preferable on the floor between the door and the driver’s seat.

  - Lighting:

    - Dome light, cab.

    - Headlights to be Halogen.

      - To include headlights "ON", audible warning device for when key is turned off and headlight switch is left on (unless the system turns off automatically).

    - Turn signals with 4-way flashers.

- GAUGES, INDICATORS, INSTRUMENTS, AND PANEL:

  - All gauges are to read in pounds, PSI, gallons, volts, MPH, miles, Fahrenheit, etc.  Gauges that also include metric are acceptable.

  - The layout of all controls, gauges and instrumentation is to be ergonomically designed.

  - Chassis to include the following, as a minimum:

    - Warning lights for low engine oil pressure and high engine coolant temperature.

    - Engine oil pressure gauge.

    - Engine coolant temperature gauge.

    - Voltmeter or ammeter.

    - Speedometer and odometer.

    - Cruise/speed control.

    - Fuel level gauge.

    - Tilt steering.

    - Parking brake warning light.

    - All non-OEM instruments shall be easily accessible for maintenance and repair and shall be mounted so that each instrument is clearly visible to the driver.

    - Each instrument shall be sufficiently illuminated.

    - AM/FM radio with CD.

    - Windshield wipers, multiple speeds, with washers and OEM installed intermittent swipe feature.

    - Heaters:  Temperature/fan controls for front and rear heater.  Front and rear to be separately controlled.

    - Air conditioning, chassis OEM minimum.

    - Padded sun visor on curbside for driver.

    - Engine High Idle System OPTIONAL ITEM:

        - Engine High Idle System shall be INTERMOTIVE “Gateway AI” or approved equal ( www.Intermotive.net ).

        - Fully automatic type activated when transmission is “Neutral” or “Park” position, air conditioning is operating and driver switch is on

        - Engine idle shall also increase to 1,200 RPM when system voltage drops below 12.75 volts.

        - The system is to disengage when the driver steps on the service brake or when the charging system reaches its upper limit.

  - Labels and Identification Plates:

    - All switches, gauges and controls are to be properly identified.

    - “DYMO” type tape labels are not acceptable.

    - Stick-on type labels are not acceptable, however, labels with OEM part numbers that are parts manual listed are acceptable.  Otherwise, the labels are to be riveted or similar to maintain position.

    - Instruction Plates and Markings:

      - Where important procedures, safety warnings, or instructions are required, instruction plates must be provided at locations applicable and readily visible.

    - NO SMOKING signs are to be placed at conspicuous locations inside the van.

      - The plates shall be of heavy plastic laminate and engraved in a manner that instructions cannot be removed or defaced.

      - The instructions shall be of marked contrasting color from the background material.

      - The unit shall also have painted or decaled in a conspicuous location an international handicap emblem to indicate the van is not for general passenger use.

  - All dials, gauges, switches, controls, and ID’s are to be properly lighted for day or night operation (a flexible cable night light is not acceptable).

- BODY:

  - Cab over engine, cut-a-way, standard body, configuration:

  - Electrical:

    - Dual Auxiliary Batteries (For Wheelchair Lift Operation):

      - Battery Compartment:

        - Circuit breakers for wheelchair lift to be located in the battery compartment.

        - Location to be accessible by a latched pull out stainless steel tray with stainless steel bearing slides.

        - Compartment to be weather proofed and sealed with one-way drain vents.

        - Battery leads and circuit breaker wires going into compartment are to be weather packed.

      - Dual Auxiliary Batteries Master Disconnect Switch:

        - COLE HERSEE #284-02 or POLLAK #51-315 or KISSLING #35-213-151-R-900, to cut off power source from battery to the ground (ground side if possible, positive side if not) and remainder of electrical system.

        - To be located for easy access.  Preferably on the floor between the door and the driver’s seat.

    - To include ‘Door Ajar’ warning buzzer(s) for wheelchair lift and rear emergency doors that are open or not completely shut when vehicle’s transmission is taken out of park position.

    - Special Lighting:

      - LED lighting for stop, turn, tail, marker (body minimum).

        - Complete flush mounted system shall conform to Federal (FMVSS) specification #108.  Flush mounted where possible.

      - To be provided at the interior roof and side wall locations to the following areas:

        - Driver's station courtesy light located in the driver area and shall be activated when driver’s door is opened or with headlamp switch.

        - Main passenger compartment lights mounted over seats equidistant throughout the vehicle.  Master switch for dome lights will be located on driver’s control panel.

        - Front passenger door and step well areas.  To be activated when door is opened.

        - Wheelchair lift area (inside and outside, above lift minimum), activated when door(s) open.

        - Emergency egress window designation lights, installed above each emergency egress window, red in color and activated with ignition switch in “ON” or “Accessory” position.

      - Roof exterior marker lights, red rear and amber front and sides shall be LED, located in uppermost areas of the front and rear caps of the vehicle to provide better visibility.

    - Back-Up Alarm:  Electronic, self-adjusting sound level, ACORN PRODUCTS Model 1D-112AA, ECCO Model SA917, PRECO Model Preco-Matic 1040, STAR Model Starmatic 63-000, or WARN Model Reactor 2100504, located on the rear of the unit as per manufacturer’s recommendations.

    - Wiring:

      - Accessory wiring to include rated breaker switches located for ease of maintenance.

      - To be color coded or continually numbered.

      - To be located for maximum protection from road splash, stone abrasion, grease, oil, fuel, and heat from engine and components.

      - Routing through structural members to be protected by grommets.

      - To be secured by clips at intervals to prevent rubbing or chafing due to movement.

      - The use of SCOTCHLOCK or similar electrical connectors is not acceptable.

  - Construction:

    - Body construction shall be manufacturer’s standard design for maximum structural integrity, passenger safety and body lift.

      - The front area of the bus, where the bus body joins the chassis, design and construction shall be done in a manner that smoothly blends the two (2) differing assemblies into one (1).

      - All metal body parts and frame members shall be treated to prevent corrosion.

      - Body sides, rear and roof shall be thermally and acoustically insulated.

      - Body construction shall have sufficient rigidity to prevent vibration, drumming or flexing in service.

    - Welding of any body understructure to the chassis frame is not acceptable.

    - (*)  All exterior panels (walls and roofs) shall be manufacturer’s standard aluminum, or fiberglass, or galvanized steel.

    - All exterior panels shall be close end riveted, welded or bonded to the body framing.

      - Stainless steel screws shall be used in connection with stainless steel rivets; however, no exposed metal screws shall be permitted.

      - Rivets, if used, shall be flush or truss head type and not protrude above the panel surface.  Rivet spacing shall be in accordance with the best practice of the bus industry.

    - All panels shall be installed so that they will shed water.  The leading panel shall be lapped over the following panel and, in no case, shall the sealing of the panels be dependent on caulking alone.

    - The body shall be made tight to prevent leakage and thoroughly water tested.  The chosen vendor shall provide written documentation, at not later than delivery of the unit, outlining water test procedures including the results of these tests.

    - Rub/trim Rail:

      - One (1) inch wide, black in color, full-length rub rail shall be on each side of the vehicle.

      - Exterior:  Rub rails of corrosion resistant material shall be provided to help protect the bus from damage from other vehicles or obstructions.

    - Welding of any body understructure to the chassis frame is not acceptable.

  - Heating, Rear Passenger and Front Passenger Area (also refer to chassis heating):

    - (*)  In addition to the chassis manufacturer’s heavy-duty front heater and defroster for the windshield and driver’s area, an above floor interior heater unit will be provided and located so as to maintain a comfortable atmosphere for the passengers and the driver during Alaska’s harsh winters.

      - To include a caged fan, adjustable, two-speed, located at the upper rear curbside corner area (rear of the wheelchair lift). Switch to be driver controlled.

      - All heaters and defrost fans to be minimum two-speed, independently switched, and driver controlled.

      - Heat is also to be directed to the interior steps to help prevent ice buildup (heated air or electric pad type heaters may be utilized).  Switch to be driver controlled.  If pad type heaters are utilized, be sure that they are properly installed so they will not come loose due to the very harsh environment.

      - An auxiliary circulation pump for the rear heaters is to be installed.

      - Heater hoses to be insulated (THERMOCEL or equivalent) and routed to prevent damage.

      - Rear heater shall be floor or ceiling mounted.

      - Auxiliary Heater OPTIONAL ITEM:

        - (*) Additional auxiliary heater when requested to be an ESPAR Hydronics 5 or WEBASTO diesel fired heater with timer.

  - Air Conditioning:

    - Auxiliary air conditioning is not required, but will be accepted if it does not increase the overall vehicle price.  Chassis OEM is required.

  - Insulation:

    - (*)  Roof, sidewalls, front and rear crown’s insulation, minimum R-6 rated, shall be either VERTICEL composite or foam seal urethane insulation, providing the most efficient insulation in an operating environment, which ranges from minus 20 to plus 110 degrees in Fahrenheit.

    - The body shall also be well insulated against operating noises and vibrations and limit interior and exterior noise levels to 85 DBA or less under full load conditions throughout the vehicle.

  - Floor:

    - The plywood sub-flooring (as described below) is to be bolted to the minimum 16-gauge cross-members utilizing corrosion resistant grade-5 or 10-guage Tek counter-sunk flat-head screws and nuts.

    - The entire floor shall be supported by a fully welded steel sub-floor understructure.

    - The long sills shall be equivalent to three (3) inch “C” channel, 11-gauge steel, minimum.

      - Long sills shall be bolted to the cab section and rubber suspension mounted to the chassis frame utilizing grade-8 nuts and bolts.

      - Steel sub-frame to be primed and treated with anti-corrosion primer, industrial grade.

      - Over the sub-frame structure shall be bolted a single sheet, or sheets, of marine grade plywood, minimum five (5) ply, ¾ (0.75) other than a single sheet, to be tongue and grove, or if not tongue and grove to have the adjoining edges sealed or 11/16” FRP one piece composite floor.

      - Floor structure to be pattern cut, with all edges sealed prior to installation.

      - To be completely undercoated with petroleum or asphalt based product such as PENNS-GARD III, TECTYL 121-BN, BEGA COAT or equal and have all edges sealed prior to installation to the steel sub-floor assembly.

      - Complete vehicle to be undercoated again, after final assembly, with PENNS-GARD III, Z-TECH or equal undercoating material.

    - Floor Covering:

      - To be smooth Rubber Solutions Flooring or R.C.A. Transit, minimum 1/8 (0.125) inch thick for areas under the seats.

        - ALTRO Vinyl OPTIONAL ITEM:

          - In lieu of above standard, floor covering to be ALTRO vinyl.

      - Entrance and aisle to be ribbed rubber type, 3/16 (0.1875) inch thick minimum.

      - All seams to be completely sealed to help prevent moisture from access to sub-floor.

      - White or Gray in color.

  - Interior:

    - Interior panels shall be of aluminum, or fiberglass reinforced plastic and shall provide a durable finish.

      - White or Gray in color

      - Aluminum shall have baked enamel or vinyl clad finish.

    - Passenger area headliner, fiberglass reinforced plastic, padded vinyl, or vinyl clad aluminum.

      - White or Gray in color

    - Stanchions, Grab Handles and Modesty Panels:

      - Step well:

        - A vertical stanchion and modesty panel shall be installed in the entryway at the rear of the step well.

        - Handrail shall be provided on both sides of the entry, accessible from the ground.

        - The handrails shall be capable of withstanding a force of 100 pounds concentrated at any point on the handrail without permanent deformation of the rail or its supporting structure.

      - A vertical stanchion and modesty panel shall be installed directly rear of the driver’s seat.

      - An over-cab storage compartment, locking, shall include a storage bin(s) to hold cleaning items such as window cleaner, paper towels, body fluid kit, engine oil, etc.

        - The overall size should be approximately 18 to 24 inches long, 6 to 8 inches wide, and 8 to 10 inches tall.

        - The storage bin does not need to have a lid.

      - A vertical stanchion and modesty panel shall be installed directly in front of the wheelchair lift entryway.

      - Contoured vertical grab handles shall be installed both right and left of the entryway, and shall be securely fastened to the interior of the doorframe.

      - All stanchions and handrails shall be between 1-1/4 (1.25) and 1-1/2 (1.5) inch minimum diameter, textured stainless steel or powder coated and shall be securely fastened into structural members at all mounting points.

        - Stanchions shall not be mounted to sheet metal, fiberglass, or other non-reinforced areas.

      - All entry and exit stanchions and railing shall be yellow powder coated. Fittings shall be stainless steel.

        - Painted, other than powder coating, stanchions and railings are not acceptable.

        - Exception is given to the wheelchair lift, which may be manufacturer’s standard.

      - (*)  Bidder shall provide a diagram with their bid package of all stanchions, handrails, and modesty panels.

  - Seating:

    - Seating will not exceed 15 persons, including driver.  The vehicle offered is not to require the driver to obtain a Commercial Driver’s License (CDL).

    - (*)  Driver and twelve (12) ambulatory passengers and two (2) wheelchair positions and have the capability to change to six (6) ambulatory passengers and four (4) wheelchair positions.  Bidder is to submit with their bid package a diagram showing the different seating arrangements.

    - Driver’s seat, high back, bucket type, with folding inside armrest.  Seat position shall be adjustable forward and rearward with tilt reclining and shall include mechanical adjustable lumbar back support.  Cloth covered, not vinyl, coordinated to interior color scheme.

      - 6-Way Adjustable Driver’s Seat OPTIONAL ITEM:

        - In lieu of standard, driver’s seat to be SEATS INC, Freedman CL-67, ADNIK, or approved equal 6-Way Adjustable Suspension Driver’s Seat

    - Ambulatory Passenger Seats:

      - To be FREEDMAN SEATING COMPANY, forward facing mid-back contour bench seats with grab handles on all seats and armrests on aisle sides.

        - The passenger side seats, three (3) rows, are to be mid-back bucket seats.

          - Ahead of these seats will be a modesty panel, as noted previously.

        - Anti-Bacterial Fabric OPTIONAL ITEM:

          - In addition to above, FREEDMAN seats to include Anti-Bacterial Seat Fabric, MOR-CARE Vinyl or equivalent.  To be priced per seat.

      - Hip to Knee Space:  Minimum 27-inch space between seats.

      - Cushion Depth:  Minimum 17-inches.

      - Isle width to comply with minimum ADA requirements.

      - Each ambulatory passenger seat position shall be equipped with passenger restraint lap belts intended to hold passengers in a secure seated position during normal operation.

    - Wheelchair Positions:

      - There will be a total of two (2) wheelchair positions.

      - Third (3rd) Wheel Chair Position OPTIONAL ITEM:

        - A third (3rd) wheelchair position is to be priced separately.  A manufacturers drawing will be submitted with the bid package for inclusion into the final contract showing the seating options available with this additional wheelchair position.  This will include raising the vehicle floor to a raised/flat floor.

      - Fourth (4th) Wheel Chair Position OPTIONAL ITEM:

        - A fourth (4th) wheelchair position is to be priced separately.  A manufacturers drawing will be submitted with the bid package for inclusion into the final contract showing the seating options available with this additional wheelchair position.

      - Single Flip Seat OPTIONAL ITEM:

        - A single flip seat, to be FREEDMAN SEATING COMPANY, Feather Weight Mid-Hi Flip Seat.  To be priced per seat.  Floor plan will be reviewed and approved prior to issuing of purchase order for accuracy.

      - Each wheelchair station shall have a four (4)-point securement device with anchor points of sufficient strength to secure a wheelchair.

      - No anchoring points or securement shall be attached to any door or walls of the vehicle.  Exception being for wheelchair shoulder restraints to walls.

      - The securement system shall conform in all respects to the requirements of the ADA and will secure the wheelchair to the vehicle and provide a seatbelt for the wheelchair passenger.

      - The system will include fully recessed SURE-LOK (  www.sure-lok.com  ) “L” track, full length (front to rear), both sides of the position and will include wheelchair passenger lap and shoulder restraints.

        - There are to be two (2) each full length (front to rear) tracks in the wheelchair position areas.

        - The inside tracks are to be on center at 15 inches.

        - The outside tracks are to be on center at 28 inches.

        - Installation as per 49 CFR Part 571.222 b (FMVSS 222), ADA, SAE J2249, and per SURE-LOK recommendations.

      - All securement devices shall be adjustable to accommodate several types and sizes of wheelchairs.

      - All wheelchair tie-downs and occupant restraint systems shall be SURE-LOK.

        - All wheelchair strap restraints shall meet recommended force requirements as specified in Guideline Specification for Wheelchair Securement Devices, US Department of Transportation and 49 CFR 38.23(d) which are hereby incorporated as part of these Technical Specifications.

        - There shall be a total of two (2) each, identical strap assemblies to secure the wheelchairs to the tie down track.  Each strap assembly shall consist of a fixed end strap, buckle with shoulder belt, closed hood, “D” ring, end fitting, and SURE-LOK RETRACTOR FF600 quick adjustment system.

        - Each wheelchair securement assembly shall include a passenger restraint lap and shoulder belts intended to hold the passenger in a secure seated position in normal operation.  Self-retractable belts are not acceptable.  Seatbelt assemblies shall attach directly to the rear strap assemblies of the wheelchair securement system.  The wheelchair tie downs are to have a rated strength of 2,500 pounds minimum.

        - Separate storage pouches shall be made and installed near the wheelchair securement areas for the storage of all seatbelt and strap assemblies.  These pouches shall not interfere with the flexible placement of wheelchairs in the various tie-down stations inside the vehicle.

        - Detailed written instructions with diagram(s) shall be supplied with detailed instructions on the proper operation, placement, and removal of wheelchair securement straps and seatbelts.

      - All wheelchair and seating positions are to have the passenger facing forward.

  - Wheelchair Lift:

    - Wheelchair lift shall be located on the right side (curbside) of the vehicle, rear of the rear drive axle.

    - (*)  To be conventional 34 x 53 inch standard platform.

    - To be fully automatic to independently lift all sizes of manual or electric wheelchairs into the vehicle.

    - Load capacity of 800 pounds, minimum.

    - All attachments will be done through the vehicle frame.  Securement of lift to vehicle shall be rigid with no flexing movement between lift frame and vehicle body.

    - Electrical connection of lift shall pass through an automatic circuit breaker at the battery end of the connection.

    - The wheelchair lift shall be completely operational while the vehicle engine is off and transmission in park.  The auxiliary battery system shall be sufficient to provide for a minimum of 12 complete cycles of the lift while the engine is off and there is 300 pounds on the lift.

    - (*)  Lift controls must be ADA 403-404 compliant and operation shall be interlocked with the vehicle emergency brakes, wheelchair lift door and transmission to ensure that the vehicle cannot be moved when the lift is not stowed and the lift cannot be deployed unless the interlock is engaged.

    - For the wheelchair lift installed, the contractor shall furnish, at time of delivery:

      - A complete set of written operating instructions.

      - A complete troubleshooting guide.

      - A factory warranty schedule of inspection and service.

      - (*)  The name, address and phone number of at least one (1) factory authorized company/mechanic who can service the lift in the State of Alaska.

      - A complete schematic color-coded wiring diagram for the lift and installation.

  - Glass/Windows:

    - Windshield, chassis manufacturer equipped with nontransparent sun visor.

    - Driver’s side window to be manual roll up/down.

    - Power Windows OPTIONAL ITEM:

      - Driver’s side window to be power roll up/down.

    - Passenger’s entry door windows to be manufacturer’s standard full glass or split glass (two-piece) located both halves of panels; minimum 31% (percent) tinted tempered glass.

    - Side passenger windows shall be manufacturer’s standard for the body style bid, T-bar sliding located at top of windows tinted approximately 31% (percent) luminous transmittance as per ASTM D-1003.

      - One (1) window on each side shall be emergency egress, kick-out type, and clearly marked as such.

    - Wheelchair lift door window(s) to be manufacturer’s standard.  If double doors, minimum of one window in each door.

    - Rear Window:  One (1) window, approximately 22 x 48 inches, tinted approximately 31% (percent) luminous transmittance as per ASTM D-1003.

      - Note:  This rear window requirement would be deleted if a REAR EMERGENCY DOOR is being requested.

    - Water deflection drip molding shall be installed over all windows and doors.

  - Doors:

    - Driver’s door per OEM specification.

    - Passenger Entrance Door:

      - To be manufacturer’s standard double-leaf outward opening.

      - Electric transit type.

      - Dimensions:

        - 74 inches high x 27 inches wide with minimum 26 inch clear opening between assist rails of doors when open.

        - Full height, minimum.

      - The meeting edges of the door shall be equipped with manufacturer’s standard design material on each section that overlaps when doors are closed, forming a tight seal.

      - The entry door, when closed, shall match the contour of the bus body as closely as possible.

      - Door pivot points shall be fitted with easy access lubrication fittings or have sealed roller bearings.

      - Door shall be equipped with a manual emergency opening system.

      - There shall be a flush-mounted heavy-duty auxiliary weatherproof-keyed lock assembly located next to the side double door, providing the driver with a means of ingress/egress while maintaining security.

    - Wheelchair Lift Door:

      - Single or double, manufacturer’s standard with clear opening to be sufficient to accommodate specified wheelchair lift assembly, with “L” handle, two (2) point latch with key lock.

      - Door(s) will be held in the open position by individual spring cylinders.

      - The door(s) shall be located on the right (curbside) side, rear of the rear axle.

    - Rear Emergency Door OPTIONAL ITEM:

      - If requested, to include upper and lower safety windows.

      - This would be in lieu of the single rear window described earlier in this specification.

      - To be centered on rear of vehicle.

      - To include inside handle with guard.

      - To be equipped with a starter interlock system that would not allow vehicle to be started until the door lock was in an unlocked position.  An interior operated door lock is required.

    - All doorframes shall be of stainless steel or galvaneal steel, 14-guage minimum.

      - If stainless steel or galvaneal steel is not available, vendor must provide FMVSS 220 and FMVSS 221 Certification with bid. (per am 3)

    - Maintenance access panel doors shall be provided for fuel pump and access doors where necessary to service the engine, radiator, batteries, electrical fuses and relays, door controls, etc.

    - All doors shall be trimmed with channel trim on edge to help protect against damage or injury.

    - Water deflection drip molding shall be installed over all doors.  If the body design is such that dripping is not a problem at the passenger and wheel chair doors, then this requirement may be deleted for those two (2) areas only.

  - Step well:

    - Enclosed type to be 14-guage stainless steel frame integrally welded to the door frame.

    - The ground to first step height is to be 12 inches maximum.

    - Step Width:  26 inches between assist rails.

    - Step Tread Depth:  Eight (8) inches minimum.  Step tread edges shall be flush with the step.

    - Step tread material to be heavy-duty, non-skid rubber.

      - Step treads to be 3/16 (0.1875) inch thick ribbed and include two (2) inch contrasting white nosing, properly sealed, on each step.

      - In addition, a 1/8 (0.125) inch thick smooth rubber cove molding shall be installed from the floor to the sidewall seat track.

      - White or Gray in color.

    - Step Risers:  Seven (7) inches minimum, 9-1/2 (9.5) inches maximum.

    - To be equipped with step well light as described later in this specification under “Special Lighting”.

  - Mirrors:

    - Exterior:  Dual (left and right sides), minimum 6 x 9 inches with six (6) inch diameter convex mirror mounted directly below the rectangular mirrors.  Right side (curbside) mirror may be fender mounted for easier viewing.  Bracing to be stainless steel or powder coated.

      - Exterior mirrors are to be heated and include a separate dash mounted switch with amber “ON” indicator light.

    - Exterior Upper Left Rear:  To be a rear cross view convex style, minimum ten (10) inches in diameter.  Bracing to be stainless steel or powder coated.

    - Interior:  Approximately 6 x 16 inches with up/down adjustment, located forward and above driver’s seat for viewing passengers (and rear emergency door’s lower glass if so equipped).

    - Bracing should be adequate to prevent vibration of the mirrors.

  - Roof Vent/Escape Hatch:

    - TRANSPEC Model 1122LP (low profile), located toward rear of vehicle.

  - Bumpers:

    - Front:  ROMEO RIM help bumper.

    - Rear:  ROMEO RIM help bumper.

  - The van and equipment shall meet or exceed all ADA requirements in all aspects.

- MATERIALS AND WORKMANSHIP:

  - Workmanship shall be of the best grade and shall conform in all respects to the best practices in the industry.

  - Welding procedures, welding materials, and qualifications of welders shall be in accordance with standards of the ASTM and the American Welding Society (AWS) or Canadian Welding Bureau (CWB).  All exposed welds shall be ground smooth after welding, if necessary, to present a smooth, workmanlike appearance.  Prior to welding, the contact surfaces shall be free of scale, grease, paint, and rust.

  - All materials used in construction of vehicle shall conform to ASTM, SAE, or similar association standards.  All materials shall be first quality.

  - All exposed bright metal shall be stainless steel, anodized aluminum, or chrome plated steel.

  - All exterior joints shall be protected and sealed by a non-shrinking sealant prior to assembly.

  - Where dissimilar metals are riveted or bolted together, contact surfaces shall be thoroughly cleaned and coated with heavy-duty zinc chromate primer or butyl tape.  All rivets shall completely fill the holes.  Rivet heads externally shall be concentric with the body of the rivets and free from rings, pits and fins.

  - (*)  All bolts or rods passing through wood shall be cadmium, zinc plated, or stainless steel.  Where wood and wood are placed together, both surfaces shall be coated with powdered aluminum and spar varnish, linseed oil and titanium oxide, urethane sealer, or ISOCLAD prior to assembly.  Sealer/coating type must be submitted with bid.

- Isolated with Sikkaflex industrial adhesive/isolator in conjunction with application of TECTYL 121BN compound.

  - All metal shall be thoroughly cleaned, acid etched, and painted, unless otherwise noted in this specification.

  - All burrs and sharp edges shall be dressed to prevent injury to passengers, operator and maintenance personnel.

  - All materials installed shall be new and free of rust.

  - Weather stripping shall have no coating of paint, adhesive or other alien material.

  - The body shall be free of all cracks, dents, and defects.  Special care shall be taken with the outside sheathing, roof, roof bonnets, and the interior finish so that all kinks and buckles are removed before assembly to present a true and smooth finish without excessive grinding off of the material so as to weaken the structure.

  - The interior cavities of the body steel sub frame structure shall be treated to prevent corrosion-using WAXOYL or ZIEBART.

  - Understructure, including body, chassis, and wheel wells, shall be undercoated, using manufacturer’s standard process.  All conversion metal to be etched with rust inhibitors prior to application of undercoat, with a minimum three (3) year rustproof warranty.

  - The cleaning, sealing, undercoating, priming, and/or painting process shall be those methods and products which are used as standard production and/or manufacturing techniques by the manufacturer.  Those products used by the manufacture in their techniques shall be acceptable unless specified otherwise in these specifications.  These processes and related products shall be submitted with the bid.

  - All windows shall be free of any defects due to welding slag or assembly damage.

  - All hoses supplied in assembling the unit described in these specifications shall have PSI and temperature ratings in excess of maximum operating temperatures and pressures of fluids or materials being transferred.

  - Toggle switches controlling electrical components to be metal, plastic is not acceptable.  Rocker arm type switches may be plastic or metal.

  - All brackets, nuts, bolts and miscellaneous features for attaching front and rear license plates shall be provided.

  - All painted surfaces shall have a true and smooth surface that will not show sanding or grinding marks after painting.

  - Water Testing:

    - After entire vehicle is assembled and painted, Body Manufacturer shall thoroughly and completely water test the vehicle for leakage.  All leaks shall be sealed or otherwise corrected before vehicle shipment.

- ALTOONA TESTS:

  - (*) These specifications describe a medium duty cutaway style paratransit bus with a design life of 7-year/200,000 miles.  The vehicle shall be a medium-size cutaway style

  - bus, which meets all FMVSS requirements and applicable ADA regulations.  Vendor shall submit copies of STURAA (Altoona) 7-year, 200,000 mile Test Report with bid submission.

- MISCELLANEOUS:

  - Instruction Plates and Markings:

    - Where important procedures, safety warnings, or instructions are required, instruction plates must be provided at locations applicable and readily visible.

    - NO SMOKING signs are to be placed at conspicuous locations inside the van.

    - The plates shall be of heavy plastic laminate and engraved in a manner that instructions cannot be removed or defaced.

    - The instructions shall be of marked contrasting color from the background material.

    - The unit shall also have painted or decaled in a conspicuous location an international handicap emblem to indicate the van is not for general passenger use.

  - Remote engine start/stop system OPTIONAL ITEM:

    - To be ASTRO START Model 803 remote engine system with minimum 1000-foot range.

  - First Aid Kit OPTIONAL ITEM:

    - To include a 24 Unit Unitized First Aid Kit in metal case w/gasket.  (FIRST AID PRODUCT # 240-U or equal).  Kit must be attached to vehicle or placed in a compartment that is clearly marked “First Aid Kit”. (www.First-Aid-Products.com)

  - Body Fluid Clean-Up Kit OPTIONAL ITEM:

    - To include one (1) each SAFETEC NATIONAL STANDARD or equal Body-fluid Clean-up Kit.  Kit must be attached to vehicle or placed in a compartment that is clearly marked “Biohazard Kit”.  (www.allmed.net)

  - Warning Triangles OPTIONAL ITEM:

    - To include one (1) set of three (3) each, collapsible warning triangles, complete with plastic storage box.  DOT approved or equivalent.

  - Flares OPTIONAL ITEM:

    - To include one (1) set of two (2) flares, LCD, Power Flare PF-200.

  - Fire Extinguisher OPTIONAL ITEM:

    - One (1) each, five (5) pound dry chemical with gauge, mounted with quick release mechanism in driver's area.

  - Safety Belt Cutter OPTIONAL ITEM:

    - Each vehicle will be equipped with one (1) each SURE-LOC or TIE TECH #1004 or equivalent webbing (safety belt) cutter.

  - Paint:  Exterior to be white.

- END OF SPECIFICATION

BID SCHEDULE INSTRUCTIONS:

All bids must include both unit pricing and extensions and be otherwise complete in the format requested.

DELIVERY:  Indicate, in the space(s) provided, the time required to make delivery after the receipt of an order (ARO).  Failure to make an entry in the space(s) provided will be construed as an offer to deliver within “Required Delivery” NUMBER of calendar days ARO.  Bids which specify deliveries in excess of “Required Delivery” NUMBER of calendar days ARO will be considered non-responsive and the bid(s) will be rejected.

The materials, equipment or services bid will conform to and meet the requirements of the Contract Bid Documents and are hereby incorporated into this document:

Complete the attached bid price sheets and return with other bid documents as required in Section II – Standard Terms and Conditions, paragraph 14.

Item  Qty          Description                                                                             Amount                     _

1a	1ea. 	Paratransit Vehicle, narrow body design,	$

minimum 14,500 pound GVWR, gas engine powered,

4x2, single rear axle, dual rear wheels, with seating for

driver, twelve (12) ambulatory passengers and two (2)

wheelchair positions.

Body - Year, Make and Model Offered:

Chassis - Year, Make & Model Offered:

OPTIONAL ITEMS (Pricing is required for the following items)

1b	1 ea.	Four Wheel Drive (in lieu of 4x2 configuration)	$

(As per Spec Item 2.3)

Provide Make/Model:

1c	1 ea.	MOR/ryde suspension	$

(As per Spec Item 3.4.1)

1d	1 ea.	Tire Chains 	$

(As per Spec Item 3.11)

1f	1 ea.	Engine high Idle System	$

(As per Spec Item 5.3.17)

1g	1 ea.	Auxiliary Heater	$

(As per Spec Item 6.4.1.7)

1h	1 ea.	ALTRO Vinyl Floor Covering	$

(As per Spec Item 6.7.4.1.1)

1i	1 ea.	6-way Adjustable Driver’s Seat	$

(As per Spec Item 6.9.3.1)

1j	1 ea.	Anti-Bacterial Seat Fabric	$

(As per Spec Item 6.9.4.1.2)

1k	1 ea.	Third (3rd) Wheel Chair Position	$

(As per Spec Item 6.9.5.2)

1l	1 ea.	Fourth (4th) Wheel Chair Position	$

(As per Spec Item 6.9.5.3)

1m	1 ea.	Single Flip Seat	$

(As per Spec Item 6.9.5.4)

1n	1 ea.	Power Windows	$

(As per Spec Item 6.11.3)

1o	1 ea.	Rear Emergency Door	$

(As per Spec Item 6.12.4)

1p	1 ea.	Remote Engine Start/Stop System 	$

(As per Spec Item 9.2)

1q	1 ea.	First Aid Kit	$

(As per Spec Item 9.3)

1r	1 ea.	Body Fluid Clean-Up Kit 	$

(As per Spec Item 9.4)

1s	1 ea.	Warning Triangles	$

(As per Spec Item 9.5)

1t	1 ea.	Flares	$

(As per Spec Item 9.6)

1u	1 ea.	Fire Extinguisher	$

(As per Spec Item 9.7)

1v	1 ea.	Safety Belt Cutter	$

(As per Spec Item 9.8)

TOTAL PRICE FOR LOT 1 (Includes items 1a through 1v)  	$

(Lot 1 Total is for bid evaluation purposes only)

Note:  The State will have the option of purchasing any or all of the OPTION items.  Items will be required to be ordered with the unit.

FOB: All prices are to be FOB dockside, Seattle/Tacoma Washington.  The contractor will be required to complete shipment to the final destination and bill back actual freight as an additional line item on the invoice.  A copy of the actual freight bill must accompany all requests for reimbursement.  Refer to Section III – Special Terms and Conditions.

Weight Verification Slips:  Bidder is to provide a certified scale house weight ticket with the MSO and Invoice.

Required Manuals including Service Bulletins:  To be delivered to the receiving entity as noted on the purchase order, no later than 30 days after receipt of the unit.  Refer to Section III – Special Terms and Conditions.

Required Delivery: Not later than 180 days ARO.

Offered Delivery Time       Days ARO.

If Offered Delivery Time is left blank, it is understood that delivery will be made in 180 days.

Contract Note:  Prices must remain firm for 180 days after award, reference Section III, Paragraph 11.0.  The State Transit Coordinator may approve purchase by the State, other government entity or approved grantee of up to sixty (60) units on the contract

END OF BID PRICE SCHEDULE

THIS IS NOT AN ORDER

SEALED BIDS MUST BE SUBMITTED TO THE STATE EQUIPMENT FLEET PRIOR TO BID OPENING AT WHICH THEY WILL BE PUBLICLY OPENED.

IMPORTANT NOTICE:  If you received this solicitation from the State’s “Online Public Notice” web site, you must register with the Procurement Officer listed on this document to receive subsequent amendments.  Failure to contact the Procurement Officer may result in the rejection of your offer.

BIDDER'S NOTICE:  By signature on this form, the bidder certifies that:

(1) the bidder has a valid Alaska business license, or will obtain one prior to award of any contract resulting from this ITB. If the bidder possesses a valid Alaska business license, the license number must be written below or one of the following forms of evidence must be submitted with the bid:

- a canceled check for the business license fee;

- a copy of the business license application with a receipt date stamp from the State's business license office;

- a receipt from the State’s business license office for the license fee;

- a copy of the bidder’s valid business license;

- a sworn notarized affidavit that the bidder has applied and paid for a business license;

(2) the price(s) submitted was arrived at independently and without collusion and that the bidder is complying with:

- the laws of the State of Alaska;

- the applicable portion of the Federal Civil Rights Act of 1964;

- the Equal Employment Opportunity Act and the regulations issued thereunder by the State and Federal 	Government;

- all terms and conditions set out in this Invitation to Bid (ITB).

If a bidder fails to comply with (1) at the time designated in the ITB for opening the State will reject the bid.  If a bidder fails to comply with (2) of this paragraph, the State may reject the bid, terminate the contract, or consider the contractor in default.  Bids must be submitted under the name as appearing on the person’s current Alaska business license in order to be considered responsive.

If a contract award is made by the State of Alaska, the undersigned offers and agrees to furnish materials, equipment and services in compliance with all terms and conditions specified in ITB/Contract.  Submittal of this document with authorized signature constitutes full understanding and a promise to comply with all specifications, terms and conditions of ITB/Contract.

| TO BE COMPLETED BY BIDDER: COMPANY SUBMITTING BID AUTHORIZED SIGNATURE PRINTED NAME DATE | ADDRESS CITY, STATE, ZIP PHONE/FAX E-MAIL ADDRESS | ALASKA BUSINESS LICENSE NO: DOES YOUR BUSINESS QUALIFY FOR PREFERENCES? YES NO DOES YOUR BUSINESS QUALIFY FOR THE VETERANS PREFERENCES? YES NO |
| --- | --- | --- |

Revised 03/14/2012